TN210_21.DOC

TELEMETRY OPTION

********************

Bill,  NJ7P has developed a prototype node telemetry option intended to work

in concert with TheNet Plus.     This adaptor has 16 lines of digital I/O which

are divided into two ports of eight lines each.  The ports are SYSOP selectable

as to input or output.  This gives the following options:

a.  Both ports TO the site.

b.  Both ports FROM the site.

c.  Port A TO and port B FROM the site.

d.  Port A FROM and port B TO the site.

An  analog to digitial converter is on the telemetry board.    It is referenced

to  0  and  +5  volts.     The  unit has  multiplier  resistors and is software

configurable.  It has  ranges of  0-5,  0-10,  0-15,  0-20, or  0-25 volts with

a  resolution  of 20  millivolts at 0-5 volts.  An external quad Op-Amp circuit

allows DC voltage  gain  matching  between external  sensors and an appropriate

voltage range.

The telemetry board fits inside the TNC.   It plugs into the Z-80 CPU socket at

U22.    The CPU then plugs into a socket on the telemetry board.   The only TNC

modification is the  addition of wires between unused RS-232 pins and telemetry

board header.      Shielded wires are ran between on-site sensors or controlled

devices and the RS-232 connector.

Telemetry  applications  include:   Remote monitoring of power supply voltages,

forward and reflected RF power, inside-outside temperature, precipatation, wind

velocity, barometric pressure or, controlling beam antenna headings, etc.

The Telemetry option is not yet available.   More design effort is required for

code and interface development.     Specifically, Bill is searching for  sensor

designs which can be home-brewed at low cost.   Circuit design is simplified if

the  sensor output is a linear voltage.    Currently under  test are interfaces

allowing battery voltage and wind velocity measurement.

This  project is going to take time.    There isn't yet a source for ready made

boards.    Rather  than  delay  release  of TheNet Plus version 2.10, it's been

decided to impliment the telemetry option in a future release.    Meanwhile, if

anyone has specific sensor design and  application information to share, please

feel free to contact Bill.