Recently, several new packeteers using PK-232s have appeared

on  our local duplex repeater which is dedicated  to  packet

radio.    This   is  one  environment  where  the  collision

frequency  should  be  very low since there  are  no  hidden

terminals.  Almost immediately we noticed that the collision

frequency had risen dramatically.   After some investigation

we discovered certain stations were almost guaranteed to  be

involved in stepping on in-progress packets.  These stations

were  contacted and in all but 1 case they were new users of

PK-232s.

We were very puzzled as there have always been some stations

on  the  repeater using PK-232s with no  apparent  problems.

The new stations were asked how their station was configured

and what method was used to get the DCD operation  adjusted.

We were very surprised at the answers we got.   Every single

one  of  the  offending stations had set  their  station  up

exactly  according to the instructions in the PK-232 manual.

However, contrary to the advice given in the manual, none of

these stations had configured their setup so that they could

hear  what was going on on the channel when the  PK-232  was

connected  to the radio.   None of these new operators  knew

what DCD meant, what it did, or why it was important that it

should be working on a multiple access packet channel.

We  obtained  a PK-232 and manual to try  and  discover  the

exact  nature  of  the  problem.   What we  found  was  that

although  the  manual is very complete  and  generally  very

well written, there are some areas where it leaves something

to be desired.  Specifically, in this case, the instructions

given  on page 2-16 (we had manual PK232UG Rev.  B 9/86  for

reference) for setting up the PK-232 and an FM radio for DCD

operation  are  simply  incorrect.   If set  up  exactly  as

described,  DCD  will  NEVER  be asserted  during  a  packet

transmission  by  another station on the channel!!   We  had

found the cause of our problem.

If  you  have a PK-232 and haven't already  discovered  this

problem for yourself,  please disregard the instructions  in

the  manual for setting up a PK-232 and NBFM radio for  1200

baud  packet  operation and use the method  presented  here.

All your packet neighbors will appreciate it very much.

The manual is quite corect in all respects EXCEPT where they

discuss setting the DCD THRESHOLD control and receiver audio

output   level  for  proper  demodulation  and  DCD  circuit

operation.   The correct way to set these adjustments is  as

follows:

1.  At  least temporarily,  arrange to be able to  hear

the  receiver audio signal which is being  sent  to

the PK-232.

2.  Set  the  squelch circuit on the radio  for  normal

squelched operation.  The DCD circuit in the PK-232

is  incapable of proper operation with  unsquelched

audio from the receiver.

3.  While  monitoring  incoming  packets,   adjust  the

receiver  audio level so that the tuning  indicator

"spreads"  fully when receiving a packet  from  the

station  on  the channel which produces  the  LEAST

amount  of audio output level.   There are  several

limiters   in  the  PK-232  demodulator  so  louder

stations will not be affected adversely by this.

4.  Once  the audio level is properly set,  adjust  the

DCD THRESHOLD control on the PK-232 so that the DCD

led lights when there is a packet being transmitted

by the station on channel which produces the  LEAST

amount  of  audio output from the  receiver.   Make

sure,  however,  that  the DCD LED is  extinguished

when  there  is no signal and the  radio's  squelch

circuit has cut off all audio from the receiver.

If the above procedure is followed, the PK-232 will properly

hold  off  transmitting  during a packet  transmission  from

another  station  and  will  not  send  acknowledgements  to

individual frames of a maxframe greater than 1 packet  while

it is still being transmitted.

We  hope PK-232 owners will find this information useful and

take  steps to assure that their DCD is operating  properly.

Multiple  access  packet  channel  throughput  is   severely

degraded when DCD is not working.

73 and happy packeting,

Eric Gustafson  N7CL

#######################################################################################