SB-951

TS-940S ERRATIC DISPLAY						  01/20/89

Intermittent control or display problems that are difficult

to correct with the reset procedure may be caused by a

faulty ROM socket. The socket should be checked to insure

that it makes good electrical contact with each pin of the

ROM. If it is found to be intermittent, remove the socket

and solder the ROM directly to the board. It should be

noted that units between serial numbers 701XXXX to 811XXXX

and from 903XXXX to 909XXXX are not likely to have this

failure. In addition, units with a serial number of 909XXXX

and above do not incorporate a socket.

CAUTION

Removing the socket requires good soldering skills. The ROM

is mounted on the Digital A unit and is designated as IC2.

The board, being double sided, has solder connections on

both component and foil sides. When the socket is removed,

it is very important to insure that each pin is completely

solder free. Having to pry up on the socket means that it

is not completely desoldered and will cause the circuit

foils to tear.

When performing any work on the Digital A board, CMOS

handling techniques must be observed. Such techniques

include using a grounded or isolated soldering tip, avoid

touching the pins of IC chips with your fingers, and ground

yourself with a wrist ground strap.

To remove the socket:

1.	Disconnect the power cord and antenna coax.

2.	Remove the top and bottom covers from the transceiver.

3.	Remove the 2 flat head screws from each side of the

front panel chassis.

4.	Loosen the round head screw on each side of the front

panel chassis.

5.	Carefully rotate the front panel forward. It will be

necessary to unplug the VS-1 cable from the transceiver.

6.	Remove the 4 screws from the speaker mount.

7.	Carefully pull up on the mount and rotate it toward the

front panel. Swing the mount toward the right side of

the transceiver and allow it to rest on the Digital B

unit shield.

8.	Remove the 8 screws from the Digital A unit shield

plate.

9.	Lift the plate and rotate it to the left side of the

transceiver.

10.	While avoiding contact with the pins, remove the ROM

(IC2) and set it aside on anti-static foam.

11.	Remove the 6 screws that mount the Digital A board.

Rotate the board toward the front panel to expose the

bottom side of the board.

12.	Carefully desolder the socket and remove it from the

board. Do not pry up on the socket. If it does not

easily pull off the board, the top foils are still

soldered to the socket.

13.	Install the ROM in the board and solder it in place.

14.	Assemble the transceiver by reversing steps 1 - 11. Do

not pinch the power switch cables between the front

panel and the body of the transceiver.

Time required for this modification is 1.5 hrs or less. (C)

011189TKC