PRO CAT

MS/DOS to Yaesu FRG9600 Interface

Program

The  FRG9600 has such a wide range of frequencys it can  receive,

that  it takes a computer to bring its capabilitys  to  practical

use.  PRO-CAT is a professional quality interface written in  the

"C"  computer language, which is fast enough to allow almost  any

MS/DOS computer full access to the FRG9600 power. Here is a  list

of the hardware that is necessary for the interface.

1. MS/DOS  computer  with DOS 2.2 or  better,  128K  memory,

serial communications port, and game card adapter.

2. Yaesu FRG9600 CAT receiver.

3. Yaesu FIF232 interface device.

4. Standard RS232 patch cable with 25 pin connectors.

5. An IBM compatible joystick cable with 15 pin connector.

The RS232 connector that attaches to the computer may have to be

modified, and the joystick cable will need to be connected to the

CAT  plug  that  fits into the back of  the  FRG9600.  This  will

require a knife, a soldering iron, and some electrical tape.

When You're done you might also want to buy a frequency directory

for  your area. The services you can receive are  limitless,  you

may  want to be careful about listening to cellular phone  calls

though!

If  you are using this program, and got it from me, then  I  have

your  name and address. If you didn't, would you send me a postcard?

I like to know who's using PRO-CAT, be sure to include any comments

about your experiences with it!

Don Rasmussen A.R.S. WB8YQJ/6

2316-214 Paseo De Laura

Oceanside, California

92056  U.S.A.

The PRO-CAT CRT Display Explained

The parts of the PRO-CAT display are shown on the enclosed

diagram of the main menu, they are numbered 1 through 4.

1. Base frequency and limit frequency show the frequency range

that  will be scanned when the  s  key is pressed. You  can

change  these limits by pressing the l key.  The  frequency

range will be scanned in 5 Khz steps.

2. This  is  the  record number and  description  of  the

selected  memory record. As you can see, memory  record  1

contains  the frequency channels of the  Oceanside  police

department.

3. Each  channel  is  listed here, and  has  a  function  key

designator  to the left of it. If you press F10,  in  this

example, the FRG9600 frequency will be set to 467.975.

4. These are the commands available to you.

l - Changes the base frequency and limit frequency.  The

FRG9600  display  will flash each  digit  you  enter

until  you press the (enter) key. You can press  the

backspace key to correct any frequency entered in error.

s - Begins a scan of the frequency  range  between  base

frequency and limit frequency.

r - Allows  you to call up a  different  record  number.

Valid record numbers are from 1 to 100.

m - Begins a scan of the channels listed on  the  right

side of the display.

e - Allows you to add, change, or delete records in  your

scan information file.

F1 to F10 - Press a function key  to  set  the  FRG9600

frequency to the value listed on the right side of the

computer display.

<shift> F1 - If you press the shift key and the F1 key  at

the  same time, the computer will allow you to change  the

value  associated  with the F1 key temporarily. In this

example, you would enter a new value for 155.055. You can

change any of the function key frequencys by pressing the

shift key at the same time as the function key you want

to change.

<ctl>  F1 - If you press the control key at the same time

that  you  press  any function key,  you  can  change  the

frequency within that function key, and your changes will be

saved to disk.

PRO-CAT CRT Display (continued)

<shift> L - If you press the shift key at the same time as

you press the letter L , whatever frequency is displayed

on the FRG9600 at the time will be locked out of the scan

sequence.

<shift> C - If you press the shift key at the same time as

you press the letter C then all of the locked out frequencys

will be cleared.

<shift> S - If you press the shift key at the same time as

you press the letter S then you will change the scan step

of the scan. Steps of 5khz, 10khz, 25khz, and 100khz are

available.

<shift> m - If you press the shift key at the same time as

you press the letter m then you will change the mode  used

in the scan. The program defaults to FM-N, except when the

frequency is in the aircraft band.

q - Allows you to quit to DOS.

Press the cursor right key -> to see these commands

on the video display main menu.

The Scan Information File

Scanning frequencys and descriptions are located in a file on

on  disk  called CATBASE.DAT. This file has to be  located  on  a

floppy diskette in drive A: or B: for the program to find it. You

can use the DOS ASSIGN command to direct the program to drive C.

Here is a sample command       >>>  C>assign a=c   <<<

The  file has 100 records, you can initialize just one record or

the entire file using the database menu. When you initialize the

file or a record you erase its frequency information.

Updates to The Scan Information File

(The Database Menu)

Enter the Database menu by pressing the letter e from the main menu.

Option  1 - If you choose this option you can update any  one  of

the 100 records. First you select the record by holding down  the

space bar until the requested record is displayed, then press the

enter key. You will then see all the frequencys available on that

record. The frequency you can change will be colored RED. As  you

change each digit it will turn BLUE. You can backspace to correct

any errors. After entering frequency 10, key in a description for

this  group of frequencys. Lower case letters look  great,  press

the enter key to finish.

Option 2 - This will allow you to clear one record, or the  whole

file if you prefer.

Option 3 - This option calls up the Configuration Options Menu.

Option 4 - Prints the scan information  file  active  records,

followed by a record cross reference to help you remember  record

numbers when you're using the program.

The Configuration Options Menu

Here  is the the last thing as well as the most  important  thing

you  need  to  do  to get your  lazy  FRG9600  receiver  scanning

frequencys  at a rate of 30 frequencys (or better!)  per  second.

Any  program can make a rough guess of how fast a  computer  it's

running on, but you can do a much better job of it. The idea is

simple. The  program  has  the default delays set for an IBM AT

style computer running at 8 Mhz. If you have a XT style computer,

then the datacomm delay will have to be reduced a little at a time

until the program  scans fast enough to suit your taste. If you set

these delays  to zero, however, rest assured that the FRG9600 will

not be  able to handle the rate at which commands are sent from the

computer. If your computer has an adjustable clock speed, set  it

to  its slowest setting before adjusting delays, that  will  keep

computer generated noise to a minimum.

DATACOMM  DELAY  - This delay determines how much of  a pause

to use between frequency set commands to the FRG9600. This

time is used to check the squelch for a possible signal on the

frequency.

CONVERSATION  DELAY  -  When  PRO-CAT is scanning, and finds a

signal on a frequency it has set, it will stop there, and resume

scanning the instant the signal drops below the squelch level.

This delay determines how long to stay on the frequency after

the signal drops below the signal level, in case the signal comes

back on frequency.

BLINK DELAY - When the program wants you to enter a value on  the

keyboard,  it will flash the FRG9600 display at you. If  you  are

entering a lower limit for example, the frequency will  alternate

between  60.000  Mhz  first and then 999.999  Mhz.  Upper  limits

alternate  between  905.000  Mhz  and  999.999  Mhz.  This  delay

determines  how  quickly  these  numbers  alternate.  Using  this

system,  you don't have to turn on your computer  system  monitor

every time you want to scan.

COMMUNICATION  PORT  -  This option allows you to  use  a  second

serial  port for your FRG9600 interface, in case your COMM1  port

is being used for another purpose.

HARDWARE CONNECTIONS

1.  Connect  your  computer's serial port  to  the  Yaesu  FIF232

interface device using a RS232 standard interface cable. This

cable  has a 25 pin connector on each end. Use this  cable,

and connect the unmodified DIN cable that goes from the FIF232

to the FRG9600. You should be able to change the displayed

frequency on the FRG9600 by pressing a computer function key

when PRO CAT is running. If the frequency on the FRG9600 display

doesn't change when you press a function key, modify the RS232

cable as follows. On the female connector, short pins

4 and 5  togather first, and then short pins 6, 8 and 20

togather. The  pins are numbered on the connector.

2.  There is also a DIN connector cable that goes from the Yaesu

FIF232 interface device to the FRG9600. You can either splice

into this cable for the joystick connection, or you can make

the joystick connection at the plug that fits into the FRG9600.

Connect the joystick lead that goes to pin  4 on  the

joystick  male connector  (ground) to pin 1  on the  CAT

plug (also  ground). Then  connect the joystick lead that

goes to pin 2 on the joystick male connector (normally the

joystick #1 fire button) to pin 6 on the CAT plug (BUSY).

This will allow the computer to know the FRG9600 squelch status.

o x o x o o o o

o o o o o o o

15 Pin Joystick Connector Plug

You are looking at the pins of the male joystick connector.

Pin 2 is the x on the top row to the left. (To CAT 6).

Pin  4 is on the top row, fourth from the left. (To CAT 1).

#################################################################################################################