Shortwave Broadcast Bands and Amateur Radio HF Band Limits

Short Wave Broadcast Bands:

120 meters:   2300 to 2500 kHz

90 meters:   3200 to 3400 kHz

75 meters:   3900 to 4000 kHz

60 meters:   4750 to 5060 kHz

49 meters:   5950 to 6200 kHz

41 meters:   7100 to 7300 kHz

31 meters:   9500 to 9775 kHz

25 meters:  11700 to 11975 kHz

19 meters:  15100 to 15450 kHz

16 meters:  17700 to 17900 kHz

13 meters:  21450 to 21750 kHz

11 meters:  25600 to 26100 kHz

These are the major frequency bands that are used by short wave broadcasters.  The 120 to 41 meter bands are usually most active after dark and 31 to 11 meters are active during hours of daylight.

Amateur Radio HF Band Limits:

Key -

- = Frequency range not allowed for class of license

N = Novice <CW only>

C = CW, FSK

V = CW, SSTV, FAX, Voice

A = CW, SSB

10 Meters -

Freq MHz 28     28.1    28.2     28.3  28.5  29   29.3   29.5 29.6   29.7

|      /       /         /     /                /              \

Novice   -------CCCCCCCCCCCCCCCCCCAAAAAA---------------------------------

General  CCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

Advanced CCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

Extra    CCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

12 Meters -

Freq MHz 24.890                     24.930                        24.990

/                          /                                  \

General  CCCCCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

Advanced CCCCCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

Extra    CCCCCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

(Amateurs must avoid interference to the fixed service outside the U.S.)

15 Meters -

Freq MHz  21.0   21.025        21.1      21.2     21.225    21.3  21.450

/      /             /         /        /         /          \

Novice    ---------------------NNNNNNNNNN-------------------------------

General   -------CCCCCCCCCCCCCCCCCCCCCCCC-------------------VVVVVVVVVVVV

Advanced  -------CCCCCCCCCCCCCCCCCCCCCCCC---------VVVVVVVVVVVVVVVVVVVVVV

Extra     CCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

20 Meters -

Freq MHz  14.0     14.025          14.150    14.175     14.225   14.350

/        /               /         /          /             \

General   ---------CCCCCCCCCCCCCCCC---------------------VVVVVVVVVVVVVVV

Advanced  ---------CCCCCCCCCCCCCCCC----------VVVVVVVVVVVVVVVVVVVVVVVVVV

Extra     CCCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

30 Meters -

Freq MHz      10.1                                               10.150

/                                                       \

General       CCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCC

Advanced &

Extra         (CW & FSK Only -  Maximum Power 200 Watts Output PEP)

40 Meters -

Freq MHz  7.0     7.025          7.100       7.150      7.225     7.300

/       /              /           /          /             \

Novice    -----------------------NNNNNNNNNNNN--------------------------

General   --------CCCCCCCCCCCCCCCCCCCCCCCCCCC-----------VVVVVVVVVVVVVVV

Advanced  --------CCCCCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVV

Extra     CCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVV

** Phone operation is allowed on 7075-7100 KHz in Puerto Rico, U.S.

Virgin Islands and areas of the Carribean south of 20 degrees north laditude; and in Hawaii and areas near ITU Region 3, including Alaska.

75/80 Meters -

Freq MHz  3.50    3.525        3.7    3.75    3.775      3.850      4.0

/       /            /      /       /          /            \

Novice    ---------------------NNNNNNN---------------------------------

General   --------CCCCCCCCCCCCCCCCCCCC-------------------VVVVVVVVVVVVVV

Advanced  --------CCCCCCCCCCCCCCCCCCCC--------VVVVVVVVVVVVVVVVVVVVVVVVV

Extra     CCCCCCCCCCCCCCCCCCCCCCCCCCCCVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

160 Meters -

Freq MHz  1.8                                                      2.0

/                                                          \

General,  VVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

Advanced  &

Extra

Amateur stations operating at 1900-2000 KHz must not cause harmful interference to the radiolocation service and are afforded no protection from radiolocation operations; See January 1986, issue of QST in Happenings for details.

*************

Operators with Technician class licensees and above may operate on all amateur bands above 50 MHz.  For more details see the FCC rulebook.

**************

5167.5 KHz Alaska emergency use only.  (SSB only) E,A,G,T,N

**************

6 Meters -

Freq MHz   50.0    50.1             52.0                53.0       54.0

/        |                                                 \

E,A,G,T    CCCCCCCCCCBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBB

2 Meters -

Freq MHz  144.0      144.1          145.0       147.0            148.0

/            |                                             \

E,A,G,T   CCCCCCCCCCCCCCBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBB

1 1/4 Meters -

Freq MHz  220.0           221.0          222.1        223.91     225.0

/                                /            \            \

Novice    --------------------------------BBBBBBBBBBBBBBB-------------

E,A,G,T   BBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBB

70 cm -

Freq MHz   420.0                                                  450.0

/                                                          \

E,A,G,T    BBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBB

33 cm -

Freq MHz   902.0                                                  928.0

/                                                          \

E,A,G,T    BBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBBB

U.S. Amateur Power Limits:

At all times, transmitter power should be kept down to that necessary to carry out the desired communications.  Power is rated in watts PEP output.  Unless otherwise stated the maximum power output is 1500 Watts.  Power for all license classes is limited to 200 watts in the 10,100 - 10,150 KHz band and in all Novice subbands below 28,100 KHz.  Novices and Technicians are restriceed to 200 watts in the 28,100 - 28,500 KHz subband.  In addition, Novices are restricted to 25 watts in the 222.1 - 223.91 MHz subband and 5 watts in the 1270 - 1295 MHz subband.

Please feel free to post this file on other bulletin board systems.  Permission to use is granted provided credit is given to the source.

This and more Ham and SWL files can be found on the -

Digital Newsletter BBS

612/HAM-0000

300/1200

24 Hours

PC Pursuitable @ MNMIN

Sysop - John Desmond  K0TG

Packet @ WB0GDB - TCPIP ID 44.94.3.7

The Digital Newsletter is an open system and is supported mainly by the Sysop and donations by users