Skylab, Americas first manned space station, was designed to study the ability of humans

to live and work in space over a prolonged period of time while conducting scientific

observation of the Earth and Sun.  Created during the heyday of the Apollo program,

Skylab was assembled from the upper-stage of a Saturn V rocket, the booster used to

launch Apollo spacecraft.  This same launch vehicle was used to put the 84 ton station into

an orbit measuring 275 by 269 miles at 50 degree inclination.

The space station consisted of an orbital workshop 48 feet long and 21 feet in diameter

where astronauts could eat, sleep, exercise, and work.  A smaller docking adapter with an

airlock gave astronauts access to docked spacecraft and outer space.  A telescope mount

on the docking adapter contained solar telescopes and Earth sensors; it also supported

four solar arrays in a windmill configuration.

Six photovoltaic arrays were to supply electricity to the station; however, during liftoff

one large array was ripped off the side of the ascending Skylab along with a

micrometeorite shield, intended to protect the living space from the heat of the Sun.

Further, the remaining large solar wing was stuck, leaving the station with very little

power.

Without its protective shield, the interior temperature of the station reached a dangerous

190 F.  Skylabs first crew deployed a replacement sunshade made of nylon, mylar, and

aluminum foil which cooled the orbital workshop.  Two astronauts, Conrad and Kerwin,

made a space-walk to release the stuck solar wing to supply more power.  After making

the station habitable again, the crew conducted research on the Sun, capturing the first

pictures of solar flares ever made from above the atmosphere.

Three crews of three men each inhabited Skylab, ferried to the space station in Apollo

Command Modules launched on Saturn 1-B rockets.  These crews made the first truly

long-duration missions in the American space program, acquiring experience on the

problems of life in zero-g.  Motion sickness hit four of the nine crewmen while in orbit,

and sleeping was difficult due to noise made by the space station.  Because of the

workshops low atmospheric pressure, crewmembers had to shout to be heard.

After two of four thrusters on their Apollo Command Module failed, the second Skylab

mission was almost aborted by Mission Control.  However, a plan was devised for the safe

return of the astronauts using the two good thrusters, and the astronauts stayed in orbit

for several weeks.  They collected data on land and water resources as well as  performing

experiments on the effects of microgravity.

The third crew of Skylab consisted of three rookies.  The crew felt they were being

pushed too hard by Mission Control and were uncooperative for some time.  Eventually,

they conducted much scientific research, including experiments in making metal alloys in

zero-g.  They observed the comet Kohoutek as it approached the Sun, and conducted

more than 22 hours of Extra-Vehicular Activity, including a 7 hour EVA on Christmas

Day, 1973.

Five years after the last crew left, Skylab descended back toward Earth.  On July 11,

1979, the space station burned up while entering the Earths atmosphere; some debris

scattered over the Indian Ocean and western Australia.  The project gathered an immense

amount of scientific data, including more than 750,000 pictures of the Sun and 46,000

photographs of the Earth, while providing essential information on humans living in space.