NFS Gateway 1.2 Update for NW 3.x            (GWY195.EXE)

---------------------------------------------------------------

This package is made up of the following files:

GWY195 DOC15HB5 - This document

GWY195 EXE15HB5 - Binary self-extracting file.

---------------------------------------------------------------

NOVELL PRODUCT and VERSION: NetWare NFS Gateway 1.2

ABSTRACT:

Accumulated bug fixes for NetWare NFS Gateway.  Read the

installation instructions INSTALL.TXT when installing.  Read

the supplement FEATURES.TXT to understand new features and

parameters.

---------------------------------------------------------------

DISCLAIMER

THE ORIGIN OF THIS INFORMATION MAY BE INTERNAL OR EXTERNAL TO

NOVELL.  NOVELL MAKES EVERY EFFORT WITHIN ITS MEANS TO VERIFY

THIS INFORMATION.  HOWEVER, THE INFORMATION PROVIDED IN THIS

DOCUMENT IS FOR YOUR INFORMATION ONLY.  NOVELL MAKES NO

EXPLICIT OR IMPLIED CLAIMS TO THE VALIDITY OF THIS INFORMATION.

---------------------------------------------------------------

SYMPTOM

1)  Server abend with GPPE when using Wordperfect 5.1 in

Windows 3.11. Windows and Wordperfect are installed on a

NetWare volume.

Steps to reproduce:

a. Start Wordperfect.

b. Open a file which resides on a gateway volume.

c. Click on Open again to open two submenus.

d. From one of the submenus double click on ".."  to go up one

directory.

e. The server abends with a GPPE error.

Also, the GPPE can occur by using the DOS command "cd ..".

2)  Documentum, or other applications which create files on the

NFS server through some other means than the Gateway itself,

is intermittently unable to access newly created files.  User

gets "file not found".  In the case of Documentum, the

workstation sends TCP messages directly to an AIX box running

Oracle.  Oracle and UNIX function calls (fcreate, fopen, fread,

fwrite) create a temporary directory called /DATA/COMMON which

lasts for duration of Documentum session.  The NFS Gateway may

not always see these files even if /ETC/NWPARAMS contains

"DIR_COMP_TIME = 0".

This problem can be recreated without Documentum as follows:

a. UNIX:  mkdir tst/d1

b. UNIX:  cp /etc/hosts tst/d1/f1

c. DOS:  DIR TST/D1 /S

d. DOS:  TYPE TST/D1/F1

e. UNIX:  rm -rf tst/d1

f. UNIX:  mkdir tst/e1 ( e1 should have the same i-node as d1 )

g. UNIX:  cp /etc/hosts tst/e1/f1 ( f1 should have the same i-

node as previous f1 )

h. DOS:  TYPE TST/E1/F1 ====> THIS SHOULD FAIL

i. UNIX:  rm-rf tst/e1

j. UNIX:  mkdir tst/g1 (g1 should have the same i-node as d1

and e1 )

k. DOS:  CD G1 ====> THIS SHOULD FAIL

3)  Cannot mount NFS server running on OS/2 because UNICON

prevents user from typing "\".  Changed to allow "\" character.

It is no longer necessary to manually edit the SYS:ETC/NWPARAMS

file.

4)  Installation of NFS Gateway 1.2 increases allowed

connections, which is shown in SYSCON and other NetWare

administration utilities.  For example, a 100-user license of

NetWare would appear as 250-user after installing NFS Gateway

1.2.  May cause problems with some applications which examine

the maximum connection allowed, such as ARCSERV.

5)  Can't read/copy files from SJF-NFSGW to local drives even

though files have read/write permissions.

6)  Abend: nfsgw (gwch read_data_ release func-release()).

Intermittent problem which occurs when gateway is under a load

greater than 40 workstations.  There was small window of time

for error in the synchronization of a write and read request

after a lock request.

7)  Workstation connection can become hung.  Hung connection

cannot be removed through MONITOR.NLM or any means other than

rebooting the server.  Fixed for known circumstances.

8)  Abend: DeleteFromHash was called with directory entry

number.

9)  Cannot copy files from the Gateway volume to local drive

even though files have read and write permissions.

10)  Certain applications, including MHS router, CC:Mail and MS

Mail, and MS Access applications cannot read files on the SYS:

volume.  They appear to be locked whenever NFSGW is loaded.

CC:MAIL programs (such as ADMIN.EXE) get message "Cannot Unlock

Database File."

11)  After heavy use of the Gateway, some files in a directory

are marked as hidden even if the file names do not start with a

leading period (.).

Steps to reproduce:

a. UNIX:  touch .xyz

b. DOS:  NDIR XYZ  (The Gateway uses carat () to indicate

hidden files.)

c. UNIX:  mv .xyz abc

d. DOS:  NDIR ABC

e. note that ABC it is marked as hidden.

12)  Windows File Manager cannot copy a sub-directory from

local disk to a drive letter mapped to Gateway volume using

drag and drop.  Error is "cannot create file/directory".

Steps to reproduce:

From DOS:

a. touch xyz

b. flag xyz ro

c. flag xyz ro

d. del xyz

From File Manager:

e. Drag the c:\dos directory to drive mapped to a Gateway

volume.  This should give an error.

NOTE: During the above test, the Gateway should not be used by

anyone else.

13)  Cannot access a remote directory if it was created with

the same inode number as a directory recently deleted from

UNIX.

14)  A filename is changed on gateway volume where NFS server

is running on IBM OS/2 version 2.1.  If correct filename

contains non-numeric characters, the character "0" is appended

to filename.  If it consists of all numeric characters, the

file name is "incremented" by one.  The correct name is

restored when volume is re-mounted.  NFS Gateway is attempting

to avoid a duplicate name collision when this is not necessary.

Problem occurs because some OS/2 systems support only upper

case, at least as seen through the NFS daemon of OS/2.  If NFS

Gateway user creates file "STUB" the Gateway will create the

file "stub" on the NFS server (assuming that most users will be

UNIX users).  OS/2 saves the file as upper case STUB".  Now

when the Gateway user does a DIR, the Gateway detects that it

already has "STUB" in its cache or shadow file, so to avoid

duplicates it displays "STUB0".

With this fix, when the volume is first mounted it will attempt

to create a lower case scratch file, then read it back to test

whether the file has been on upper or lower case on the NFS

server.  If upper, then the NFS server only supports upper case

filenames.

NOTE: The exported directory of the remote NFS volume must be

writable by the UID used for mounting, or the scratch file test

will fail.

NOTE:  A related tunable parameter UPPER_CASE has been added to

the SYS:ETC/NWPARAMS file.  It forces the NFS Gateway to create

only upper case file and directory names.  See the FEATURES.TXT

for details.

15)  Gateway time stamps incorrect for a while after DOS COPY.

To reproduce, copy some files from C: drive onto Gateway drive.

Use DIR to see time stamp.  Time stamp reflects  the server

system time for an unpredictably long time.

16)  Need DBCS Character support.  NFS Gateway 1.2J  running on

NW 3.12J does not support DBCS on NW volumes.

17) Source code error found during code review.

18)  On Gateway volume with directories with up to 15 levels of

subdirectories, the deletion of  directories via DELDIR or

FILER is fast when there are 5 levels or less.  More than 5

levels incurs a delay of up to 10 seconds for each level.

19)  File is truncated to some fixed length when copying files

in non-packet burst mode from a Gateway volume.  The file is

large and the NFS transfer size (NFS_XFER_SIZE) is set  less

than 8k.

SOLUTION

Apply GWY195.EXE

Self-Extracting File Name:  GWY195.EXE     Revision:  A

Files Included     Size     Date      Time

\

GWY195.TXT         (This File)

NFSGW.NL_     227007   12-02-94    5:26p

SPACE.DAT         10   02-01-93    3:58p

PFILES.DAT        388   01-23-95    3:19p

GWADMIN.NL_      19607   12-02-94    5:28p

INSTALL.TXT       3116   01-27-95    3:34p

PCONFIG.DAT         64   12-02-94    2:55p

PCONFIG.HLP       2183   07-29-92    1:27p

PCONFIG.NLM      15934   09-17-93    6:03p

FEATURES.TXT       4163   01-27-95    3:36p

PINSTALL.HLP       6798   06-21-93    2:53p

PINSTALL.NLM     130422   11-08-93    6:39p

P_UNINST.HLP        892   07-27-92   12:02p

P_UNINST.NLM      72042   11-08-93    6:40p

Installation Instructions:

See INSTALL.TXT.

Solution Specifics:

GWY195.EXE corrects all of the listed symptoms.

---------------------------------------------------------------

Any trademarks referenced in this document are the property of

their respective owners.  Consult your product manuals for

complete trademark information.

---------------------------------------------------------------