OASAS I Version 2.01.15 CSD

===========================

This package is made up of the following files:

OAS20115 DOC19AJ4 - This document

OAS20115 EXE19AJ4 - Binary self-extracting file.

The file once downloaded with the binary option needs to be run as

it is self-extracting.  Please refer to the READEM before installing

this CSD.  This has been included below for your reference.

OASAS I INSTALLATION README FILE

================================

***********OASAS I INSTALLATION README FILE*******************

IBM 320 MB SCSI drives shipped in PS/2 model 95's or PS/2 3511 External

Enclosures for SCSI Devices have the recommended barcode level.

IBM 320 MB drives that do NOT have the recommended barcode level were

shipped only with the PS/2 Models 80-121 and 80-A31 base system units and

as Model 80 options.

NOTE:

----

For IBM 320 MB SCSI media, a barcode level of "L1 An5 nnnnnn" where "n"

is any hexadecimal number from 0 to F, and a value of "5" (or greater)

in the position shown above is recommended.

This level number is under the barcode on the top front of the drive

in the format of "L1 Ann nnnnnn".  For example, a barcode of

"L1 A11 003306" would have a barcode level less that the recommended

value.

********************TABLE OF CONTENTS************************

DEINSTAL.EXE - Used to de-install the OASAS I software.

INSTALLI.CMD - Used for initial installation of the OASAS I Disk Array

Management Software.

OAS.MSG      - OASAS I messages

OASADMIN.EXE - This is a menu driven control panel for the OASAS I

software.

Note: All messages displayed by any part of the OASAS

I system are from OAS.MSG or OASH.MSG.

OASAS.FLT    - The OASAS I array driver.

OASCNFG.EXE  - Program for configuring or re-configuring an OASAS I

array.

OASCNVT.EXE  - Used to convert an OASAS I array RAID Level.

Converts RAID5 EXT to (R)AID0 EXT and vice versa.

OASFMT.EXE   - Program for formatting an OASAS I array.

OASH.MSG     - Help for OASAS I messages

OASLOG.EXE   - This program handles the logging of information sent

by the OASAS I utilities.  OASLOG.EXE may not be

called from the commland line.  Information is logged

in OASAS.LOG.

OASMON.EXE   - Program for handling OASAS I driver messages.

OASRECRT.EXE - Program for recreating the data of a down OASAS I array

array drive to a replacement drive.

OASETUP.EXE  - Program called by INSTALLI.CMD to make OASAS I an

integral part of OS/2.

OASSCAN.EXE  - Used to verify ECC blocks on an array.