NetWare Client for OS/2 version 2.11 (LAN DRIVERS) (WSDRV1.EXE)

---------------------------------------------------------------

This package is made up of the following files:

WSDRV1 DOC09DA5 - This document

WSDRV1 EXE09DA5 - Binary self-extracting file.

---------------------------------------------------------------

NOVELL PRODUCT and VERSION: NetWare Client for OS/2 2.11

ABSTRACT:

The complete product consists of the following seven files:

(WSOS21.EXE) contains the installation program, (WSOS22.EXE)

contains the requester files, (WSOS23.EXE) contains the Windows

libraries and RPL files, (WSDRV1.EXE) contains the LAN drivers,

(OS2UT1.EXE) contains LOGIN, NLIST, MAP, CX, SYSCON utilities,

(OS2DC1.EXE) contains the electronic documentation in OS/2 IPF

format, and (OS2VMB.EXE) contains the VLM Boot diskette image.

---------------------------------------------------------------

DISCLAIMER

THE ORIGIN OF THIS INFORMATION MAY BE INTERNAL OR EXTERNAL TO

NOVELL.  NOVELL MAKES EVERY EFFORT WITHIN ITS MEANS TO VERIFY

THIS INFORMATION.  HOWEVER, THE INFORMATION PROVIDED IN THIS

DOCUMENT IS FOR YOUR INFORMATION ONLY.  NOVELL MAKES NO

EXPLICIT OR IMPLIED CLAIMS TO THE VALIDITY OF THIS INFORMATION.

---------------------------------------------------------------

ISSUE

This product contains the following enhancements:

*    OS/2 Warp support

*    More robust Windows application support

*    Ability to run NetWare Administrator GUI utility in

private Windows session

This product also includes the following:

*    Support for Remote Boot (RPL)

*    Private and global DOS/MS Windows support

*    IPX/SPX support for enhanced mode MS Windows applications

*    Automated ODINSUP setup

*    Documentation in native OS/2 format (costs less to

download and runs with the OS/2 book display program

already on your system

The NetWare Client for OS/2 version 2.11 electronic

documentation is now available in native OS/2 format.  Because

the native OS/2 formatting does not require much disk space,

the documentation is now available as a standard part of the

client kit download rather than as a separate download from

NetWire.

Self-Extracting File Name:  WSDRV1.EXE     Revision:  C

Files Included     Size     Date      Time

\

WSDRV1.TXT         (This File)

WSDRV_1       9617   04-14-94   12:59p

\OS2\

NE2.SY_      10820   12-08-92    1:55p

3C503.SY_       9413   07-10-92    9:53a

3C523.SY_       8140   09-17-92   11:22a

ROUTE.SY_       2963   08-12-93   10:58a

TCNSR.SY_       6744   09-22-92   10:35a

TOKEN.SY_      13727   11-17-93    1:22p

E21ODI.SY_       9261   01-18-93   11:14a

E31ODI.SY_       8973   01-18-93   11:42a

ES3210.SY_      10202   09-01-92    3:54p

ILANAT.SY_      10080   10-06-92    1:27p

LANSUP.SY_      12686   05-14-93    4:20p

NE2_32.SY_      10730   11-23-92    4:49p

NE1000.SY_      10405   11-23-92    4:40p

NE2000.SY_      10931   12-15-92    6:48p

NE2100.SY_      10729   08-20-92    4:15p

NE3200.SY_      12607   11-17-93    1:26p

NI5210.SY_       9818   09-22-92    9:17a

NI6510.SY_       9504   09-15-92    3:24p

NI9210.SY_       9961   09-02-92    3:07p

SMCARC.SY_       9308   10-08-92   12:07p

T20ODI.SY_      26978   09-02-92    9:33a

T30ODI.SY_      27041   08-13-92   10:52a

TCCARC.SY_       6738   09-15-92   10:33a

TCCTOK.SY_      11840   08-10-92    1:31p

TRXNET.SY_       9478   11-04-92    2:56p

DRIVERS.OS2        594   04-14-94    9:44a

NCRWL04.SY_      11462   01-22-93    1:24p

NE1500T.SY_      10704   08-20-92    4:15p

NTR2000.SY_      13487   03-02-94    3:06p

ODINSUP.SY_       6338   11-11-92   11:45a

REQ89XR.SY_      14076   08-26-92    8:35p

REQ391R.SY_      14028   12-21-92    3:25p

REQ392R.SY_      13952   08-05-92    4:54p

REQ990R.SY_      13861   07-16-92   12:37p

SMC8000.SY_      16739   12-09-92    8:58a

SMC8100.SY_      21433   01-25-93    3:25p

SMCARC2.SY_       9323   11-09-92   10:09a

TRXNET2.SY_       9482   11-04-92    2:57p

EXP16ODI.SY_      13172   10-14-92   11:18a

HPISAODI.SY_      11336   09-01-92    1:19p

HPMCAODI.SY_      10631   09-01-92    1:16p

INTEL593.SY_      10561   12-09-92   11:21a

INTEL596.SY_      11530   12-09-92   11:33a

MADGEODI.SY_      50090   09-14-92   11:38a

TCE16ATR.SY_      11002   08-10-92    1:43p

TCE32MCR.SY_      11083   08-10-92    1:40p

Installation Instructions:

INSTALL INSTRUCTIONS for the NetWare Client for OS/2 version

2.11

This client product supports all OS/2 2.x releases made before

December 19, 1994, and replaces all previous 2.x versions of

the NetWare Client for OS/2.

NOTE: If you have OS/2 1.3, you need to obtain the NetWare

Requester 1.3 for OS/2 instead of this product.

The NetWare Client for OS/2 version 2.11 consists of seven

self-extracting files: WSOS21.EXE contains the installation

program WSOS22.EXE contains the requester files WSOS23.EXE

contains the Windows libraries and RPL files WSDRV1.EXE

contains the LAN drivers OS2UT1.EXE contains LOGIN, NLIST, MAP,

CX, SYSCON utilities OS2DC1.EXE contains the electronic

documentation in OS/2 IPF format OS2VMB.EXE contains the VLM

Boot diskette image

Once these files are downloaded, there are 3 methods that can

be used to complete the installation of the NetWare Client for

OS/2 version 2.11:

Method 1: Extract to Network Drive/Install From Network Drive

Method 2: Extract to Hard Drive/Make Diskettes/Install From

Diskettes

Method 3: Extract to Diskettes/Install From Diskettes

Instructions for each method follow.

*********

Method 1: Extract to Network Drive/Install From Network Drive

NOTE:     You must have a version of the NetWare Client

software already installed on your workstation and be connected

to the network in order to use this install method.  You cannot

install to and from the same drive. Example: You cannot install

from C: to C:

1.   To install the NetWare Client software to your hard disk,

you must have adequate disk space to allow for the client to

install. (about 4 MB)

2.   At the network drive, create a directory called TEMP.

This directory can be deleted later.

Example:  At the J:\ prompt, type

MD TEMP

3.   Extract WSOS21.EXE, WSOS22.EXE, WSOS23.EXE, WSDRV1.EXE,

OS2UT1.EXE, and OS2DC1.EXE into the TEMP directory.  The self

extracting file will automatically create the required

subdirectories.  Change to the directory where WSOS21.EXE,

WSOS22.EXE, WSOS23.EXE, WSDRV1.EXE, OS2UT1.EXE, and OS2DC1.EXE

reside.  DO NOT change to the TEMP directory.

Example:  At the command line, type

WSOS21 J:\TEMP\

Do this for each self extracting file in turn.  This will

extract all the files into the TEMP subdirectory.

4.   In an OS/2 Command Line Session, CD into the TEMP

directory and type INSTALL.  The NetWare Client for OS/2

Installation screen will appear. Follow the instructions that

appear on the screen to install the NetWare Requester on the

workstation. NOTE: Make sure and place the local disk drive

letter as the target directory.

5.   Delete your TEMP directory if desired.

********************

Method 2: Extract to Diskettes/Install From Diskettes

1.   Format six 3.5" high density double-sided diskettes.

2.   Label each diskette with the following names:

Disk 1    WSOS2_1

Disk 2    WSOS2_2

Disk 3    WSOS2_3

Disk 4    WSDRV_1

Disk 5    OS2UTIL1

Disk 6    OS2DOC_1

Example:  At the command line, type

Label A: WSOS2_1

2.   Extract the .EXE files to their corresponding diskettes.

The correct directory structure will be created automatically

on the diskettes. Self-extract WSOS21.EXE onto WSOS2_1,

WSOS22.EXE onto WSOS2_2, etc.

Example:  At the command line, type

WSOS21 A:

3.   After creating the six diskettes, insert the disk labeled

WSOS2_1 diskette in drive A.

4.   Open the drive A: icon on your desktop and open the

INSTALL.EXE program. The NetWare Client for OS/2 install screen

appears.

5.   Follow the instructions on the screen or in the manual to

complete the install.

******************

Method 3: Extract to Hard Drive/Make Diskettes/Install From

Diskettes

1.   Create the following subdirectories on your hard disk:

WSOS2_1, WSOS2_2, WSOS2_3, WSDRV_1, OS2UTIL_1, and OS2DOC_1.

Example:  At the C:\ prompt, type

MD WSOS2_1

2.   Extract the .EXE files into their corresponding

subdirectories, the correct directory structure will be created

in each subdirectory.  WSOS21.EXE into WSOS2_1, WSOS22.EXE into

WSOS2_2, etc.

Example:  At the command line, type

WSOS21 C:\WSOS2_1\

3.   Format six 3.5" high density double-sided diskettes.

4.   Label each diskette with the following names:

Disk 1    WSOS2_1

Disk 2    WSOS2_2

Disk 3    WSOS2_3

Disk 4    WSDRV_1

Disk 5    OS2UTIL1

Disk 6    OS2DOC_1

Example:  At the command line, type

Label A: WSOS2_1

5.   Insert diskette labelled WSOS2_1 into drive A:.

6.   Change directory to WSOS2_1 on your hard disk.

7.   Type the following at the command line:

XCOPY *.* /S /E /V A:

8.   Repeat steps 6 and 7 for the remaining five diskettes.

Ensure that the files are copied from the correct directory to

the corresponding labelled diskettes.

9.   After creating the six diskettes, insert WSOS2_1 diskette

in drive A.

10.  Open the drive A: icon on your desktop and open the

INSTALL.EXE program. The NetWare Client for OS/2 install screen

appears.

11.  Follow the instructions on the screen or in the manual to

complete the install.

**********

SPECIAL INSTRUCTIONS FOR VLM BOOT

OS2VMB.EXE contains a DOS boot image file and a Novell DOS 7

image utility (DISKCOPY.COM).

IMPORTANT:  DISKCOPY.COM can only be run from an actual DOS

workstation, not from an OS/2 DOS session.

Instructions for Creating the VLM Boot Diskette:

1.  On the actual DOS workstation, create a subdirectory named

VLMBOOT.

2.  Copy OS2VMB.EXE to the newly created VLMBOOT directory.

3.  Extract OS2VMB.EXE by running it.

4.  Insert a 1.44 MB formatted diskette into drive A:.

5.  Change to the VLMBOOT subdirectory.

6.  Create the image disk by typing DISKCOPY VLMBOOT.350 A:

7.  For more information about VLM Boot, refer to the NetWare

Client for OS/2 version 2.11 User Guide in both printed and

online formats.

---------------------------------------------------------------

Any trademarks referenced in this document are the property of

their respective owners.  Consult your product manuals for

complete trademark information.

---------------------------------------------------------------