TCP/IP to BBS, BBS to TCP/IP version 1.10 of WA3PXX PXX Gateway.

Once the gateway is established any "local users" users should specify

the gateway station as the default SMTP gateway.

Users may:

1      Send mail to any user connected via existing BBS store-n-forward

WA7MBL/RLI links.

2      Get mail delivered directly to there personal computer.

3      Get current WA7MBL Bulletins delivered directly to users

via SMTP.

4      Create WA7MBL style bulletins from SMTP mail messages.

5      Create MBL/RLI style Private (SP) messages from SMTP.

To send a message to a user W3XYZ on a hostname W3IWI via BM.EXE :

To: W3XYZ@W3IWI                 { Since each user defines the gateway}

{ as your station...The mail gets }

{ shipped out to the BBS }

or

To: W3XYZ%W3IWI@WA3PXX

{  Where WA3PXX is the specified }

{  gateway to PBBS world.        }

To get bulletins delivered directly to your PC ask your PXX Gateway

SYSOP to add your call ("host") to the FORWARD.LST file.

To send mail to a TCP/IP user via the PXX Gateway.  Assuming the gateway

serving you is WA3PXX have users send mail to you at WA3PXX.  Just as you

would with a traditional BBS.

To send a WA7MBL Bulletins by a TCP/IP :

Specify [B] as the first 3 characters of a Subject.  This will force the

whole message to become a Bulletin as it is passed to the BBS.

To send a "private" message specify [P] as the first 3 characters of the

subject.