CONNECTEUR DB30 MINIATURE CASIO FX850P

Pierre GIRALT / William SAINT_CRICQ / ENIT Tarbes

N�Broche            Signal                        IN/OUT

1              _____________________              __

2              DSR signal from RS232              IN

3              CD signal from RS232               IN

4              CTS signal from RS232              IN

5              _____________________              __

6              Busy signal from PRT               IN

7              _____________________              __

8              _____________________              __

9              DATA IN signal RS2323              IN

10             INIT signal to PRT                 OUT

11             STROB signal to PRT                OUT

12             DTR signal to PRT                  OUT

13             RTS signal RS232                   OUT

14             DATA OUT signal RS232              OUT

15             RESET Output                       OUT

16             Data Bus                           IN/OUT

17             Data Bus                           IN/OUT

18             Data Bus                           IN/OUT

19             Data Bus                           IN/OUT

20             Data Bus                           IN/OUT

21             Data Bus                           IN/OUT

22             Data Bus                           IN/OUT

23             Data Bus                           IN/OUT

24             Adresse Bus                        OUT

25             Adresse Bus                        OUT

26             Adresse Bus                        OUT

27             Output enable signal               OUT

28             Write enable signal                OUT

29             CHIP select to FA6                 OUT

30             MASSE

_____________________________________________________________

Vue ext�rieure DB30

_____________________

\1 3 5 .......... 29/

\2 4 6 .........30/

~~~~~~~~~~~~~~~~~

_____________________________________________________________

(c) Wtel