Turbo BBS Version 1.00 - System Operator Notes - Page 1

1 - Features:

- Self-contained Message and Files system written in Pascal;

- Program not operating system dependent;

- Information files editable with WordStar;

- Variable Terminal width with word-wrap;

- User profile (password, terminal parameters) kept on file;

- Real-time clock time stamping of files and messages;

- Private and public messages supported;

- Variable access levels available;

- File and message sub-sections for easier user access;

- All uploaded files stamped with name of contributor;

- Number of accesses of each file recorded;

- XMODEM protocol (CRC & checksum) fully supported;

- Calls logged with sign-on and sign-off times;

- Comments to System Operator (Sysop) file;

- Directly drives serial port hardware.

2 - System files:  The following files are included in the  Turbo

BBS package:

* Required to compile the program: BBS.PAS

IO.INC

CLOCK.INC

MAILSYS.INC

FILESYS.INC

All the above files must be on the logged drive when compiling!

The BBS program should be compiled to a .COM file.

* Required on the default drive to run the compiled BBS program:

BBS.COM (obviously);

MESSAGES.BBS - the system message table:  contains records  for

each message giving user numbers of sender and receiver, subject,

time stamp,  message and section number, and whether the receiver

has  read  the message or not.  This table is loaded into  a  RAM

buffer  whenever the system is active (ie.  caller on-line),  and

restored to disk at sign-off.

FILES.BBS - the table of files available on-line. Each file has

a record noting name,  size,  contributor, access count, section,

and whether the file is private or public.

LOG.BBS - Call Log, holding Caller, time on and time off info.

COMMENTS.BBS - Contains comments to Sysop, includes caller name

and time stamp.

IDS.BBS  - Contains registered user  names,  passwords,  access

levels,  terminal  parameters,  last call date and  high  message

number.

Turbo BBS Version 1.00 - System Operator Notes - Page 2

The  following text files containing on-line user information are

also expected on the default drive:

WELCOME.TXT  - The pre-sign-on user welcome file.  Should  give

details about the system's features, hours, and policies, as well

as  any news about new system features.  Newest bulletins  should

occur  first in a file so frequent users do not have to read long

stretches of familiar information to get the new stuff. This file

is also listed by the W (Welcome) command.

BBSHELP.TXT  - The  on-line Help file,  containing  the  system

commands and user hints. Also should list the subcommands for the

commands that support them.  This file is listed by the H  (Help)

command.

BBSLIST.TXT - Contains a list of other remote access systems of

interest. Listed by the O (Other systems) command.

SYSINFO.TXT - Gives information about the equipment running the

BBS,  and  general  information about the BBS.  Listed by  the  Y

(sYstem information) command.

MESS0001.TXT  - The message table contains a permanent  message

number 1, which contains specific help for the message system.

All messages are stored in individual text files,  using the

file  name MESSxxxx.TXT,  where xxxx is the message  number.  All

.TXT files are expected on the default drive. They can be edited

using WordStar,  in the Document mode.  The BBS program is set up

to ignore the WordStar "soft" carriage returns and line feeds, so

the  word  wrap output driver of the BBS can format the  file  to

suit  the user's terminal.

Messages stored by the BBS editor can be in two formats: one

contains  "hard"  carriage returns so special formatting used  in

message entry is preserved (P subcommand in E command). The other

contains  NO carriage returns,  which are inserted by the  output

routine as needed. Editing a message created by the latter option

with WordStar will show the entire message as a single  line:  ^B

will  fix  this by inserting soft carriage returns which the  BBS

will ignore.

The  top of the include file MAILSYS.INC defines a  constant

called  "maxmess,"  which defines the maximum number of  messages

that  can  be held at one time on the  system.  The  distribution

version sets this at 52,  which corresponds to the maximum number

of  files available on a Kaypro 2 disk (64) minus the  number  of

support  files on the default drive.  This limit can be increased

if a larger directory is available, but remember that the message

table is buffered in RAM and so each message uses 43 bytes.

Turbo BBS Version 1.00 - System Operator Notes - Page 3

3 - Installing the BBS on your system

The  machine specific codes is contained in the include file

IO.INC.  All system serial I/O is routed through these  routines.

The  procedures  and  functions that will need  changing  (unless

you're running a Kaypro 4-84 or modified 2-84) are flagged by the

string  "(*  Machine Dependent *)" for easy  spotting.  There  is

another  routine,  called "clearmodem" which is marked "(*  Modem

Dependent *)".  The distribution version was written for a  Rixon

212A Intelligent Modem.

To properly install the BBS,  it is necessary to know how to

properly  program  and  control  the  serial  port.   Since  most

operating  system drivers to not allow access to reading the  CTS

(Clear To Send) line, this is necessary... It is unreasonable for

the BBS to be expected to capture and comprehend the "NO CARRIER"

or equivalent string from a smart modem.  In particular, you must

know how to read the CTS line,  control the RTS line,  detect the

input buffer full signal, and the output buffer empty line. Also,

remember  the  character  data send and  character  data  receive

functions!  To allow XMODEM transfers,  it is necessary to set up

for 8 bits, no parity, and one stop bit.

Since  the system expects to use the CTS line as  a  carrier

detect  line,  it will be necessary to ensure that the modem does

send  a  suitably readable signal for  this  purpose.  The  Rixon

initialization  string does this.  On Hayes Smartmodems,  this is

controlled by a switch that must be set so the CTS line  reflects

an  on-line  status.  The  DSR or DCD  lines,  if  available  and

satisfactory,  can  be  used:  just  read that line  in  the  cts

function  in  IO.INC.

The  modem should also disconnect when the system lowers the

DTR   (Data  Terminal  Ready  - RS-232  pin  20)  line  for   400

milliseconds.  If this feature is not available, another reliable

disconnect method must be found and implemented in the  procedure

hangup  in IO.INC.  The DTR line is also lowered when the BBS has

been   accessed  locally  to  inhibit  the  modem's   auto-answer

function.  The  procedures setlocal and clearlocal (IO.INC)  will

require editing if this feature is not supported by your modem.

If  your system has a real-time clock,  the routines in  the

include file CLOCK.INC must be adapted to use it. Once again, the

distribution  file uses the Kaypro 4-84 clock.  The  Kaypro  2-84

normally omits this, but the CPU board has the necessary hardware

to  implement it with the addition of a few parts.  If the system

lacks a real-time clock, the clock-related features can be turned

off  by setting the constant "clockin" to FALSE at the  beginning

of the BBS.PAS file.

The  special  interest  sections  feature  of  the  BBS   is

currently  limited to 9 sections,  numbered 1-9.  If sections are

not  wanted,  set  the  "sectsin" constant at  the  beginning  of

BBS.PAS to FALSE.  The Section names are defined at the beginning

of  MAILSYS.INC.  Future  updates of the Turbo BBS will  offer  a

Turbo BBS Version 1.00 - System Operator Notes - Page 4

larger  number  of  available  sections,   with  either  password

protection   or  individual  user  access  flags  for   protected

sections. It is possible to not use sections in the BBS at first,

then  implement  them later,  since all files and messages  in  a

system  without  sections  are  placed  in  section  1  (Default:

General).

If  your  system  lacks sufficient memory  to  run  the  BBS

program as distributed, it will be necessary to overlay the files

system with the message entry editor. This is done by putting the

word  "overlay"  immediately before the procedure declaration  of

the  enter routine (near the end of MAILSYS.INC) and the  filesys

routine (the file FILESYS.INC).

That is, change          procedure enter;

to          overlay procedure enter;

and change          procedure filesys;

to          overlay procedure filesys;

For  this to work properly,  it is absolutely necessary to  avoid

putting  any  new procedure/function declarations at the  end  of

MAILSYS.INC,   the  beginning  of  FILESYS.INC,  or  between  {$I

MAILSYS.INC} and {$I FILESYS.INC} in BBS.PAS!

(Note that using overlays will create 2 more files on the default

drive: adjust the maximum number of messages as required.)

The  number  of files in the file system is limited  by  the

fact  that  the  directory  table is  buffered  in  RAM.  In  the

distribution version,  a limit of 40 is set due to the fact  that

the  Kaypro  2's disks have a mere 191K capacity (All  "freeware"

contributions go to the Hard Disk fund). To expand the number, it

is  necessary to change the filetab array declaration and  change

the  number in the procedure newfile.  Future versions  will  set

this  limit  with  a  compilation  constant.  Each  file  in  the

directory  uses  39 bytes of memory.  Future versions will  allow

large  directory  files  to be read directly from  disk  (at  the

expense  of operation speed).  (Please remember that the BBS  was

NOT originally written for distribution!) The BBS puts all  files

on the "B:" drive. To change this, you must change the following:

procedure addfile;

procedure newfile;  <-- "B:" occurs twice!

function legaltab.

Turbo BBS Version 1.00 - System Operator Notes - Page 5

4 - Running the system

Once  BBS.COM  has been compiled successfully,  it  must  be

placed  on the default drives with the .TXT and .BBS files listed

previously in this document.  The first thing the program  should

do  when  run is initialize the modem.  If the modem  echoes  the

initialization  string,  the display will show the echo mixed  in

with the outgoing string. This is disconcerting, but harmless.

The  file "IDS.BBS" contains two pre-declared  names,  which

are SYSOP and BOB MAXWELL (I couldn't resist).  The usernames and

passwords  are  always  mapped to capitals by the  BBS  to  avoid

errors.  SYSOP's  password is set to TURBO.  To sign on  locally,

press  the  'ESC' key on the host keyboard when the "Waiting  for

call..."  string appears.  You can then sign on and test out  the

system.  The  first  thing  to  do is  change  your  password  to

something VERY secure by using the P command: the system supports

certain  features for Sysops that are best not made available  to

the general public.

The BBS main program supports two commands not listed in the

menus. They are "!" and "@". "!" is the printer switch: if a user

with level 5 (Sysop) access uses this command,  system output  is

also sent to the lst:  device,  until another "!" is entered. "@"

is  the  comments file reader and ID access  editor.  First,  the

system comments file is displayed.  Next,  a "Kill (Y/N)?" prompt

appears,  allowing  the sysop to delete  the  file.  Lastly,  the

system will ask for a user name:  if it is in the userlist,  that

user's  current access level will be displayed,  and you will  be

prompted  for  a new user level (pressing only RETURN  leaves  it

unchanged).  The system will loop through the Name/Access prompts

until a non-existent user name is entered,  whereupon the command

ends.

Several commands contain Sysop extensions:  the L (call Log)

command  gives  a "Kill (Y/N)?" prompt to level 5  users.  The  U

(User  list)  command shows the access level of  each  user.  The

R  (message  Read) function prompts "Delete  (Y/N)?"  after  each

message read. Furthermore, all messages are visible to the Sysop.

The  file  system  also contains extensions for  the  Sysop.

These are the I,  R and K commands.  I is used to Install a  file

that  is on the files disk but not in the BBS directory,  so  the

Sysop  can  "upload"  files by just copying them onto  the  Files

disk. R releases a file: the public flag in the directory becomes

"true".  All uploaded/installed files are initially private,  and

do  not  appear on the BBS directory to anyone  without  level  5

access.  Level  3 users have the ability to access private  files

provided  they  know the file name.  Any files on the files  disk

that are not in the BBS directory are totally  unaccessable,  but

cannot  be  overwritten.  The K command deletes a file  from  the

directory table, and erases the file.

Turbo BBS Version 1.00 - System Operator Notes - Page 6

5 - General Information

This  software and its associated documentation and  support

files are released to the public domain by the author.  It is not

to  be  distributed  commercially or re-sold for  profit  without

prior  permission from the author.  If you feel the  software  is

well-suited  to your purposes,  and would like to be informed  of

any updates or new versions, please send $30 to:

Robert H. Maxwell,

201-2275 West 7th Avenue,

Vancouver, B.C.  CANADA

V6K 1Y3

This contribution entitles you to support,  error correction, and

updates.  Customization  of the code for specific purposes is not

included,  but can be made available at extra cost.  It would  be

appreciated if the specifics of the system you plan to run (phone

number,  hours,  baud  rates,  etc.) were also included,  so  any

improvements  YOU  make can be observed and possibly included  in

updates.  Also,  a  compilation of systems using this BBS will be

maintained on the Vancouver Turbo BBS from this information.

The  original Turbo BBS is on-line in Vancouver 24  hours  a

day  at  300 or 1200 baud at (604) 738-7811.  If the BBS  is  not

available,  another  computer  is usually on-line to  report  the

reason for the unavailability and when the BBS will  return.

For  fastest  response  to  questions  or  problems,  it  is

recommended  you  use  the  Vancouver  BBS.  Slower  response  is

available  through CompuServe's Borland SIG,  where the  author's

PPN is 70206,174. The use of voice calls is not appreciated until

the details have been arranged.

##################################################################