NETCHANGE(5)

NAME

netchange - format of netchange files read by irsim.

DESCRIPTION

A netchange file consists of a series of lines, each of

which begins with a key letter.  The key letter beginning a

line determines how the remainder of the line is inter-

preted.  The following are the list of key letters under-

stood.

| any text

Lines beginning with a vertical bar are treated as com-

ments and ignored by the program.

add type gate source drain length width [area]

Add a new transistor of type to the network.

Currently, type may be:

n   n-channel enhancement transistor.

p   p-channel enhancement transistor.

d   depletion transistor (for NMOS).

The names of the nodes to which the gate, source, and

drain of the transistor are connected are given by

gate, source, and drain respectively.  The length and

width of the transistor are given by length and width

respectively.  The area parameter, if given, will use

that number as the area for calculating the gate capa-

citance.  Length and width should be given in lambda

units, area should be in lambda^2 units, these will

internally be multiplied by the LAMBDA factor from the

configuration (.prm) file.

delete type gate source drain length width [area]

Delete an existing transistor from the net.  All the

parameters have the same meaning as for the add com-

mand.

move type gate source drain length width [area] g s d

Move an existing transistor to a new location the net.

type, gate, source, drain, length, width, have the same

meaning as for the add command.  g, s, and d are the

names of nodes to which the gate, source and drain

should be connected.  If a particular terminal(s) is

not to be re-connected, the name can be specified using

an "*".  Any or all of g, s, and d may be "*".  For

example, to move the gate of an n-channel transistor

from node old to new the following command would be

used:

m n old src_node drn_node 4 2.2 new * *

Note that the drain and source terminals, and the g and

s terminals are interchangeable; the simulator will

know if these are swapped.  So the last example could

also have been written:

m n old drn_node src_node 4 2.2 new * *

capacitance node value

Change the capacitance of a node by value picofarads.

Value may be negative, thereby decreasing the node's

capacitance.  Node is the node name.

N node metal-area poly-area diff-area diff-perimeter

Change the capacitance of node using the area and per-

imeter information of the metal, polysilicon, and dif-

fusion layers.  All the parameters should be in lambda

(or lambda^2 for areas) units, they will internally be

converted to the appropriate capacitance as defined in

the configuration file.  The values can be negative to

decrease the capacitance.

M node M2A M2P MA MP PA PP DA DP PDA PDP

Change the capacitance of node, using the following

geometrical information:

M2A   area of 2nd-level metal

M2P   perimeter of 2nd-level metal

MA    area of 1st-level metal

MP    perimeter of 1st-level metal

PA    area of polysilicon

PP    perimeter of polysilicon

DA    area of n-diffusion

DP    perimeter of n-diffusion

PDA   area of p-diffusion

PDP   perimeter of p-diffusion

All perimeter values should be in lambda units, area

values should be in lambda^2 units.  The perimeter

measures are half of the actual total perimeter (i.e.,

they are the sum of the lengths of the top and one

side).  Again, the values may be negative to decrease

the capacitance of the node.

threshold node low high

Change the threhsold voltages of node. Low and high

should be in normalized voltage units (i.e. floating-

point numbers in the range 0.0 to 1.0).

Delay node tplh tphl

Change the delays for node to be tplh nanoseconds for

low-to-high transistions, and tphl ns. for high-to-low

transistions.  These should be absolute numbers, not

relative increments/dercrements.

NOTE: For all commands, only the first letter is signifi-

cant, the rest of the string will be ignored.  They

are only shown here for clarity.

BUGS

This is an experimental interface for the incremental simu-

lator and is very likely to change in the future.

SEE ALSO

irsim(5) sim(5) presim(1)