irsim-analyzer(3)

NAME

irsim-analyzer

SYNOPSIS

analyzer node...

DESCRIPTION

The analyzer provides a graphical interface to irsim;

displaying the state of the nodes (or vectors) node...

Every invocation of the analyzer command adds a new set of

signals to the display list.

The view is split into 3 regions: a trace window, a text

window, and a banner.  The banner displays the name of the

simulation (from the sim file), and the pull-down menu

headers.  The text window is a 1-line window used to request

and display certain information.

The trace window displays the following information:

left side:   names of the signals (nodes/vectors) displayed.

top side:    On the left and right sides, the time

corresponding to the left and right edges of

the trace window.  In the center, the time of

the current cursor position.

right side:  Values of the signals under the cursor.

MENU FUNCTIONS

zoom:

in   Zooms in by a factor of 2 (magnify).

out  Zooms out by a factor of 2 (reduce).

base:  Changes the numerical base used to display the value

of the selected vector (see below how to select a

trace) to one of the following:

bin  (base 2)

oct  (base 8)

hex  (base 16)

window:1

delta T     Allows to determine the difference (in

time) between any two edges.   First click

the mouse (any button) to the right of the

first edge, then to the left of the second

edge.  The time of the two edges as well as

their difference will be shown in the text

window.  As long as the button remains

depressed, a line is shown between the two

edges.  Times are all shown in ns.

move to     Moves the left edge of the trace window to

the specified time.  The time is requested

in the text window.

set width   Sets the number of ns. that will be

displayed.  The time is requested in the

text window.

name length Since the most important differentiating

information in a long signal name is

usually at the end, the analyzer will

display the last 15 characters of a signal

name.  This option allows changing the

maximum number of characters that will be

displayed, which must be in the range [8-

256].

scroll      Usually the analyzer displays simulation

activity by scrolling the traces to the

left, so the last changes are shown. The

scroll feature enables/disables this

scrolling, effectively freezing the traces.

When scrolling is enabled, a check mark is

shown on this menu entry.

print:  These options control the generation of a PostScript

file suitable for obtaining a hardcopy of the trace

window.

file    Requests the filename to hold the PostScript

output.  Hitting <return> will use the

default name (shown in parens).  Hitting

<ctrl-C> will abort generation of the file.

The following menu entries control the

aspect of the PostScript file; a check mark

in the corresponding menu entry implies that

the feature will be included in the output.

banner  Generate a banner, showing the simulation

name and current date.

legend  Generate (in a separate page) a description

of the signals displayed: the complete names

of nodes (since the first characters may be

stripped), and for vectors the names of the

nodes of wich it is composed.

times   Generate a time ruler at the bottom of the

traces.

outline Draw an outline around the trace window.

MOUSE FUNCTIONS

The analyzer provides various functions depending on where

in the window a mouse button is depressed.  All buttons have

the same meaning, except inside the scrollbar.  Pressing a

mouse button in the following regions will provide:

Signal names:

The signal name where the button was depressed will be

highlighted and will be moved to the position where the

button is released, scrolling the other traces if

necessary.  If the button is released under the same

signal on which it was pressed, that signal will become

selected. The selected signal is always underlined, and

some information regarding that signal is printed in

the text window.

If the button is released outside the traces area

(above or below) the signal will be removed from the

display.  It can only be added again by invoking the

analyzer command.

Left Arrow:

The traces are scrolled left by half a page (move back

in time).

Right Arrow:

The traces are scrolled right by half a page (move

forward in time).

Double Left Arrow:

Scrolls traces left by a full page.

Double Right Arrow:

Scrolls traces right by a full page.

Scrollbar:

The left button allows to stretch the left side of the

window, thereby zooming in/out by an arbitrary amount.

The right side of the window remains the same.

The right button allows to stretch the right side of

the window, thereby zooming in/out by an arbitrary

amount.  The left side of the window remains the same.

The middle button moves the whole view back and forth

in time.  The magnification factor remains the same.

Traces

The cursor is moved to the time-step where the mouse is

clicked.  Some lack of accuracy in selecting the time

may become apparent due to screen resolution round off.

Holding down the shift key simultaneously with the

mouse button will not move the cursor, but rather show

the value of the signal at the time-step where the

mouse is clicked.  The value is printed in the text

window using the following format:

name @  time:value=value,input=status

The value is always shown in binary, regardless of the

base selected for that signal.  The status is a string

showing which nodes where inputs at that time.  Nodes

that were inputs have an i in their respective

position, otherwise an -.

Cursor Values:

The selected signal value is highlighted and, when the

button is released, the value of the signal is expanded

in the text window.  This is useful for displaying each

of the bits that make up a vector.  The information

printed has the following format:

name:value=value input=status

where value and status have the same meaning as above.

Banner:

Clicking on the banner brings the window to the

foreground.

Box on Banner:

Clicking in the little box on the left side of the

banner iconizes the analyzer window.  To de-iconize the

window, simply click any button within the icon.

Banner Menus:

Pressing a mouse button in one of the banner menus (on

the right side) will pull down the corresponding menu.

X DEFAULTS

The analyzer application uses the appropriate resource

specification at startup time to customize the appearance of

its window.  The format for a resource specification in the

.Xdefaults file is:

[name.]resource:  value

For the analyzer, the available names are irsim and

analyzer. The available resources are:

background     Specifies the traces window background color.

The default is black.

foreground     Specifies the text color, or the background

of everything else besides the traces window.

The default is white.

geometry       Specifies the default geometry (window size

and screen location) of the graphic window.

The default Xgeometry is "=1000x300+0+0".

reverseVideo   Specifies whether the foreground and

background colors are to be reversed (on

monochrome displays only).  The default is

off.

font           Specifies the fixed-width font displayed.

The default is 6x13.

borderWidth    Specifies the width of the border (in

pixels).  The default is 2.

borderColor    Specifies the border color when the window is

selected.  The default is black.

highlight      Specifies the color used for highlighting.

The default is red (color displays only).

traceColor     Specifies the color in which the traces are

drawn.  The default is white.

bannerBg       Specifies the background color for the

banner.  The default is white.

bannerFg       Specifies the foreground (text) color for the

banner.  The default is black.

SEE ALSO

irsim(1)