SH-Ware Compact Disk Control Program v1.00

This program is released as freeware. You may use and redistribute

it freely for no charge. The author can not be held responsible for

any or some data loss or hardware damage caused directly or otherwise

by the use or misuse of the program. You will be using the program a

your very own risk.

Operation

Format:                    shcd <Command> [UNIT] [DEVICE=device]

Template:                  COMMAND/A,UNIT/N,DEVICE/K

Purpose:                   To turn a drive motor on or off, or to open

or close a removable media.

Specification:

This is a simple command, that will send out an SCSI command to the

specified SCSI unit. You can switch the motor of a hard disk drive

on or off, or open or close the CD compartment of a CD drive. The

first argument specifies, what to send to the unit. There are four

commands supported, two for HDs and two for CDs:

Command  Device type       Operation

Start    Hard disk         Starts the drive motor

Stop     Hard disk         Stops the drive motor

Open     Compact disk      Ejects the CD

Close    Compact disk      Injects the CD

Table 1. Listing of commands supported, device types each applies to,

and what they do.

The SCSI device defaults to "scsi.device" and by default the unit to

bother is number 6. This is the target ID of the SCSI device on the

first board. Change this to address another device. If you have a

third party SCSI controller, that uses a device other than that

described earlier, use the DEVICE keyword to specify your device.

Examples:

shcd OPEN

Will eject the compact disk from the CD drive connected at target ID 6.

shcd CLOSE 4

Will inject a compact disk in the CD drive at target ID 4. These both

assume that you are using scsi.device.

shcd STOP 1 DEVICE suprascsi.device

Will stop the drive motor of the HD drive at target ID 1. This one

assumes a Supra board.

Note! The device name must be lowercase.

Note! You may pass junk as "<Command>". The operation performed defaults

to starting the drive motor, so this is what you get, if your input

does not match any of the valid commands.

Note! This program will only work if you have an SCSI CD drive.

This distribution in whole is copyright 1996 by Sami Klemola.

All rights reserved.