Kingston Technology Company

INSTALLATION GUIDE

Artisoft LANtastic Ver. 7.0 Driver

for EtheRx LC Ethernet Adapter

Copyright (C) 1996 Kingston Technology Company.

All rights reserved.

Introduction:

This installation guide provides the following information:

-  Installing the Artisoft LANtastic 7.0 driver.

The configuration of Kingston EtheRx LC ISA Ethernet adapter is shown

in the Custom install window of Qstart. You can manually change the

current configuration by selecting the Manual Configuration option. The

default hardware settings for the EtheRx LC ISA Ethernet adapter are:

IRQ (INT) = 3

I/O address (PORT) = 300

Note: You may need this information in order to install the driver.

Note:

For Windows 95 users: please refer to Windows 95 Driver

installation option in QSTART to load the adapter driver

first. Only load Lantastic 7.0 after Windows 95 has

recognized your adapter. To install Lantastic 7.0, run

a:setup from Windows 95's Start button.

Driver Installation Procedure:

1. Make sure the EtheRx LC adapter is properly installed in

your computer.

2. Place the LANtastic Disk 1 diskette in the A: or B: floppy

drive.

3. Run the LANtastic INSTALL program (for DOS/Windows 3.1x).

A:SETUP <Enter> or B:SETUP <Enter>

4. Proceed with the installation procedure, follow all the

prompts for each screen. Choose 'OK' or press <Enter> to

move from screen to screen.

5. In the Network Adapter Setup window, check 'NEX016

NOVELL NE2000 16-bit ISA BUS or COMPATIBLES'.

6. Make sure you carefully follow the prompts for each screen.

7. When the installation is complete, SETUP will automatically

reboot your computer.

8. The installation procedure is now complete.

If more than one EtheRx LC ISA Ethernet adapters are found in your PC,

you need to make sure two adapters use different IRQ and IOBASE

settings. After changing the adapter settings within QStart,

you need to change those parameters in file STARTNET.BAT

(located in the default directory: \LANTASTI) to match the adapter

settings.

sample STARTNET.BAT file

:

:

@echo=====Begin LANtastic configuration=====

PATH C:\LANTASTI;%PATH%

SET LAN_DIR=C:\LANTASTI.NET

LOADHIGH NEX000 16-BIT IRQ=3 IOBASE=300

LOADHIGH AILANBIO @STARTNET.CFG

:

:

By changing the values of IRQ and IOBASE settings specified by Qstart,

reboot the machine for the new settings to be updated.

Kingston Technology, EtheRx, and QStart are registered trademarks

of Kingston Technology Company.  All others are trademarks or

registered trademarks of their respective holders.