Kingston Technology Company

INSTALLATION GUIDE

IBM OS/2 LAN Server Ver. 3.0 NDIS 2 Driver

for EtheRx LC ISA Ethernet Adapter

Copyright (C) 1995-1996 Kingston Technology Company.

All rights reserved.

This installation guide provides the following instructions:

-  Using QStart to transfer IBM OS/2 LAN Server 3.0 NDIS driver

files from the Kingston Driver Diskette to the destination

directory.

-  Installing the IBM OS/2 LAN Server software.

Copying the Driver files:

When you use the Generic or Custom install options in QStart, the

files associated with the network operating system environment you

choose will be displayed on the screen and you can copy them to the

default destination directory (A:\NDIS2); unless you specify

otherwise.

The configuration of the Kingston EtheRx LC ISA Ethernet adapter is

shown in the Custom install window of Qstart. You can manually change

the current configuration using the Manual Configuration option. The

default hardware configuration settings for the EtheRx LC ISA Ethernet

adapter are:

IRQ (INT) = 3

I/O address (PORT) = 300

Note: You will need this information to install the driver.

After the files are copied to the destination directory, you will

need to perform the procedure outlined in the next section to install

the driver.

Driver Installation Procedure:

This installation procedure assumes that IBM OS/2 is up and running.

1. Make sure the EtheRx LC ISA adapter is properly installed in

your computer.

2. Open the 'OS/2 System' icon on the OS/2 desktop.

3. Click the 'Command Prompts' icon.

4. Click on the 'OS/2 Window' icon or the 'OS/2 Full Screen' icon.

5. Insert the 'IBM Network Transport Service LAN Adapter and

Protocol' diskette into your floppy drive. Type 'A:\LAPS' or

'B:\LAPS'.

6. Click 'Install' when the IBM logo appears.

7. Type in the target drive in the "Target Drive" window and

click 'OK'.

8. When the "Installation Complete" window appears, click 'OK'.

9. Click 'Install' again when IBM logo appears.

10. Insert the Kingston Driver Diskette into the floppy drive.

11. Enter the default destination directory, A:\NDIS2; if you

changed the default directory name when you ran QStart,

enter the new directory name and click OK.

12. Click 'OK' when the "Installation Complete" screen appears.

13. Select 'Configure' when the IBM logo appears.

14. In the "Configuration" window, select 'Configure LAN

transports' and click 'OK'.

15. In the "Configure Workstation" window, select 'Kingston EtheRx

LC ISA Ethernet Adapter' from the Network Adapters list and

click 'Add' to put the Kingston EtheRx LC ISA adapter into the

Current Configuration list.

16. Highlight "Kingston EtheRx LC ISA Ethernet Adapter" in the

Configuration list and click 'Edit'. Change the Interrupt

level and I/O Base address if necessary and click 'OK'. The

default settings for the Kingston Ethernet adapter are:

I/O base address = 300

IRQ = 3

17. Select a protocol from the Protocols list and select 'Add'.

18. Select 'OK' in the "Configure Workstation" screen.

19. Select 'Exit'.

20. Select 'Continue' in the "Update CONFIG.SYS" window.

21. Select 'OK' in the "CONFIG.SYS Update" window.

22. Select 'Exit' in the "Exiting LAPS" window.

23. Shutdown OS/2 and reboot your workstation.

24. When OS/2 desktop comes up, select "OS/2 Full Screen" or "OS/2

windows".

25. Insert 'Server Install Diskette 1' in floppy drive and type

'A:\LANINST'.

26. Follow the instructions on the screen to install OS/2 server.

27. Shut down OS/2 and reboot your workstation and the installation

is complete.

Kingston Technology, EtheRx, and QStart are registered trademarks

of Kingston Technology Company.  All others are trademarks or

registered trademarks of their respective holders.