Kingston Technology Company

INSTALLATION GUIDE

Wollongong Pathway NDIS 2 Driver

for EtheRx LC ISA Ethernet Adapter

Copyright (C) 1996 Kingston Technology Company.

All rights reserved.

Introduction:

This installation guide provides instructions for installing the

Wollongong Pathway NDIS driver.

This installation guide provides the following instructions:

-  Using QStart to transfer Wollongong Pathway NDIS driver

files from the Kingston Driver Diskette to the destination

directory.

-  Installing the Wollongong Pathway software.

Copying the Driver files:

When you use the Generic or Custom install options in QStart, the

files associated with the network operating system environment you

choose will be displayed in the Drivers Installation screen and you can

copy them to the default destination directory (A:\NDIS2);

unless you specify otherwise.

After the files are copied to the destination directory, you will

need to perform the procedure outlined in the next section to install

the driver.

Driver Installation Procedures:

I. Installing Wollongong Pathway NDIS 2 driver

This Installation assumes that your EtheRx LC Ethernet Adapter is

dedicated for Wollongong Pathway. For installation of Pathway over

existing network setup, in which case your LC adapter will be shared,

please refer to your Pathway manual.

1. Make sure the EtheRx LC ISA adapter is properly installed in

your computer.

2. Pathway Runtime should be loaded first to configure the LC adapter.

For Runtime v2.0 for DOS/Windows, execute A:\PWSETUP from

your Runtime diskette (this version requires the Interrupt setting for

your LC adapter).

For Runtime v4.1.1 for Windows, execute A:\SETUP -from Windows-

off your Runtime setup diskette.

3. At the Adapter Selection:

*For Runtime v2.0: select Ethernet adapter, NDIS driver

*For Runtime v4.1.1: select "Manufacturer Supplied Disk

[NDIS drivers]"

For adapter type, select "ETHERNET/ARCNet"

When asked to input the path for PROTOCOL.INI, insert your

Kingston driver diskette, enter A:\NDIS2 as the path.

4. Follow instructions on the screen to complete your installation.

5. Reboot your PC.

6. Next step is loading your Pathway Access.

7. Please refer to your manual instructions.

For your reference only:

*Sample CONFIG.SYS:

...DEVICE=C:\PATHWAY\PROTMAN.DOS /I:C:\PATHWAY

DEVICE=C:\PATHWAY\KTC2000.DOS...

*Sample AUTOEXEC.BAT:

...PATH=C:\PATHWAY;%PATH%

C:\PATHWAY\NDIS -D:1

C:\PATHWAY\NETBIND...

*Sample PROTOCOL.INI:

...[KTC2000$_NIF]

DRIVERNAME=KTC2000$

[PROTMAN]

DRIVERNAME=PROTMAN$

DYNAMIC=YES...

If more than one EtheRx LC ISA Ethernet adapters are found in your PC,

you must also add the following line into this section of the

PROTOCOL.INI file as  following:

(Note: Location of PROTOCOL.INI

*For Runtime v2.0 for DOS\Windows, the default location is in:

\PATHWAY

*For Runtime v4.1.1 for Windows, the default location is in:

\PATHWAY\LANMAN)

:

[KTC2000]

AdapterID = @00F0C0xxxxxx

:

where "xxxxxx" is the last 6 alphanumeric characters of the

corresponding  MAC address of the EtheRx LC ISA adapter you are

using. Reboot the machine for the new settings to be updated.

Kingston Technology, EtheRx, and QStart are registered trademarks

of Kingston Technology Company.  All others are trademarks or

registered trademarks of their respective holders.