Kingston Technology Company

INSTALLATION GUIDE

Windows for Workgroups Ver.3.11 NDIS 3.0 Driver

for EtheRx LC ISA Ethernet Adapter

Copyright (C) 1995-1996 Kingston Technology Company.

All rights reserved.

This installation guide provides the following instructions:

-  Using QStart to transfer Windows for Workgroups Ver.3.11

NDIS driver files from the Kingston Driver Diskette to the

destination directory.

-  Installing the Windows for Workgroups Ver.3.11 NDIS driver.

Copying the Driver files:

When you use the Generic or Custom install options in QStart, the

files associated with network operating system environment you choose

will be displayed on the screen and copied to the default destination

directory (A:\NDIS3); unless you specify otherwise.

The configuration of Kingston EtheRx LC ISA Ethernet adapter is shown

in the Custom install window. You can manually change the current

configuration if desired. The default hardware settings for the EtheRx

adapter are:

IRQ (INT) = 3

I/O address (PORT) = 300

Note: You may need this information in order to install the driver.

After the files are copied to the destination directory, you will

need to perform the procedure outlined in the next section to install

the driver.

Driver Installation Procedure:

This installation procedure assumes Windows for Workgroups 3.11 is

properly installed on your computer and that no network driver is

currently installed.

1. Make sure the EtheRx LC ISA adapter is properly installed in

your computer.

2. Once the adapter is installed, turn your computer on and start

Windows for Workgroups 3.11.

3. Choose the 'Network Setup' icon from the 'Network' group.

4. From the "Network Setup" dialog box, choose 'Drivers'.

5. From the "Network Driver" dialog box, choose 'Add Adapter'.

6. Select 'Unlisted or Updated Network Adapter' from the Adapter

list.

7. The program will prompt you to insert the driver diskette and

enter the path of the driver directory. Insert the Kingston

Driver diskette into your floppy drive. Enter the default

destination directory, A:\NDIS2; if you changed the default

directory name when you ran QStart, enter the new directory

name and click OK.

8. The 'Kingston EtheRx LC ISA Ethernet Adapter' will appear in

the list of adapters; select it, then choose OK.

9. Specify the hardware configuration for your Kingston EtheRx LC

ISA Ethernet Adapter.

10. The 'Kingston EtheRx LC ISA Ethernet Adapter [NDIS3/NDIS2]'

message appears, it indicates the NDIS driver has been copied

into your Windows directory.

11. In the "Network Driver" dialog box, choose 'Close'.

12. In the "Network Setup" dialog box, choose 'OK'.

13. Windows prompts you to insert several WFW system diskettes.

14. When message 'Windows Setup has modified your SYSTEM.INI and

PROTOCOL.INI' appears, choose 'OK'.

15. When the "Restart" dialog box appears, choose 'Restart

Computer'.

16. The installation procedure is now complete.

Kingston Technology, EtheRx, and QStart are registered trademarks

of Kingston Technology Company.  All others are trademarks or

registered trademarks of their respective holders.