Kingston Technology Company

INSTALLATION GUIDE

Windows for Workgroups Ver.3.11 ODI Driver

for EtheRx LC ISA Ethernet Adapter

Copyright (C) 1996 Kingston Technology Company.

All rights reserved.

This installation guide provides the following instructions:

-  Using QStart to transfer Windows for Workgroups Ver.3.11

ODI driver files from the Kingston Drivers Diskette to the

destination directory.

-  Installing the Windows for Workgroups Ver.3.11 ODI driver.

Procedure:

The OEMSETUP.INF file provided for WFW 3.11 NDIS 3 driver in this

diskette allows user to install one of the drivers supported in

WFW 3.11, NDIS3, NDIS2 or ODI driver.  In order to access both

Novell NetWare server and Microsoft Network, it is easier to configure

the WFW 3.11 to use the ODI driver.  Novell provides some DOS/Windows

Client software which can access Microsoft network through ODI driver.

These DOS/Windows Client software need to be installed before

configuring the WFW 3.11 to use ODI driver.

I.  Run QSTART to extract the WFW311 NDIS3 Driver and DOSODI/VLM

drivers to directory  C:\NWCLIENT .

1. Make sure the EtheRx PCI adapter is properly installed in

your computer.

2. Insert the Kingston Drivers diskette 1 into your floppy

drive A:.

3. Type A:QSTART[Enter] to run QStart.

4. Select 'Generic Install', after the adapter test completed

select [Continue].

5. Select "Microsoft Windows for Workgroups ver. 3.11 / ODI Client"

in Driver screen.

6. Change the default destination directory if desired then click

'Copy'.

7. After QStart copied the files to the destination directory,

click 'Exit' to get back to DOS.

II. Install Novell NetWare DOS ODI driver:

We recommend using NetWare DOS/Windows Client Software (It is a package

of 5 diskettes).It contains all required drivers for both DOS and

Windows. Otherwise you need to download the Windows drivers packet

from Novell (WINUP9.EXE).

Note: make sure WFW 3.11 is configured for "No Windows support for

Network"  at this point.

- Run Install.exe from DOS/Windows Client Software (diskette 1).

- Select 'Select the driver for your network board' Option.

- Select 'Other Drivers' from the list of available drivers.

- When asked for the driver diskette, enter 'C:\NWCLIENT'.

- All drivers for DOS and Windows will be copied into c:\nwclient and

c:\windows.

- Reboot after finish.

III.  Configure WFW 3.11 Network:

- Run Network Setup under Network Group.  Click on  'Network' and

select Support for Microsoft Network.  Mark Additional Network

Support and select Novell Network (Workstation Shell 4.0 and above).

Click 'Close'.

- Click on Driver and select 'Add Adapter'.  Select 'Unlisted or

Updated Network Adapters'.  When asked for the diskette, enter the

path (C:\NWCLIENT)  to KTC2000.386 and OEMSETUP.INF.

Follow instructions to finish the installation.

NOTE: When ask to overwrite your Netware drivers, select NO.

- Reboot the system.

- Upon entering Windows, you should now be able to see both

Novell  NetWare Server and all Microsoft workstations.

Kingston Technology, EtheRx, and QStart are registered

trademarks of Kingston Technology Company.  All others are

trademarks or registered trademarks of their respective holders.