Kingston Technology Corporation

Drivers Diskette

for

PC Card Ethernet Adapter

Version 2.4    January 4, 1996

Copyright (c) 1995 Kingston Technology Corporation.

All rights reserved.

INTRODUCTION

This file (README.DOC) describes the driver files on this disk. It

provides general information that became available after the User

Guide was printed.

Your Kingston Ethernet PC Card is shipped with DOS ODI driver, Packet

driver and DOS NDIS 2.01 driver on this diskette. The DOS ODI driver

and associated files are in the \DOSODI directory. The Packet Driver

is located under \PKTDRV directory. You can find the DOS  NDIS 2.01

driver and associated files located in the \MSLANMAN.DOS and \NDIS

directories.  There is an associated file called 'INSTALL.DOC' for

each driver.  The INSTALL.DOC is a text file that explains how to

use and install the driver in a particular Network Operating System

environment.  In some cases, the INSTALL.DOC also includes a

"Troubleshooting Guide" section that describes how to solve common

installation problems.

The INSTALL.DOC files provide you with valuable information.  Please

refer to them if you encounter any installation problem.

DISKETTE VERSION HISTORY

Version 2.4 (Current Version) January 4, 1996

- Updated Document for WIN95.

Version 2.3 December 27, 1995

- Add a new Packet Driver.

- Add a new Novell Server Driver.

- Add a new Windows for Workgroup 3.11 NDIS 3 Driver.

- Support Banyan Vines, SUN PC-NFS and FTP PC/TCP NDIS 2 Driver.

- Support Windows 95.

- Update PCMDM.COM (DOSODI driver) for TCPIP compatibility.

Version 2.2 March 31, 1995

Update Card and Socket Services software to version 3.12.

Version 2.0 February 24, 1995

SYSTEM RESOURCES REQUIRED

The Ethernet PC Card uses the following system resources:

o  32 bytes of I/O space on a 32-byte boundary;

o  one Interrupt Request (IRQ) line;

o  16 KB of system memory address space (the adapter card

contains 16 KB of memory, which is accessed by mapping it

into the system's memory address space, using a technique

known as "shared memory");

o  additional 4 KB of system memory address space for Card

Services

DISKETTE CONTENTS

\

|  README.DOC              This file you are reading.

|  KTCPCM.ID               Disk ID file.

|

+- DOSODI

|  |  NET.CFG              Configuration file for DOSODI driver.

|  |  PCMDM.COM            DOSODI driver.

|  |  PCMDM.INS            Installation support file for ODI driver.

|  |  INSTALL.DOC          Installation guide for DOSODI driver.

|  |

|  +- NETX

|  |     IPXODI.COM        Novell NetWare IPXODI program.

|  |     LSL.COM           Novell NetWare LSL program.

|  |     NET.CFG           Configuration file for PCMDM.COM.

|  |     NETX.EXE          Novell NetWare Shell file.

|  |     PCMDM.COM         DOSODI driver.

|  |

|  \- VLM

|        *.VLM             Novell Netware VLM files.

|        IPXODI.COM        Novell NetWare IPXODI program.

|        LSL.COM           Novell NetWare LSL program.

|        NET.CFG           Configuration file for ODI/VLM.

|        PCNTNW.COM        DOSODI driver.

|        PCNTNW.INS        Installation support file ODI driver.

|        STARTNET .BAT     Batch file for start netware.

|        VLM.EXE           Novell Virtual Loadable Module Manager.

|

+- NOVELL

|  +- 3.1X

|  |     MSM31X.NLM        Novell MSM needed to run the ODI driver

|  |                       with NetWare v3.11.

|  |     ETHERTSM.NLM      Novell TSM needed to run the ODI server

|  |                       driver.

|  |     PATCHMAN.NLM      Novell module needed in NetWare v3.11.

|  |     MONITOR.NLM       NetWare server MONITOR module for

|  |                       NetWare v3.11.

|  |     LSLENH.NLM        Novell module needed in NetWare v3.11.

|  |     KTCPCM.LAN        Novell NetWare ODI server driver.

|  |     INSTALL.DOC       NetWare v3.1x driver installation guide.

|  |

|  \- 4.X

|        MSM.NLM           Novell MSM needed to run the ODI server

|                          driver.

|        ETHERTSM.NLM      Novell TSM needed to run the ODI server

|                          driver.

|        KTCPCM.LAN        Novell NetWare ODI server driver.

|        KTCPCM.LDI        Novell network installation file.

|        INSTALL.DOC       NetWare v4.x driver installation guide.

|

+- MSLANMAN.DOS

|  |  INSTALL.DOC          LAN Manager v2.x Driver installation

|  |                       guide.

|  +- DRIVERS

|     +- ETHERNET

|     |  \- PCMDM

|     |     PCMDM.DOS      NDIS workstation driver for DOS.

|     |     PROTOCOL.INI   Protocol file for PCMDM.DOS driver.

|     |

|     \- NIF

|           PCMDM.NIF      NIF file for PCMDM.DOS driver.

|

+- NDIS2

|  +- LANTASTI.60

|  |      INSTALL.DOC      Installation guide for Lantastic 6.0.

|  |      PCMDM.DOS        NDIS 2 Driver.

|  |      PROTOCOL.INI     PROTOCOL.INI file for Lantastic 6.0.

|  |

|  +- WFW.311

|  |      PCMDM.DOS        NDIS 2 Driver.

|  |      OEMSETUP.INF     OEMSETUP.INF file for WFW 3.11.

|  |      INSTALL.DOC      Installation guide for WFW 3.11.

|  |

|  +- BANYAN

|  |      INSTALL.DOC      Installation guide for Banyan VINES.

|  |      PCMDM.DOS        NDIS 2 Driver.

|  |      PROTOCOL.INI     PROTOCOL.INI file for Banyan VINES.

|  |

|  +- PCTCP

|  |      INSTALL.DOC      Installation guide for FTC PC/TCP.

|  |

|  \- PCNFS

|         INSTALL.DOC      Installation guide for SUN PC-NFS.

|

+- NDIS3

|  \- WFW.311

|         KTCPCM.386       NDIS 3 Driver.

|         OEMSETUP.INF     OEMSETUP.INF file for WFW 3.11.

|         INSTALL.DOC      Installation guide for WFW 3.11.

|

+- PKTDRV

|     KTCPCMP.COM          Packet Driver.

|     INSTALL.DOC          Installation guide for Packet driver.

|

\- WIN95

NETSNIP.INF

INSTALL.DOC          Installation guide for Windows 95.

COMPATIBILITY LISTS

This section includes two types of lists:

o  KNOWN Compatibility - products that have been tested and

verified for compatibility

o  OTHER PC Compatibility - products that HAVE NOT been tested,

but which should be compatible based on their architecture

KNOWN Compatibility

1.  Using Card and Socket Services Provided by Kingston

The following hardware has been tested and verified with the Kingston

Ethernet PC Card using the Card and Socket services supplied on

the PC Card Ethernet Adapter Diskette.

Computer:                Appropriate software:

----------------------   ------------------------------------------

AST PowerExec 3/25SL     Kingston Card/Socket Services (SS365SL.EXE);

and 4/25SL            can also use POINTETH.SYS Enabler

Dell 320SLi and 325SLi   Kingston Card/Socket Services (SS365SL.EXE);

can also use POINTETH.SYS Enabler

DEC PC325SL              Kingston Card/Socket Services (SS365SL.EXE);

can also use POINTETH.SYS Enabler

Grid 2260 Convertible    Kingston Card/Socket Services (SS365SL.EXE);

can also use POINTETH.SYS Enabler

NEC UltraLite Versa      Kingston Card/Socket Services (SS365SL.EXE);

can also use POINTETH.SYS Enabler

NCR Safari SSL/25        Kingston Card/Socket Services (SS365SL.EXE);

(model 3170)           can also use POINTETH.SYS Enabler

Toshiba T4500            Kingston Card/Socket Services (SS365SL.EXE);

can also use POINTETH.SYS Enabler

Zenith Z*Lite            Kingston Card/Socket Services (SS365SL.EXE);

can also use POINTETH.SYS Enabler

2.  Using the System Vendor's Card and Socket Services

The following hardware has been tested and verified with the Kingston

Ethernet PC Card using the Card and Socket services provided by the

system's vendor. These systems DO NOT work with the Card and Socket

services on the Ethernet PC Card Diskette, so you SHOULD NOT have the

INSTALL program remove the existing Card Services and Socket Services

command lines from your CONFIG.SYS file.

Computer:                Appropriate software:

----------------------   ------------------------------------------

IBM PS/2 E               IBM's Card/Socket Services (shipped with PC)

(Energy Workstation)

IBM ThinkPad             IBM's Card/Socket Services (shipped with PC)

(Models 720 and 720C)

3.  Using Enabler Software Provided by Kingston

The following hardware has been tested and verified with the Kingston

Ethernet PC Card using one of the ENABLER programs supplied on the

Ethernet PC Card Diskette. These systems DO NOT work with the Card

and Socket services on the Ethernet PC Card Diskette, and therefore

require Enabler Installation.

Computer:                Appropriate software:

----------------------   ------------------------------------------

Sharp PC-6700            SHARPETH.SYS direct Enabler only

Toshiba T3300SL          TOSHETH.SYS direct Enabler only

PC Compatibility Information

Most computers use Intel's 82365SL ("PCIC") or DataBook's PCMCIA

controller and thus SHOULD work with the Kingston Ethernet PC Card

using CS.EXE Card Services and SS365SL.EXE (for Intel) or

SSDBOOK.EXE (for DataBook) Socket Services provided by Kingston.

RESTRICTIONS

Sharp PC-6700

The Sharp PC-6700 works only in the shared memory mode of the network

drivers. For the ODI driver's NET.CFG file, a MEM line is required.

For the NDIS driver's PROTOCOL.INI file, a RAMADDRESS line is

required.

The Sharp PC-6700 has Socket Services in its flash ROM BIOS memory.

This software can interfere with the operation of the network

drivers. To prevent this with the ODI driver, delete (or insert a

semicolon [";"] at the beginning of) the line in the ODI driver's

NET.CFG file that contains the keyword PCMCIA. To prevent this with

the NDIS driver, modify the NDIS driver's PROTOCOL.INI file to delete

(or insert a semicolon [";"] at the beginning of) the line in the

PROTOCOL.INI file that contains the keyword PCMCIA.

Grid 2260 Convertible

The Grid 2260 Convertible works only in the shared memory mode of the

network drivers. For the ODI driver's NET.CFG file, a MEM line is

required. For the NDIS driver's PROTOCOL.INI file, a RAMADDRESS line

is required.

IBM ThinkPad 350

When using the DOS NDIS driver on the IBM ThinkPad 350, you may

experience system hangs if I/O base address 0x300 is used.  Since

this is the default I/O base address, you should edit the

PROTOCOL.INI file for the DOS NDIS driver and change the value of the

IOBASE keyword to any other available I/O base address, such as:

IOBASE = 0x320

and then save the modified PROTOCOL.INI file.  If you are using the

POINTETH.SYS enabler, you also need to edit the CONFIG.SYS file so

that the enabler uses the new I/O address, such as:

DEVICE=C:\KNEPCM\POINTETH.SYS IO=320

NEC UltraLite VERSA

The available IRQ (Interrupt) resources on the NEC UltraLite VERSA

change unexpectedly.  IRQs 3 and 5 are not always available, even

though the CSALLOC.EXE program may detect that they are available and

write these IRQs in the CSALLOC.INI file.  For best results on this

notebook, select IRQ 10 or 11 for use by editing the NET.CFG file if

you are using the DOS ODI driver or by editing the PROTOCOL.INI file

if you are using the DOS NDIS driver.  In the NET.CFG file, change

the value of the INT keyword, such as:

INT A

for interrupt 10 or

INT B

for interrupt 11, and then save the modified NET.CFG file.

In the PROTOCOL.INI file, change the value of the INTERRUPT keyword,

such as:

INTERRUPT = 10

for interrupt 10 or

INTERRUPT = 11

for interrupt 11, and then save the modified PROTOCOL.INI file.

If you are using the POINTETH.SYS enabler, you also need to edit the

CONFIG.SYS file so that the enabler uses the new interrupt setting,

such as:

DEVICE=C:\KNEPCM\POINTETH.SYS IRQ=A

for interrupt 10 or

DEVICE=C:\KNEPCM\POINTETH.SYS IRQ=B

for interrupt 11.

TROUBLESHOOTING

PC Card Driver "Beep" Errors

There are a few cases in which the Ethernet PC Card drivers cannot

display error messages, so they sound the alarm (beeper) on your

computer to indicate that something is wrong.

For the NDIS driver, beeps occur at initialization time only.

The driver beeps if Card Services is not present or the driver's

PROTOCOL.INI parameters do not match the Enabler's parameters

(when an Ethernet PCMCIA Card is present).  If no card has been

inserted, then the NETADDRESS and INTERRUPT must be specified

in the PROTOCOL.INI file; otherwise the driver will beep.

For the ODI driver, beeps occur during driver initialization or

during card insertion handling.  The following conditions cause

the DOS ODI driver to signal the error beep:

o  Card Services error while attempting to map a memory window

to the buffer memory on the Ethernet PCMCIA Card

o  Card Services error while attempting to allocate or release

I/O space for the card

o  Card Services error while attempting to allocate an interrupt

for the card

o  Card Services error while attempting to allocate or release

a memory window to the Flash memory on the card

o  Card Services error while attempting to map a memory window

to the Flash memory on the card

o  Card Services error while attempting to modify the attributes

of a memory window on the card

o  Card Services error while attempting to request configuration

of the card

o  ST-NIC Ethernet controller error

o  Driver is unable to reset the ST-NIC Ethernet controller

o  Buffer memory read/write test error

See the "Network Driver" section in the User Guide for more details.

TECHNICAL SUPPORT INFORMATION

Accessing the Kingston BBS for the Latest Information

The Kingston BBS can be accessed by using a modem and modem

communications program to call the BBS at 714-435-2636.

Communications settings are no parity, 8 data bits, and 1 stop bit

(xxxxx-N-8-1, where xxxxx is your modem's maximum speed).

When you have logged on to the BBS, check under the "Public" section

for updates or other new Ethernet PC Card information.

Technical Support: 6:00 am to 6:00 pm Pacific Time

Monday through Friday

(800) 435-0640 or (714) 435-2639

BBS: Kingston's Bulletin Board Service

(714) 435-2636, up to 14.4Kbps, 8-N-1

CompuServe: 71333,3714 or type: GO KINGSTON

Internet: For Product Information: sales@kingston.com

For Technical Support: tech_support@kingston.com

RAMfax: Automatic Faxback System (U.S. and Canada)

(800) 435-0056 or (714) 435-2677

Kingston Technology Corporation and EtheRx are registered trademarks

of Kingston Technology Corporation.  All others are trademarks or

registered trademarks of their respective holders.