STANDARD MICROSYSTEMS CORPORATION

IBM LAN Server v 2.0 and v3.0 OS/2 Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the IBM LAN server driver from the

SuperDisk to your PC.

o  How to install the IBM LAN server driver.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default Destination Directory is a:\NDIS.  Please make

a backup copy of your SuperDisk before EZStart copies files to the

SuperDisk to create an "option" diskette.

INSTALLING THE DRIVER

1. Insert the SMC or IBM LAN Adapter Protocol Support (LAPS)

Version 1.00 diskette into your computer's disk drive.

2. Type: LAPS

3. Press the Enter key.

The IBM logo appears.

4. Select the INSTALL button.

5. Specify the target drive (usually C:\) and press Enter or

select OK.

6. Select the INSTALL button a second time to load the SMC

drivers and press ENTER.

7. Place the SuperDisk in drive a:.

8. Change "Install Additional Network Driver" panel to read

"a:\NDIS", then select OK.

9. Press OK for each driver.

10. Select the Configure button.

11. Select "Configure LAN transports" followed by Continue.

The Configure Workstation screen appears.

12. From the Network Adapters window, select "SMC EtherEZ for

OS/2" followed by the Add button.

This adapter name appears in the Current Configuration window.

13. If you want to enable "RIPL" features, select "IBM IEEE 802.2"

from the Protocols window followed by Add.

14. Select "IBM OS/2 NETBIOS" followed by Add. (You may also need to

select the "IBM NetWare Requester" option and Add, depending on

your network.)

Each time you select the Add button, the selected option appears

in the Current Configuration window, below the chosen driver.

15. Select OK.

16. Select Exit.

The CONFIG.SYS Update screen appears.

17. Specify the drive where the CONFIG.SYS file resides.

18. Select the Continue... button.

19. Select OK.

20. Select Exit.

21. Reboot your computer. Use the OS/2 Shutdown process to exit from

the program in an orderly fashion.

This completes the LAPS installation procedure.

Refer to IBM's server installation procedure guides to install a

standard OS/2 server.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.