STANDARD MICROSYSTEMS CORPORATION

Artisoft's LANtastic Network Software v6.0 Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the driver files for Artisoft's

LANtastic Network Software version 6.0 from the SuperDisk to

your PC.

o  How to install the driver files for Artisoft's LANtastic

Network Software version 6.0

o  Sample file contents after LANtastic Network Software 6.0 is

installed.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files' to

copy all of the files used by Artisoft's LANtastic Network Software

version 6.0 driver from the SuperDisk to the Destination Directory

recommended by EZStart, or to a different directory that you specify.

The EZStart default Destination Directory is A:\

Make a backup copy of your SuperDisk before EZStart copies files to

the SuperDisk.

INSTALLING THE DRIVER FILES

The following section describes how to install Artisoft's LANtastic

Network Software version 6.0 along with the SMC driver file. If you

already have Artisoft's LANtastic Network Software version 6.0

installed and only want to install the SMC driver, proceed to

"Installing the SMC Driver Only".

Installing Artisoft's LANtastic Network Software version 6.0 and

the SMC Driver

The following procedure describes how to install Artisoft's LANtastic

Network Software version 6.0 along with the driver files. This

procedure is designed to expedite the driver installation.

1. Edit the PROTOCOL.INI file you copied from EZStart so it

resembles the following:

[SMC8000_NIF]

Drivername = SMC8000$

2. Insert Artisoft's LANtastic Network Software version 6.0

Disk 1 of 4 in your computer's disk drive.

3. Type:  a:\install

4. Press the Enter key to start the setup procedure.

5. Follow the on-screen prompts.

6. When the Select Network Adapter Window prompts you for a LAN

driver, select "NDIS Support for Network Adapters" followed

by "OK".

7. When you are prompted for the manufacturer's driver diskette,

insert the SuperDisk in your computer's disk drive. Then

specify the path where the modified PROTOCOL.INI resides on the

SuperDisk.

Note: If you used EZStart to copy the driver to a different

Destination Directory, specify that directory's path in step

7. If that directory resides on a diskette other than the

SuperDisk, insert that diskette in your computer's disk

drive.

8. Select "OK".

9. Proceed with the rest of the LANtastic Network Software

version 6.0 driver installation procedure.

10. At the end of the installation procedure, reboot your computer.

Installing the SMC Driver Only

The following procedure describes how to install the SMC driver

and assumes that Artisoft's LANtastic Network Software version 6.0

is already installed.

If you are using Artisoft's LANtastic Network Software version 6.0

and want to install the SMC driver:

1.  Double-click on the Control Panel icon in the Main window.

2.  Double-click on the Network icon.

3.  Select the Adapters icon in the Options area.

4.  Select the adapter you want to remove followed by "Remove".

5.  When a message asks whether you are sure you want to remove

the adapter, select "Yes".

6.  Select "Add". The Install Network Adapter dialog box appears.

7.  Select the SMC adapter you want to install followed by "OK".

The Install Driver dialog box appears.

8.  Type the path where the driver resides (for example, A:\)

and select "OK". The Unlisted or Updated Network Adapter

dialog box appears.

9.  Select "OK" when the next two dialog boxes appear.

10. Select "Close". The Network Settings dialog box appears.

11. Select "OK". You will be prompted whether you want to accept

all the options you selected and restart your computer or

return to DOS. Restart your computer so your options can take

effect.

LANtastic NETWORK SOFTWARE v6.0 DRIVER FILES

Included on the SuperDisk are the sample files, CONFIG.SYS, PROTOCOL.INI

AUTOEXEC,BAT, and STARTNET.BAT generated during the LANtastic Network

Software version 6.0 installation.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.