SMC

Novell NetWare DOS ODI VLM Client Driver

For EtherPower, EtherPower^2, and the EtherPower 10/100 Family

of PCI Network Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1997 SMC Networks, Inc.. All rights reserved.

This installation guide describes:

o  Supported adapters

o  Preinstallation hardware and software requirements

o  Supplied files

o  Adapter installation notes

o  Using Lanalyzer for Windows

o  Dual Channel support (EtherPower^2 Only)

o  Driver installation

o  Loading the driver

o  Using STARTNET.BAT

o  Modifying the line speed (EtherPower 10/100 and

EtherPower 10/100 B only)

o  Error messages

SUPPORTED ADAPTERS

o  EtherPower Ethernet PCI adapters (8432 board types)

o  EtherPower^2 Dual-Channel PCI Ethernet adapters (8434 board types)

o  EtherPower 10/100 Fast Ethernet PCI adapters (9332 board types)

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

o  An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus.

o  An EtherPower, EtherPower^2, or EtherPower 10/100

PCI adapter for that bus.

Firmware Requirements:

o  Fully compliant Rev. 2.0 or above PCI BIOS supplied with the PC.

Software Revision Requirements:

o  MS-DOS v3.0 or above.

o  Novell LSL (v2.11 or higher, provided on this SuperDisk).

o  Novell IPXODI (available on this SuperDisk).

o  Novell VLM (available on this SuperDisk).

or

o  Novell NETX (available from Novell).

SUPPLIED FILES

Files provided in the Novell NetWare DOS ODI Client Directory include:

o  INSTALL.DOC     This file.

o  RELEASE.DOC     Provides a description of updates to the driver.

o  SMCPWR.COM      SMCPWR Novell NetWare ODI Client driver.

o  SMCPWR.INS      SMCPWR Novell NetWare ODI Client installation file.

o  NET.CFG         Novell network configuration file for the driver.

o  LSL.COM         Needed for Novell NetWare ODI DOS protocol stack.

o  IPXODI.COM        Needed for Novell NetWare ODI DOS protocol stack.

o  STARTNET.BAT    Batch file that automatically executes Client software.

o  SMCVLM.EXE     Self-extracting VLM Client software for basic

connectivity.

NOTE: You can also find the NetWare DOS ODI shell drivers on the

Novell-supplied DOS ODI workstation diskette.

ADAPTER INSTALLATION NOTES

This driver conforms to the ODI Developer's LAN Driver Toolkit

Guide for DOS Workstation HSMs, version B, 18 June 1992.

For custom installations involving multiple frame types, refer to

your Novell NetWare ODI Shell for DOS manual.

When installing an SMC PCI adapter in a PCI computer, make sure the

slot you select supports bus master operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using the

appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the appropriate

motherboard jumper.

3. Enter your computer's setup menu and:

- Enable bus master on the PCI slot.

- Enable I/O on the PCI slot.

- Define interrupt number on the PCI slot.

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

EtherPower 10/100 Users:

Ensure that the line speed (10 Mbps vs. 100 Mbps) configured

for the adapter matches the speed of the hub to which it is

attached. The EtherPower 10/100 adapter is default configured to

operate at 100 Mbps.

Configuration information is stored in the NET.CFG file.

It is important to set the adapter at the correct line speed

(10/100 Mbps) using the Line_Speed parameter. Refer to the

MODIFYING THE LINE SPEED section later in this document.

EtherPower 10/100 B Users:

The Auto-Negotiation feature of the EtherPower 10/100 B adapter will set the

operating speed of the adapter (10 Mbps [MB10] or 100 Mbps [MB100]). If the

attached hub is full duplex capable and uses Auto-Negotiation, the

EtherPower 10/100 B adapter will configure itself for full duplex operation.

If the attached hub is full duplex capable, but does not use Auto-

Negotiation, you must manually specify full duplex mode. Refer to the

MODIFYING THE LINE SPEED section later in this document.

USING LANALYZER FOR WINDOWS

The SMC DOS ODI VLM client driver (SMCPWR.COM) conforms to version 4.0

of the DOS ODI specification. To use any 4.0 DOS ODI driver with

Lanalyzer for Windows, a file provided from Novell, RXMONSTK.COM,

must be loaded before loading the driver:

Example

LSL

RXMONSTK

SMCPWR

Another file provided from Novell, RXMONSTK.MSG is also required

to use Lanalyzer for Windows and must reside in the same directory as

RXMONSTK.COM.

Use EZStart to copy the DOS ODI client driver files from the

SuperDisk to a Destination Directory on the target client. The

RXMONSTK.COM and RXMONSTK.MSG files, available from Novell,

must be copied to the same directory.

DUAL CHANNEL SUPPORT (EtherPower^2 Only)

The EtherPower^2 Dual Channel PCI Ethernet adapter conforms to the

PCI-to-PCI Bridge Architecture Specification, Revision 1.0, April 5,

1994. Networking environments that support dual channel operation

include Novell server (32-bit ODI driver), Windows NT, and SCO. Other

networking environmnets do not support the unique dual channel capability

of the EtherPower^2 adapter. These environments include Novell client

(DOS ODI driver), Windows for Workgroups v3.11, and networking

environments that use DOS or OS/2 NDIS2 drivers.

Refer to the EtherPower^2 User Guide for complete installation instructions.

DRIVER INSTALLATION

There are three different methods you can use to install the

DOS ODI driver and a VLM client:

One Step Installation With EZdosodi:

In one simple step EZdosodi will attach your computer to the nearest

NetWare server. It will automatically detect your network's frame type

and configure your client files accordingly.  It will also modify your

CONFIG.SYS file to add the LASTDRIVE=Z statement and the

AUTOEXEC.BAT file to call STARTNET.BAT which calls the NetWare

client files that attach your workstation to the network server. If you

do not wish to be automatically connected to a network server at this

time, use one of the two procedures described below. Otherwise, refer to

the 'Quick and EZ Guide' that accompanied your adapter for information

about how to use EZdosodi (Exit to a DOS prompt, change to the Super-

Disk target drive, type EZDOSODI, and press Enter).

Automatic Installation With EZStart:

Use EZStart's 'Automatic Novell DOS ODI' feature when you require

greater customization than offered by EZdosodi (for example, if you

wish to specify a server name or a specific Ethernet frame type). You

will have the choice of modifying your CONFIG.SYS and AUTOEXEC.BAT

files. If you choose this option, the workstation will automatically

attach to the server upon reboot.

Custom Installation With EZStart:

For a basic NetWare Client installtion which will copy the client

files and the SMC driver to your hard drive, use EZStart's Custom

Setup procedure. Refer to Loading the Driver (following) for

information about how to use EZStart.

LOADING THE DRIVER

1. Switch to the drive containing the SuperDisk, type EZStart, and press

Enter. You will be presented with the main screen.

2. From the main screen, select Custom.

3. From the Custom Installation screen, select Drivers/Documents.

4. From the Driver Installation screen, select:

- Automatic Novell DOS ODI (customization capability)

or

- Novell (will not modify AUTOEXEC.BAT or attach to a server

upon reboot. Greater capability to customize.)

5. Select 'Copy Files' to copy all the files used by this driver from the

SuperDisk to the destination directory recommended by EZStart, or to a

different directory that you specify.

6. Edit NET.CFG file and check that the line speed (EtherPower 10/100 only),

cable type, and network protocol type settings are correct for your

network. Note that there is no need to specify PCI slot number or

Interrupt number.

If you do specify the slot number; EISA slots are counted from

1 to 15, PCI slots are counted from 16. The first PCI adapter

installed in your computer will be treated as if it is installed

in slot 16, the next one will be in slot 17, and so on. Refer to the

README.DOC file located in the root of the SuperDisk for details

('Specifying the PCI Logical Slot').

7. Reboot your workstation. The DOS ODI driver will automatically

load each time you reboot if you have modified your AUTOEXEC.BAT

file to call STARTNET.BAT.

If you installed the driver using EZStart and did not edit your

AUTOEXEC.BAT file to call STARTNET.BAT, see USING STARTNET.BAT below.

This completes the driver-loading procedure. Refer to the Novell NetWare

ODI Shell for DOS manual for further details about running a DOS VLM client.

USING STARTNET.BAT

If you do not use EZdosodi or EZStart's 'Automatic Novell DOS ODI'

installation procedure to automatically modify your AUTOEXEC.BAT file

to call STARTNET.BAT upon reboot, you can modify it yourself as shown below:

REM            Change directory to DOS ODI driver files.

CD \NWCLIENT

REM            Load network software.

STARTNET

REM            This batch file, STARTNET.BAT, loads the DOS

REM            ODI driver and Novell network shell program.

If you do not use the STARTNET.BAT file as described above, use the

following loading sequence to connect to a server:

LSL.COM

SMCPWR.COM

IPXODI.COM

VLM.EXE

MODIFYING THE LINE SPEED

EtherPower 10/100 ONLY:

The NET.CFG file contains various configuration parameters for use

with the adapter. The default line speed for the EtherPower 10/100

adapter is 100 Mbps. Make no changes to the Line_Speed parameter in

NET.CFG if you wish to attach to a 100 Mbps hub. To operate at 10 Mbps,

remove the comment (;) indicator from the line:

Line_Speed=10

Other parameters, such as the active serial interface, are also

specified using this file.

EtherPower 10/100 B ONLY:

The methods used to manually specify operating speed will vary

depending on your networking software and the driver being used.

All SMC drivers for the EtherPower 10/100 B adapter support five

different data rate settings.

AUTOSENSE -   the driver will automatically detect the

line speed of the attached hub and configure

the adapter to operate at the highest possible

data rate

10 Mbps

Full duplex - adapter operates at 10 Mbps only

10 Mbps

Half duplex - adapter operates at 10 Mbps only

100 Mbps

full duplex - adapter operates at 100 Mbps only

100 Mbps

half duplex - adapter operates at 100 Mbps only

The appropriate SuperDisk driver installation document (specific

to the network operating system) describes how to perform the

above tasks.

After completing the installation and configuration, you must reboot

your computer.

ERROR MESSAGES

FATAL: DC21X4 driver works only with 386 or above!

The driver failed to identify the CPU as a 386 or above. The

driver can run on 386 or above computers only.

FATAL: Slot value error!

Illegal slot number specified in NET.CFG file.

50 - FATAL: Board not found in system

Driver could not detect a supported adapter in the computer.

FATAL: Failed to lock system memory

Driver call to operating system memory support routines for

locking its internal buffers and descriptors failed.

200 - FATAL: Timeout when initializing DC21X4 with setup packet!

Occurs when the PCI slot does not support DMA. Check that BIOS

setup and motherboard switches are set to enable DMA and bus

master operation for the slot.

210 - FATAL: DC21X4 Does not support this bus type

Computer is not identified as an EISA- or PCI-bus system. You

may have specified a BUS ID that is not EISA or PCI.

217 - FATAL: Failed to find PCI BIOS

Computer is not identified as a PCI system. The error is

generated when the driver is directed to load a PCI adapter using

the BUS ID keyword and PCI 4 is specified, or SLOT keyword is

specified and slot value is a PCI slot number.

218 - FATAL: Bad PCI BIOS status

PCI BIOS returned a fail status in one of the calls. Ensure your

computer fully complies with Rev. 2.0 or above of the PCI BIOS

specification.

219 - FATAL: Bad PCI BIOS Register err

PCI BIOS returned a fail status in one of the calls. Ensure your

system fully complies with Rev. 2.0 or above of the PCI BIOS

specification.

220 - FATAL: PCI BIOS failed to initialize DecChip 21X40 CBIO reg!

PCI BIOS has initialized a DWORD register in the adapter's

configuration space (at offset 10 hex) with illegal value (in x86

systems, this value is between 0 and FFFF hex). Check the BIOS

using an external utility.

Also, verify the BIOS setup enables the PCI slot, allocates I/O

addresses to it, and initializes the adapter correctly.

225 - FATAL: PCI BIOS allocated an illegal Interrupt to DecChip 21X40!

PCI BIOS has initialized a byte register in the adapter's

configuration space (at offset 3C hex) with illegal value (in X86

systems, legal values are between 1 to 15). Check the BIOS using

an external utility.

Also, verify the BIOS setup enables the PCI slot, allocates I/O

addresses to it, and initializes the adapter correctly.

226 - WARNING: Failed to handle Turbo switch

PCI BIOS returned a fail status in one of the calls involved with

operating this mechanism. Chipset on motherboard does not support

Turbo mode.

227 - FATAL: Internal CRC table corrupted

Driver found an error in internal CRC table either because the

driver file is corrupted or the file contents have been changed.

228 - FATAL: Could not handle chipset bug workaround successfully.

Failed to operate chipset bug workaround because PCI BIOS

returned a fail status in one of the calls involved with

operating this mechanism.

229 - WARNING: This machine Host to PCI bridge has a known bug.

Driver limits DC21X4 bursts as a workaround.

This message indicates that your computer does not correctly

support the PCI Local Bus Specification (v2.0). The SMC driver

will work in this environment, but you are urged to contact the

manufacturer of your computer to determine if your BIOS correctly

supports the features of the PCI Local Bus Specification (v2.0).

230 - FATAL: This machine has a known bug when using cache in WB

mode which may cause data corruption or system hang. Change the

cache mode to WT (Write Through) as a workaround.

240 - FATAL: Unable to locate or read the Ethernet ROM address on

board!

Could be caused by corrupt data stored in the IEEE serial ROM.

250 - FATAL: Failed to read ROM!

Driver failed to read the IEEE serial flash ROM due to hardware

handshake failure. Make sure the IEEE flash ROM on your adapter

is operational using EZStart's Basic Adapter Test.

This machine host to PCI bridge has a known bug. Driver limits

DC21040 bursts as a workaround.

Indicates your computer does not correctly support the PCI Local

Bus Specification (v2.0). The SMC driver will function in this

environment, but you are urged to contact the manufacturer of

your computer to determine if your BIOS correctly supports the

features of the PCI Local Bus Specification (v2.0).

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is SMC Networks, Inc.,  350 Kennedy,

Hauppauge,  NY    11788.

SMC is a registered trademark; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are trademarks

of SMC Networks, Inc.. Other product and company names

are registered trademarks or trademarks of their respective holders.