STANDARD MICROSYSTEMS CORPORATION

IBM LAN Server v2.0 and v3.0 OS/2 Requester

For EtherPower 10/100 Fast Ethernet PCI Network Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide:

o  Provides an overview.

o  Describes supplied files.

o  Describes how to use EZStart to copy the IBM LAN server OS/2

requester from the SuperDisk to your PC.

o  Describes how to install the IBM LAN server OS/2 requester.

NOTE: Refer also to the NDIS.DOC file located in the NDIS2 directory

for complete information regarding DOS and OS/2 NDIS2 drivers.

NDIS.DOC describes:

o  An overview of the NDIS2 driver.

o  Preinstallation hardware and software requirements.

o  Adapter installation notes.

o  Basic driver installation (for any network operating system).

o  Installing multiple adapters.

o  How to define line speed, mode of operation, PCI slot

number and other parameters via PROTOCOL.INI.

NOTE: Ensure that the line speed (10 Mbps vs. 100 Mbps) configured

for the adapter matches the speed of the hub to which it is

attached.

Configuration information is stored in the PROTOCOL.INI file.

It is important to set the adapter at the correct line speed

(10/100 Mbps) using the PROTOCOL.INI parameter DATA_RATE.

For example, to enable operation at 10 Mbps, add the DATA_RATE

line to the '9332OS2_NIF' section of PROTOCOL.INI as shown

below:

[9332OS2_NIF]

Drivername=SMCPWR$

Data_Rate=MB10

Two other important PROTOCOL.INI parameters are SIA_MODE, which

defines the serial interface connection (including full-duplex

or half-duplex operation), and SLOT, which specifies the PCI

slot number location of the adapter.

Refer to the section in NDIS.DOC titled PROTOCOL.INI PARAMETERS

for further details.

OVERVIEW

This driver supports SMC EtherPower Ethernet and SMC EtherPower

10/100 Fast Ethernet PCI adapters.

This driver conforms to the Network Driver Interface Specification

(NDIS) v2.0.1 defined by Microsoft/3Com. The driver has been tested

with Microsoft LAN Manager, 3Com 3+Open, IBM LAN Server, IBM PC LAN

Program, DEC Pathworks, and other NDIS-compatible network operating

systems and protocol stacks.

SUPPLIED FILES

General files

o  RELEASE.DOC      Release notes for this version of the driver.

o  LANRQOS2.DOC     This file.

Files to install the driver

o  SMCPWR.OS2       OS/2 NDIS2 driver.

o  PROTOCOL.INI     Includes driver binding information and

operational parameters.

o  SMC9332O.NIF     Network installation file.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default Destination Directory is a:\NDIS.  Please make

a backup copy of the SuperDisk before EZStart copies files to the

SuperDisk to create an "option" diskette.

INSTALLING THE DRIVER

1. Insert the SMC or IBM LAN Adapter Protocol Support (LAPS)

Version 1.00 diskette into your computer's disk drive.

2. CD to IBMCOM\. Type LAPS

3. Press the Enter key.

The IBM logo appears.

4. Select the INSTALL button.

5. Specify the target drive (usually C:\) and press Enter or

select OK.  Files will be copied to target drive, then

select OK.

6. Select the INSTALL button a second time to load the SMC

drivers and press ENTER.

7. Place the SuperDisk in drive a:.

8. Change "Install Additional Network Driver" panel to read

"a:\NDIS", then select OK.

9. Press OK for each driver.

10. Select the Configure button.

11. Select "Configure LAN transports" option, then select Continue.

The Configure Workstation screen appears.

12. From the Network Adapters window, select "SMC EtherPower for OS/2"

followed by the Add button.

This adapter name appears in the Current Configuration window.

13. If you want to enable "RIPL" features, select "IBM IEEE 802.2"

from the Protocols window followed by Add.

14. Select "IBM OS/2 NETBIOS" followed by Add. (You may also need to

select the "IBM NetWare Requester" option and Add, depending on

your network.)

Each time you select the Add button, the selected option appears

in the Current Configuration window, below the chosen driver.

15. Select OK.

16. Select Exit.

The CONFIG.SYS Update screen appears.

17. Specify the drive where the CONFIG.SYS file resides.

18. Select the Continue button.

19. Select OK.

20. Select Exit.

21. Reboot your computer. Use the OS/2 Shutdown process to exit from

the program in an orderly fashion.

22. Reboot your computer. Enter OS/2, and insert the OS/2 Requester

Install diskette into your computer's disk drive (a:\).

23. From the OS/2 prompt, type: LANINST.

24. Press ENTER.

25. Select OK at the IBM logo, then select OK again.

26. Select the "Advanced" option.

The Installation Tasks panel appears.

27. Select "Install or Configure this workstation", then select OK.

28. Select "Install or remove a component", then select OK.

29. From the displayed list, highlight all other components you

want to install.  Use the arrow keys to choose each component

and press the space bar to highlight it.  Select the Install

button and the word "Installed" appears in the Status column.

After you select all the components you need for this

installation, select the OK button.

Note: Some components have the word "Install" next to them. It is

recommended that you select these components for installation

along with any other components you choose to install.

30. Select the "Configure a component" option and select OK.

31. Select a component to configure from the list and select the

Configure button (Default is "Requester").

32. Specify the requester name and the domain name for that

component and select OK.

33. If you want the server to start automatically when one of

its workstations is started, select YES.

34. Select the requester services you require (either

"Messenger" or "Message pop-up") and select OK.

The Configure screen reappears.

35. Select OK.  At "Apply the Changes" select OK and input the

discs as required.  At the file backup panel select OK.

36. Select the "Configuration complete" option.

37. Select the LAN adapter to which this configuration applies.

If there is only one adapter in this computer, select

"Adapter 0" followed by OK.

38. Select OK when the Configure screen appears.

39. Select the "Apply the changes" option followed by OK.

40. Follow the screen prompts to copy all files from the

Requester diskettes to your computer's hard disk.

This completes the installation procedure.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and

EtherPower 10/100, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation. Other product and company names are

registered trademarks or trademarks of their respective holders.