SMC

DOS NDIS2 Driver for DEC PATHWORKS v4.1 for DOS

For EtherPower, EtherPower^2, and the EtherPower 10/100 Family

of PCI Network Cards

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1997 SMC Networks, Inc.. All rights reserved.

This installation guide describes:

o  Overview

o  Supported network cards

o  Supplied files

o  Network card installation notes

o  Dual channel support

o  Using EZStart to copy the driver files from the SuperDisk

to your file server or client machine

o  Installing the DOS NDIS2 driver on PATHWORKS

for DOS v4.1

o  Modifying the data rate

OVERVIEW

This document explains how to install the DOS NDIS2 driver for

DEC PATHWORKS v4.1 for DOS for X86-based platforms.

SUPPORTED NETWORK CARDS

o  EtherPower Ethernet network cards

(SMC8432 board types)

o  EtherPower^2 Dual-Channel Ethernet network cards

(SMC8434 board types)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332DST board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332BDT board type)

o  EtherPower 10/100 Dual-Channel

Fast Ethernet 100BASE-TX network cards

(SMC9334BDT board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-T4 network cards

(SMC9332BVT board type)

SUPPLIED FILES

General files

o  RELEASE.DOC      Release notes for this version of the driver.

o  DEC41DOS.DOC     This file.

Files to install the driver on PathWorks

o  SMCPWR.DOS       DOS NDIS2 driver.

NETWORK CARD INSTALLATION NOTES

Refer also to the NDIS.DOC file located in the NDIS2 directory

for complete information regarding DOS and OS/2 NDIS2 drivers.

Important PROTOCOL.INI Parameters:

Configuration information is stored in the PROTOCOL.INI file. Two

important PROTOCOL.INI parameters are SIA_MODE, which defines the

serial interface connection (including full-duplex or half-duplex

operation), and DATA_RATE, which specifies the speed the network card

will operate at (10 Mbps [MB10] or 100 Mbps [MB100]).

SMC9334BDT EtherPower 10/100 network card ONLY:

During the network card selection process, the SMC9334BDT

EtherPower 10/100 dual channel network card may be represented as two

SMC9332BDT EtherPower 10/100 adapters (one for each channel). If no

option for 'SMC9334BDT EtherPower 10/100 Dual Channel' is presented,

select the option for 'SMC9332BDT EtherPower 10/100' for both channels.

SMC9332DST EtherPower 10/100 network card ONLY:

Ensure that the line speed (10 Mbps vs. 100 Mbps) configured for the

network card matches the speed of the hub to which it is attached.

The SMC9332DST EtherPower 10/100 network card is default configured

to operate at 100 Mbps.

SMC9332BDT/SMC9334BDT/SMC9332BVT EtherPower 10/100 network cards ONLY:

The Auto-Negotiation feature of these cards will set the operating

speed of the network card (10 Mbps [MB10] or 100 Mbps [MB100]). If the

attached hub is full duplex capable and uses Auto-Negotiation, the

card will configure itself for full duplex operation. If the attached

hub is full duplex capable, but does not use Auto-Negotiation, you must

manually specify full duplex mode. Refer to the MODIFYING THE DAT

RATE section later in this document.

DUAL CHANNEL SUPPORT

EtherPower^2 and SMC9334BDT EtherPower 10/100

The EtherPower^2 Dual Channel PCI Ethernet and the SMC9334

EtherPower 10/100 Fast Ethernet network cards conform to the

PCI-to-PCI Bridge Architecture Specification, Revision 1.0, April 5,

1994. Networking environments that support dual channel operation

include Novell server (32-bit ODI driver), Windows NT, and SCO.

Other networking environments do not support the unique dual channel

capability of the EtherPower^2 or the SMC9334 EtherPower 10/100

network card. These environments include Novell client (DOS ODI

driver), Windows for Workgroups v3.11, and networking environments

that use DOS or OS/2 NDIS2 drivers.

USING EZSTART TO COPY THE DRIVER FILES

You can run EZStart from the SuperDisk or, for faster operation, you

can copy it onto a hard disk and run it from there. To run EZStart

from the hard disk, all files must be copied from the SuperDisk to

your computer's hard disk.

EZStart may copy files back to the SuperDisk to create an installation

diskette. Please make a backup copy of your SuperDisk before using

EZStart.

1. Insert the SuperDisk into a floppy disk drive.

2. Change to the floppy drive where the SuperDisk is installed.

3. To run EZStart from the DOS command line, type:

[path]ezstart<Enter>

To run EZStart in text mode (instead of graphics mode), type

[path]ezstart/T<Enter>

To run EZStart from Microsoft Windows v3.x:

select the 'File' menu in 'Program Manager'. When the 'Run'

dialog box appears, type:

[path]ezstart

in the 'Command Line' box, then select the 'OK' button.

To run EZStart with Windows 95:

1. Select 'Shut Down' from the 'Start' menu

2. Select the option to 'Restart the computer in MS-DOS mode'

NOTE: Do not run EZStart from a DOS window!

To run EZStart with Windows NT or OS/2:

EZStart does not run under these operating systems. Instead,

boot your  computer using an MS-DOS operating system.

4. Select 'Custom' from the EZStart main screen.

5. Select 'Drivers/Documents' from the 'Custom Installation' screen.

6. There are two 'Network Drivers and Documentation Selection'

screens. Select the appropriate vendor. The screen appears again.

Select the appropriate driver (make sure the selection you want is

highlighted). Then press the 'Enter' key or select 'OK'.

7. At the 'Network Drivers and Documentation' screen, the default

destination directory and a list of files to be copied to the

default directory are displayed. To accept the default destination

directory, select 'Copy Files'. Otherwise, enter a destination

directory at the prompt and select 'Copy Files'.

8. Follow the prompts to continue the installation.

9. Select 'Exit' to exit EZStart.

10. Remove the SuperDisk from the floppy disk drive.

INSTALLING THE DOS NDIS2 DRIVER

On PATHWORKS for DOS v4.1

NOTE: The PathWorks scheduler (SCHK) must be started with the switch

/N. If /N is not used, initialization may fail with the following

message:

"No NDIS Interrupt Activity.  Maybe NI_IRQ set wrong or cable

not attached or hardware/DLL broken"

o Automatic Installation for DOS PathWorks Using the Netsetup Utility:

If you install PATHWORKS for the first time or update it using

the installation program NETSETUP, use the following procedure

to load the driver.

1.  Select your desired choices for

- Type of configuration

- Destination Drive (boot device)

- LK250 Keyboard setup information

2.  Select <Ethernet> network card type.

3.  Select <Other NDIS> Ethernet network card.

4.  For full DRIVE:\PATH \FILENAME for NDIS2 driver, enter

A:\SMCPWR.DOS

5.  For full DRIVE:\ PATH\ for the PROTOCOL.INI stub, enter

A:\

The files are copied from the installation diskette to

its directory.

NOTE: If PATHWORKS is updated using the NETSETUP installation

program it is advisable to rename or copy the CONFIG.SYS

and PROTOCOL.INI files before starting NETSETUP.

o Manual installation for DOS PathWorks

1. Copy the SMCPWR.DOS file from the driver installation diskette

to the PATHWORKS directory.  For example:

C:\DECNET> copy A:\SMCPWR.DOS

2. Add the following line to your CONFIG.SYS file to load the driver:

DEVICE=<drive>:\DECNET\SMCPWR.DOS

3. Modify the PROTOCOL.INI file using any DOS editor as follows:

Copy the PROTOCOL.INI stub from the driver installation

disk to the end of the old PROTOCOL.INI file.

[SMCPWR]

DRIVERNAME=SMCPWR$

4. Modify DATALINK and IPX4MAC modules in the PROTOCOL.INI file.

Change the value of the keyword BINDINGS from:

BINDINGS = driverlabel

to

BINDINGS = SMCPWR

MODIFYING THE DATA RATE

SMC9332DST EtherPower 10/100 ONLY:

Ensure that the data rate (10 Mbps or 100 Mbps) configured for

the network card matches the speed of the hub to which it is

attached. Set the network card to operate at the correct data

rate (10 Mbps or 100 Mbps) using the PROTOCOL.INI parameter

DATA_RATE.

To operate at 10 Mbps, remove the comment (;) indicator from the

line:

DATA_RATE=MB10

this makes 10 Mbps the active data rate selection. Other parameters,

such as the active serial interface, are also specified using this

file.

If the SIA_MODE is set to AUTOSENSE, the driver will detect and

set the correct data rate during power-up.

SMC9332BDT/SMC9334BDT/SMC9332BVT EtherPower 10/100 ONLY:

This network card supports NWAY (Auto-Negotiation). Setting the

SIA_MODE to AUTOSENSE will allow the driver to automatically

detect the line speed (10 Mbps [MB10] or 100 Mbps [MB100]) and

operate at the highest possible data rate.

The recommended setting for the SIA_MODE is AUTOSENSE. If you

manually set the SIA_MODE to anything other than AUTOSENSE,

(for example, either half-duplex or full-duplex operation), the

settings you select must be supported by the hub to which you

are attaching.

The methods used to manually specify operating speed will vary

depending on your networking software and the driver being used.

All SMC drivers for the SMC9332BDT/SMC9334BDT/SMC9332BVT

EtherPower 10/100 network card support five different data rate settings.

AUTOSENSE -  when using this default setting, the driver

will automatically detect the line speed of the attached hub

and set the network card to operate at the highest possible

data rate

10 Mbps

Full duplex - network card operates at 10 Mbps only

10 Mbps

Half duplex - network card operates at 10 Mbps only

100 Mbps

full duplex - network card operates at 100 Mbps only

100 Mbps

half duplex - network card operates at 100 Mbps only

After completing the installation and configuration, you must

reboot your computer.

Refer to the section in NDIS.DOC titled PROTOCOL.INI PARAMETERS

for further details.

Below are sample CONFIG.SYS, PROTOCOL.INI and AUTOEXEC.BAT files.

PROTOCOL.INI

[PROTOCOL MANAGER]

DriverName=PROTMAN$

[IPX4MAC]

DriverName=IPX$MAC

BINDINGS=SMC

[SMC]

DriverName=SMCPWR$

[DATALINK]

DriverName=DLL$MAC

LG_BUFFERS=16

SM_BUFFERS=6

OUTSTANDINGS=32

HEURISTICS=0

BINDINGS=SMCPWR

NI_IRQ=10

# 	NOTE: NI_IRQ MUST MATCH THE IRQ THE CARDS IS CONFIGURED FOR.

CONFIG.SYS

DEVICE=C:\DECNEC\LADDRV.SYS /D:8

DEVICE=C:\DECNEC\PROTMAN.DOS /I:C:\DECNEC

DEVICE=C:\DECNEC\SMCPWR.DOS

LASTDRIVE=Z

AUTOEXEC.BAT

NETBIND

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is SMC Networks, Inc.,  350 Kennedy,

Hauppauge,  NY    11788.

SMC is a registered trademark; and EtherPower, EtherPower^2,

EtherPower 10/100, EZStart, and SuperDisk

are trademarks of SMC Networks, Inc.. Other product and

company names are registered trademarks or trademarks of their

respective holders.