SMC

Sun PCNFS

For EtherPower, EtherPower^2, and the EtherPower 10/100 Family

of PCI Network Cards

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1997 SMC Networks, Inc.. All rights reserved.

This installation guide:

o  Supported network cards

o  Using EZStart to copy the driver files

o  Preparing your operating system

SUPPORTED NETWORK CARDS

o  EtherPower Ethernet network cards

(SMC8432 board types)

o  EtherPower^2 Dual-Channel Ethernet network cards

(SMC8434 board types)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332DST board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332BDT board type)

o  EtherPower 10/100 Dual-Channel

Fast Ethernet 100BASE-TX network cards

(SMC9334BDT board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-T4 network cards

(SMC9332BVT board type)

USING EZSTART TO COPY THE DRIVER FILES

You can run EZStart from the SuperDisk or, for faster operation, you

can copy it onto a hard disk and run it from there. To run EZStart

from the hard disk, all files must be copied from the SuperDisk to

your computer's hard disk.

EZStart may copy files back to the SuperDisk to create an installation

diskette. Please make a backup copy of your SuperDisk before using

EZStart.

1.  Insert the SuperDisk into a floppy disk drive.

2.  Change to the floppy drive where the SuperDisk is installed.

3.  To run EZStart from the DOS command line, type:

[path]ezstart<Enter>

To run EZStart in text mode (instead of graphics mode), type

[path]ezstart/T<Enter>

To run EZStart from Microsoft Windows v3.x:

select the 'File' menu in 'Program Manager'. When the 'Run'

dialog box appears, type:

[path]ezstart

in the 'Command Line' box, then select the 'OK' button.

To run EZStart with Windows 95:

1. Select 'Shut Down' from the 'Start' menu

2. Select the option to 'Restart the computer in MS-DOS mode'

NOTE: Do not run EZStart from a DOS window!

To run EZStart with Windows NT or OS/2:

EZStart does not run under these operating systems. Instead, boot

your computer using an MS-DOS operating system.

4. Select 'Custom' from the EZStart main screen.

5. Select 'Drivers/Documents' from the 'Custom Installation' screen.

6. There are two 'Network Drivers and Documentation Selection'

screens. Select the appropriate vendor from the first screen. The

screen appears again. Select the appropriate driver you wish to

install (make sure the selection you want is highlighted). Then

press the 'Enter' key or select 'OK'.

7. At the 'Network Drivers and Documentation' screen, the default

destination directory and a list of files to be copied to the

default directory are displayed. To accept the default destination

directory, select 'Copy Files'. Otherwise, enter a destination

directory at the prompt and select 'Copy Files'.

8. Follow the prompts to continue the installation.

9. Select 'Exit' to exit EZStart.

10. Remove the SuperDisk from the floppy disk drive.

PREPARING YOUR OPERATING SYSTEM

For driver setup and configuration instructions, refer to the

documentation provided with the Sun PCNFS network operating system.

Below are sample CONFIG.SYS, PROTOCOL.INI and AUTOEXEC.BAT files.

PROTOCOL.INI

[PROTOCOL MANAGER]

drivername=PROTMAN$

[SMC]

DRIVERNAME=SMCPWR$

[NFS-NDIS]

drivername=NFSLINK$

BINDINGS=SMC

CONFIG.SYS

DEVICE = C:\NFS\PCNFS.SYS

DEVICE = C:\NFS\SOCKDRV.SYS

DEVICE = C:\NFS\PROTMAN.SYS /I:C:\NFS

DEVICE = C:\NFS\SMCPWR.DOS

DEVICE = C:\NFS\NFS-NDIS.SYS

LASTDRIVE = V

AUTOEXEC.BAT

SET TZ = EST5EDT

SET PATH = C:\NFS;%PATH%

SET NFSDRIVE = C

PRT *

C:\NFS\NETBIND

# C:\NFS\NFSRUN	# FOR VERSION 3.5 OF PC-NFS.

C:\NFS\NETINIT	# FOR VERSION 4.0 AND ABOVE OF PC-NFS.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is SMC Networks, Inc.,  350 Kennedy,

Hauppauge,  NY    11788.

SMC is a registered trademark; and EtherPower, EtherPower^2,

EtherPower 10/100, EZStart, and SuperDisk

are trademarks of SMC Networks, Inc.. Other product and

company names are registered trademarks or trademarks of their

respective holders.