SMC

Microsoft Windows for Workgroups v3.1 and v3.11 DOS NDIS2 Driver

For EtherPower, EtherPower^2, and the EtherPower 10/100 Family

of PCI Network Cards

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1997 SMC Networks, Inc.. All rights reserved.

This installation guide:

o  Overview

o  Supported network cards

o  Supplied files

o  Network card installation notes

o  Dual channel support

o  Install the driver

o  Modifying the data rate

OVERVIEW

This document explains how to install the Windows for Workgroups

v3.1 and v3.11 DOS NDIS2 driver for X86-based platforms.

SUPPORTED NETWORK CARDS

o  EtherPower Ethernet network cards

(SMC8432 board types)

o  EtherPower^2 Dual-Channel Ethernet network cards

(SMC8434 board types)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332DST board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332BDT board type)

o  EtherPower 10/100 Dual-Channel

Fast Ethernet 100BASE-TX network cards

(SMC9334BDT board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-T4 network cards

(SMC9332BVT board type)

SUPPLIED FILES

General files

o  RELEASE.DOC      Release notes for this version of the driver.

o  WIN_WKGP.DOC     This file.

Files to install the driver on Windows for Workgroups

o  SMCPWR.DOS       DOS NDIS2 driver.

o  OEMSETUP.INF     Network information file.

NETWORK CARD INSTALLATION NOTES

Refer also to the NDIS.DOC file located in the NDIS2 directory

for complete information regarding DOS and OS/2 NDIS2 drivers.

Important PROTOCOL.INI Parameters:

Configuration information is stored in the PROTOCOL.INI file. Two

important PROTOCOL.INI parameters are SIA_MODE, which defines the

serial interface connection (including full-duplex or half-duplex

operation), and DATA_RATE, which specifies the speed the network card

will operate at (10 Mbps [MB10] or 100 Mbps [MB100]).

SMC9334BDT EtherPower 10/100 network card ONLY:

During the network adapter selection process, the SMC9334BDT

EtherPower 10/100 dual channel network card may be represented as two

SMC9332BDT EtherPower 10/100 adapters (one for each channel). If no

option for 'SMC9334BDT EtherPower 10/100 Dual Channel' is presented,

select the option for 'SMC9332BDT EtherPower 10/100' for both channels

SMC9332DST EtherPower 10/100 network card ONLY:

Ensure that the line speed (10 Mbps vs. 100 Mbps) configured for the

network card matches the speed of the hub to which it is attached.

The SMC9332DST EtherPower 10/100 network card is default configured

to operate at 100 Mbps.

SMC9332BDT/SMC9332BVT/SMC9334BDT EtherPower 10/100 network cards ONLY:

The Auto-Negotiation feature of the SMC9332BDT EtherPower 10/100 network

card will set the operating speed of the network card (10 Mbps [MB10]

or 100 Mbps [MB100]). If the attached hub is full duplex capable and uses

Auto-Negotiation, the SMC9332BDT EtherPower 10/100 network card will

configure itself for full duplex operation. If the attached hub is full

duplex capable, but does not use Auto-Negotiation, you must manually

specify full duplex mode. Refer to the MODIFYING THE DATA RATE section

later in this document.

DUAL CHANNEL SUPPORT

EtherPower^2 and SMC9334BDT EtherPower 10/100

The EtherPower^2 Dual Channel PCI Ethernet and the SMC9334

EtherPower 10/100 Fast Ethernet network cards conform to the

PCI-to-PCI Bridge Architecture Specification, Revision 1.0, April 5,

1994. Networking environments that support dual channel operation

include Novell server (32-bit ODI driver), Windows NT, and SCO.

Other networking environments do not support the unique dual channel

capability of the EtherPower^2 or the SMC9334 EtherPower 10/100 network

card. These environments include Novell client (DOS ODI driver),

Windows for Workgroups v3.11, and networking environments that use DOS

or OS/2 NDIS2 drivers.

INSTALLING THE DRIVER

If you already have Windows for Workgroups installed and only want

to install the SMC driver, proceed to "Installing the SMC Driver

Only".

Installing Windows for Workgroups and the SMC Driver

The following procedures describe how to install Windows for

Workgroups v3.1 or v3.11 along with the driver files. These procedures

are designed to expedite the driver installation. For information

on customizing your installation and Windows desktop, refer to

your Windows for Workgroups manual.

Note: If you are installing an ODI or NDIS3 driver, you will

need the NETWARE.DRV file. You can download the current

version of this file from Novell's NetWare Forum on

CompuServe.

Windows for Workgroups v3.1

1. Insert the Windows for Workgroups Disk 1 Setup diskette in your

computer's disk drive.

2. Change to the drive where you inserted the disk in step 1.

Then, at the DOS prompt, type: SETUP.

3. Press the 'Enter' key to start the setup procedure. Typically,

this procedure takes from 10 to 15 minutes.

4. Follow the screen prompts until you receive a prompt that

says Windows has detected the SMC card and asks: "Do you want

to install the driver for this network card now".

5. Select 'No' since this driver is not as current as the one

that SMC provides.

6. When the 'Install New Network Adapter' dialog box appears with

a list of network cards, select 'Unlisted or Updated Network

Adapter'.

7. Select 'Continue'.

8. When the 'Install Driver' window appears, insert your SuperDisk

into your computer's disk drive.

9. In the 'Install Driver' window, type the path where the

driver resides (for example, A:\).

10. Select 'OK'. The 'Unlisted or Updated Network Adapter'

window appears.

11. Select the appropriate SMC network card (8432 - EtherPower,

8434 - EtherPower^2, 9332 or 9334 - EtherPower 10/100) you are

using followed by 'OK'.

12. Follow the screen prompts to finish your Windows for Workgroups

v3.1 installation.

Windows for Workgroups v3.11

1. Insert the Windows for Workgroups Disk 1 Setup diskette in your

computer's disk drive.

2. Change to the drive where you inserted the disk in step 1.

Then, at the DOS prompt, type: SETUP.

3. Press the 'Enter' key to start the setup procedure. Typically,

this procedure takes from 10 to 15 minutes.

4. Follow the screen prompts until the 'Network Setup' window

appears.

5. Select 'Network'.

6. Select 'Install Microsoft Windows Network'

7. Select 'Sharing' and specify whether you want other users to

be able to share your files and printer(s). Then select

'OK'.

8.  Select 'Drivers'.

9.  Select 'Add Adapter'. The 'Add Network Adapter' dialog box appears.

10. Select 'Unlisted or Updated Network Adapter' followed by 'OK'.

11. When the 'Install Driver' window appears, insert your SuperDisk

into your computer's disk drive.

12. In the 'Install Driver' window, type the path where the

driver resides (for example, A:\).

13. Select 'OK'. The 'Unlisted or Updated Network Adapter' window

appears.

14. Select the appropriate SMC network card (8432 - EtherPower,

8434 - EtherPower^2, 9332 or 9334 - EtherPower 10/100) you are

using followed by "OK".

15. Follow the screen prompts to finish your Windows for Workgroups

v3.11 installation.

Adding the SMC Driver to an Existing Windows for Workgroups System

The following procedures describe how to install the SMC driver

and assumes that either Windows for Workgroups v3.1 or v3.11 is already

installed on your system.

If you are using Windows for Workgroups v3.1 and want to add the

SMC driver:

1. Double-click on the 'Control Panel' icon in the 'Main' window.

2. Double-click on the 'Network' icon.

3. Click on the 'Adapter' button and then on the 'Add' button.

4. Select 'Unlisted or Updated Network Card' followed by 'OK'. The

'Install Driver' dialog box appears.

5. Specify the path where the SMC Windows for Workgroups v3.1 driver

is located. If the driver resides on a floppy diskette, insert

the diskette into an available disk drive.

6. Select 'OK'. The 'Unlisted or Updated Network Adapter' dialog

box appears.

7.  Select 'OK' when the next two dialog boxes appear.

8. Select 'Close'. The 'Network Settings' dialog box appears.

9. Select 'OK'. You will be prompted to either accept all of the

options you selected and restart your computer or return to DOS.

Restart your computer so your options can take effect.

If you are using Windows for Workgroups v3.11 and want to add the

SMC driver:

1.  Double-click on the Network Setup icon in the Network window.

2.  When the 'Network Setup' dialog box appears

3.  Select 'Drivers'.

4.  Select 'Add Adapter'. The 'Add Network Adapter' dialog box appears.

5.  Select 'Unlisted or Updated Network Adapter' followed by 'OK'.

6.  When the 'Install Driver' window appears, insert your SuperDisk

into your computer's disk drive.

7.  In the 'Install Driver' window, type the path where the

driver resides (for example, A:\).

8.  Select 'OK'. The 'Unlisted or Updated Network Adapter' window

appears.

9.  Select the appropriate SMC network card (8432 - EtherPower,

8434 - EtherPower^2, 9332 or 9334 - EtherPower 10/100) you are

using followed by "OK".

10. Follow the screen prompts to finish your Windows for Workgroups

v3.11 installation.

MODIFYING THE DATA RATE

SMC9332DST EtherPower 10/100 ONLY:

Ensure that the data rate (10 Mbps or 100 Mbps) configured for the

network card matches the speed of the hub to which it is attached.

Set the network card to operate at the correct data rate (10 Mbps

or 100 Mbps) using the PROTOCOL.INI parameter DATA_RATE.

To operate at 10 Mbps, remove the comment (;) indicator from the

line:

DATA_RATE=MB10

To make 10 Mbps the active data rate selection. Other parameters,

such as the active serial interface, are also specified using this

file.

If the SIA_MODE is set at AUTOSENSE, the driver will detect and

set the correct data rate during power-up.

SMC9332BDT/SMC9334BDT/SMC9332BVT EtherPower 10/100 ONLY:

This network card supports NWAY (Auto-Negotiation). Setting the

SIA_MODE to AUTOSENSE will allow the driver to automatically

detect the line speed (10 Mbps [MB10] or 100 Mbps [MB100]) and

operate at the highest possible data rate.

The recommended setting for the SIA_MODE is AUTOSENSE. If you

manually set the SIA_MODE to anything other than AUTOSENSE,

(for example, either half-duplex or full-duplex operation), the

settings you select must be supported by the hub you are attaching to.

The methods used to manually specify operating speed will vary

depending on your networking software and the driver being used.

All SMC drivers for the SMC9332BDT/SMC9334BDT/SMC9332BVT EtherPower

10/100 network card support five different data rate settings.

AUTOSENSE -  when using this default setting, the driver

will automatically detect the line speed of the attached hub

and set the network card to operate at the highest possible data

rate

10 Mbps

Full duplex - network card operates at 10 Mbps only

10 Mbps

Half duplex - network card operates at 10 Mbps only

100 Mbps

full duplex - network card operates at 100 Mbps only

100 Mbps

half duplex - network card operates at 100 Mbps only

After completing the installation and configuration, you must

reboot your computer.

Refer to the section in NDIS.DOC titled PROTOCOL.INI PARAMETERS

for further details.

Below is a Sample CONFIG.SYS, PROTOCOL.INI, AUTOEXEC.BAT for WFW 3.11

PROTOCOL.INI

[network setup]

version=0x3110

netcard=SMC$8432,1,SMC$8432,3

transport=ms$ndishlp,MS$NDISHLP

transport=ms$netbeui,NETBEUI

transport=ms$nwlinknb,NWLINK

lana0=SMC$8432,1,ms$nwlinknb

lana1=SMC$8432,1,ms$netbeui

lana2=SMC$8432,1,ms$ndishlp

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NDISHLP

[SMC$8432]

DriverName=SMCPWR$

SIA_MODE=TP

[SMPWRW]

Adapters=SMC$8432

[NWLINK]

BINDINGS=SMC$8432

[MS$NDISHLP]

DriverName=ndishlp$

BINDINGS=SMC$8432

[NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=12

BINDINGS=SMC$8432

LANABASE=1

CONFIG.SYS

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\DOS\EMM386.EXE NOEMS

DEVICE=C:\WINDOWS\IFSHLP.SYS

LASTDRIVE=Z

AUTOEXEC.BAT

@ECHO OFF

C:\WINDOWS\net start

PATH=C:\WINDOWS;C:\DOS;

WIN

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is SMC Networks, Inc.,  350 Kennedy,

Hauppauge,  NY    11788.

SMC is a registered trademark; and EtherPower, EtherPower^2,

EtherPower 10/100, EZStart, and SuperDisk

are trademarks of SMC Networks, Inc.. Other product and

company names are registered trademarks or trademarks of their

respective holders.