STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows NT v4.0 NDIS v3.1 Driver

For EtherPower, EtherPower^2, and the EtherPower 10/100 Family

of PCI Network Cards

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1996 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Supported network cards

o  An overview

o  Preinstallation hardware and software requirements

o  Supplied files

o  Network card installation notes

o  Dual channel support

o  Driver installation

- Primary installation (network card not presently installed)

- Network card already installed or older driver previously loaded

o  Modifying the data rate

SUPPORTED NETWORK CARDS

o  EtherPower Ethernet network cards

(SMC8432 board types)

o  EtherPower^2 Dual-Channel Ethernet network cards

(SMC8434 board types)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332DST board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332BDT board type)

o  EtherPower 10/100 Dual-Channel

Fast Ethernet 100BASE-TX network cards

(SMC9334BDT board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-T4 network cards

(SMC9332BVT board type)

OVERVIEW

This file explains how to install the Windows NT driver for x86-based

platforms.

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

- An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus.

- An EtherPower, EtherPower^2, or EtherPower 10/100 PCI

network card for that bus.

Software Requirements:

- Windows NT v4.0

SUPPLIED FILES

General files

o  NT_40.DOC        This file

o  RELEASE.DOC      Release notes for this version of the driver

Files to install the driver on Windows NT

o  SMCPWRN.SYS      The driver

o  SMCPWRN.HLP      The driver's associated help file

o  OEMSETUP.INF     The driver's setup file

NETWORK CARD INSTALLATION NOTES

Refer to the user guide for complete installation instructions.

When installing an SMC PCI network card in a PCI computer, make sure

the slot you select supports bus master operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using the

appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the appropriate

motherboard jumper.

3. Enter your computer's setup menu and:

- Enable bus master on the PCI slot.

- Enable I/O on the PCI slot.

- Define interrupt number on the PCI slot.

- Modify CMOS settings to make an IRQ available to the PCI bus

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

DUAL CHANNEL SUPPORT

SMC8434 EtherPower^2 and SMC9334BDT EtherPower 10/100

The SMC8434 EtherPower^2 Dual Channel PCI Ethernet and the SMC9334

EtherPower 10/100 Fast Ethernet network cards conform to the

PCI-to-PCI Bridge Architecture Specification, Revision 1.0, April 5,

1994. Networking environments that support dual channel operation

include Novell server (32-bit ODI driver), Windows NT, and SCO

UNIX. Other networking environments may not support the unique

dual channel capability of the EtherPower^2 or the SMC9334

EtherPower 10/100 network card.

SMC9334BDT EtherPower 10/100 network card ONLY:

During the network adapter selection process, the SMC9334BDT

EtherPower 10/100 dual channel network card may be represented as two

SMC9332BDT EtherPower 10/100 adapters (one for each channel). If no

option for 'SMC9334BDT EtherPower 10/100 Dual Channel' is presented,

select the option for 'SMC9332BDT EtherPower 10/100' for both channels.

SMC9332DST EtherPower 10/100 network card ONLY:

Ensure that the Data Rate (10 Mbps vs. 100 Mbps) configured for the

network card matches the speed of the hub to which it is attached.

The SMC9332DST EtherPower 10/100 network card is default configured

to autodetect the Data Rate.

DRIVER INSTALLATION

o Primary installation (network card not presently installed)

If you've already installed another card or loaded another driver, refer

to the procedure titled 'Network card already installed or older driver

previously loaded'.

1.  Complete the Windows NT installation without an SMC network card

installed in your PC.

2.  Start Windows NT.

3.  Double-click the 'My Computer', 'Control Panel', and 'Network' icons.

The 'Network Configuration' window appears, prompting you to install

Windows. Select 'Yes'. Windows NT invokes the 'Network Setup Wizard'.

Click on 'Next'.

4.  When prompted to 'Search for a Network Adapter', click on 'Select from

List'.

5.  When prompted to 'Select a Network Adapter', click on 'Have Disk'.

Insert the SuperDisk when prompted and click 'OK'.

6.  You are presented with the 'Select OEM Option' window. Highlight

'SMC EtherPower Family Network Card' and click 'OK'.

7.  When prompted to 'Search for an Adapter', click on 'Next'.

8.  Select network protocols when prompted and click 'Next'.

NOTE:  To complete a TCP/IP installation, you will need to know your

IP Address and Subnet Mask.

9.  Click 'Next' and click 'Next' again when prompted to 'Install Network

Components'. When prompted for Windows NT files, type the path to your

Windows NT files on CD-ROM (e.g., D:\WINNT), or change the path to

those files on your hard drive, and click 'OK'.

10. You are presented with the 'Network Card Setup' window where you can

specify the network card data rate. 'AutoSense' is the recommended

setting for all EtherPower family network cards. Select 'Continue'

after verifying network card settings.

11. Click on 'Next' and click 'Next' again when prompted to 'Start the

Network'.

12. Enter the 'Workgroup' or 'Domain' names (optional) when prompted and

click on 'Next'.

13. Select 'Finish' when prompted and click on 'Yes' when prompted to

reboot. Remove the SuperDisk.

14. FOR FURTHER CUSTOMIZATION

You can modify card settings via the 'Network' applet. Install

additional protocols and network components by selecting the

appropriate folder (e.g., Customizing Protocols, Services,

Identification, or Bindings).

o Network card already installed or older driver previously loaded

Follow this procedure to replace Windows NT driver files with the latest

SMC driver files. By removing driver files, you will lose network settings.

Make sure you have a record of settings such as IP address, workgroup

name, etc. Check with your network administrator for complete information.

1.  Install the SMC network card, attach network cable, and boot Windows NT.

2.  Double-click the 'My Computer', 'Control Panel', and 'Network' icons.

From the 'Network' window, select the 'Adapter' tab.

3.  Highlight the network driver you wish to remove and click 'Remove'.

(You may choose to record network card settings for future reference.

If so, highlight the network card and click on 'Properties'.)

Click 'Yes'.

4.  Click 'Close' and select 'Yes' when prompted to restart.

5.  Double-click the 'My Computer', 'Control Panel', and 'Network' icons

and select the 'Adapters' tab.

6.  Do not select any of the network cards listed. Select 'Add'.

7.  You are presented with the 'Select Network Adapter' window. Click

on 'Have Disk'.

8.  You are presented with the 'Insert Disk' window. Specify the path to

the root directory of the SuperDisk (e.g., A:>) and click 'OK'.

9.  You are presented with the 'Select OEM Option' window. Highlight

'SMC EtherPower Family Network Card' and click 'OK'.

10. You are presented with the 'Network Card Setup' window where you can

specify the network card data rate. 'AutoSense' is the recommended

setting for all EtherPower family network cards. Select 'OK' after

verifying network card settings.

11. You are presented with the 'Network' window where the SMC network card

is now listed as an installed adapter. At this point you can select

'Close' to exit the 'Network' applet and follow the prompts to restart

Windows NT or, you may choose to perform the optional procedures

described below.

Optional:  Install additional protocols and network components by

selecting the appropriate folder (e.g., Customizing Protocols,

Services, Identification, or Bindings). Refer to the

Microsoft Windows NT installation guide for more information.

After completing your changes click on 'Close' to exit the

'Network' applet.

12. Select 'Yes' when prompted to reboot for the changes to take effect.

MODIFYING THE DATA RATE

o SMC9332DST EtherPower 10/100 ONLY:

The default parameter for data rate is AutoDetect. This default

setting will automatically detect the line speed at power up only.

If AutoDetect fails to work, ensure that the data rate (10 Mbps or

100 Mbps) configured for the network card matches the speed of the

hub to which it is attached.

o SMC9332BDT/SMC9332BVT/SMC9334BDT EtherPower 10/100 ONLY:

This network card supports NWAY (Auto-Negotiation). Setting the

data rate to AutoSense will allow the driver to automatically

detect the line speed (10 Mbps or 100 Mbps) and operate at the

highest possible data rate.

The recommended setting for the data rate is AutoSense. If you

manually set the data rate to anything other than AutoSense,

(for example, either half-duplex or full-duplex operation), the

settings you select must be supported by the hub you are attaching

to.

The data rate options available will vary depending on the network

card you are using.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and

EtherPower, EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk

are trademarks of Standard Microsystems Corporation. Other product and

company names are registered trademarks or trademarks of their

respective holders.