SMC

Novell NetWare 386 v4.x Server Driver

For EtherPower, EtherPower^2, and the EtherPower 10/100 Family

of PCI Network Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1997 SMC Networks, Inc.. All rights reserved.

This installation guide describes:

o  An overview

o  Supported adapters

o  Preinstallation hardware and software requirements

o  Supplied files

o  Adapter installation notes

o  Using EZStart to copy the Novell NetWare 386 v4.x server

driver (SMCPWR.LAN) from the SuperDisk to your server

o  Loading the LAN driver and establishing a network connection

o  Driver command line parameters

o  Installing multiple adapters

o  Troubleshooting

o  Error messages

OVERVIEW

This file explains how to install and load the Novell NetWare 386

v4.x server driver.

SUPPORTED ADAPTERS

o  EtherPower Ethernet PCI adapters (8432 board types)

o  EtherPower^2 Dual-Channel PCI Ethernet adapters (8434 board types)

o  EtherPower 10/100 Fast Ethernet PCI adapters (9332 board types)

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

- An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus supporting bus master mode.

- An EtherPower, EtherPower^2, OR EtherPower 10/100

PCI adapter for that bus.

Software Requirements:

- Novell NetWare 386 v4.x

SUPPLIED FILES

General files

o  INSTALL.DOC      This file.

o  RELEASE.DOC      Release notes for this version of the driver.

Files to install the Novell NetWare 386 v4.x driver

o  SAMPLE.4X

o  SMCPWR.LAN

o  SMCPWR.LDI

o  SMCNLM.EXE     Self-extracting *NLM Server software for basic

connectivity.

* NLM files are copyrighted by Novell. Do not distribute these

files without permission from Novell.

ADAPTER INSTALLATION NOTES

Refer to the user guide for complete installation instructions.

When installing an SMC PCI adapter in a PCI computer, make sure

the slot you select supports bus master operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using the

appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the appropriate

motherboard jumper.

3. Enter the computer's setup menu and:

- Enable bus master on the PCI slot.

- Enable I/O on the PCI slot.

- Define interrupt number on the PCI slot.

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

EtherPower 10/100 Users Only:

Ensure that the data rate (10 Mbps [MB10] or 100 Mbps [MB100])

configured for the adapter matches the speed of the hub to which

it is attached.

EtherPower 10/100 B Only:

This adapter supports NWAY (Auto-Negotiation). Setting the

SIA_MODE to AUTOSENSE will allow the driver to automatically

detect the line speed (10 Mbps [MB10] or 100 Mbps [MB100]) and

operate at the highest possible data rate.

The methods used to manually specify the data rate of the adapter

will vary depending on your networking software, the driver

being used, and the hub (or concentrator) you are connecting to.

All SMC drivers for the EtherPower 10/100 B adapter support five

different data rate settings.

AUTOSENSE -  when using this default setting, the driver

will automatically detect the line speed of the attached hub

and set the adapter to operate at the highest possible data rate

10 Mbps

Full duplex - adapter operates at 10 Mbps only

10 Mbps

Half duplex - adapter operates at 10 Mbps only

100 Mbps

full duplex - adapter operates at 100 Mbps only

100 Mbps

half duplex - adapter operates at 100 Mbps only

COPYING THE DRIVER FILES

From the Driver Installation screen, select 'Copy Files' to copy all

the files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that

you specify.

LOADING THE DRIVER

Installing the LAN driver is part of the NetWare installation

process. Use the following instructions with those

in the NetWare 386 installation manual to install the driver.

You will need the SuperDisk during this procedure.

1. Refer to the "Load LAN driver module" section in the Novell

NetWare 386 v4.x installation manual for instructions on

installing NetWare.

2. Follow the instructions on starting NetWare and configuring

the server. Use the Load command to load the LAN driver.

If the LAN driver is not located in the default directory,

specify the path to the file on the command line. The

Load command will prompt you for configuration parameters

if they are not supplied on the command line.

The Load command line parameters may be saved in an

AUTOEXEC.NCF file, so they execute automatically every time

the server starts. Refer to the NetWare 386 installation

manual on how to create an AUTOEXEC.NCF file.

3. From the command line on the server, type: LOAD MSM.NLM.

4. Press the Enter key to load MSM.NLM.

5. From the command line on the server, type: LOAD ETHERTSM.NLM.

6. Press the Enter key to load ETHERTSM.NLM.

Note: The following steps are specific to loading SMCPWR.LAN.

For examples, please refer to the SAMPLE.4X demo file on

this diskette.

ETHERPOWER 10/100 USERS ONLY:

Default line speed of the adapter is 100 Mbps. To change the

line speed to 10 Mbps, add the following to the command line

load string:

"LOAD SMCPWR LINE_SPEED=10"

7. If the adapter is installed into PCI slot 17, add the following

command line load string:

"LOAD SMCPWR SLOT=17"

8. At this point, a protocol must be bound to the driver. For example:

BIND IPX TO SMCPWR NET=XX

where xx is the logical network address for the frame type

indicated in your load command.

DRIVER COMMAND LINE PARAMETERS

The only required parameter is the slot number if the adapter is

not installed in slot 3.

Example: load C:SMCPWR SLOT=4<Enter>

ISA/EISA slots for this driver are counted from 1 to 15, PCI slots

for this driver are counted from 3. The first PCI adapter installed

in your computer will be treated as if it is installed in slot 3,

the next one will be in slot 4, and so on.

INSTALLING MULTIPLE ADAPTERS

If you have only one PCI adapter in the server, there is no need to

specify slot number in the LOAD command line. If there is more than

one PCI adapter, the driver will detect it and ask you for a slot

selection, or you can specify slot number in the LOAD command line.

For examples, please refer to the SAMPLE.4X demo file on this

diskette.

TROUBLESHOOTING

If you experience excessive 'Tx FIFO Underflow' or 'Rx FIFO

Overflow' counts in your server Monitor program in the LAN/WAN menu

in your driver custom counters, and you have a PCI computer with

Saturn or Mercury chipset, try using the TURBO keyword when you

load your driver.

Example:

load SMCPWR frame=ethernet_II TP TURBO

Note that some PCI computers may not correctly support 32-bit

protected mode PCI bios calls. To load the driver on these

computers, use the BIOS16 keyword in the driver invocation line.

This will cause the driver to access the computer's BIOS in real

mode fashion.

Note that accessing the PCI BIOS in 32-bit protected mode is the

native mode in 32-bit protected mode operating systems.

Note that during the driver loading process the file NBI.NLM is auto-loaded

and produces the following warning message:

"This version of NetWare does not support

protected mode BIOS accesses.  Without a

loader patch some PCI drivers may fail."

This is a Novell supplied warning and should not affect the operation

of the driver.

ERROR MESSAGES

o 50 - No EISA nor PCI board found in machine!

Driver could not detect a supported adapter.

o 73 - Unable to allocate memory

Driver failed to allocate needed memory from the operating system

(receive/transmit descriptors or needed transmit buffers).

o 200 - Timeout when initializing DecChip 21X4 with setup packet!

Occurs when the PCI slot does not support DMA. Check that BIOS

setup or motherboard switches are set to enable DMA and/or bus

master operation for the slot.

o 210 - Driver failed to locate either EISA or PCI bus!

The computer is not identified as an EISA- or PCI-bus computer.

o 213 - Unexpected OS behavior. Driver supports 3.11 and later

Identifies a failed assumption regarding the operating system

behavior.

o 227 - Internal CRC table corrupted

Driver found an error in internal CRC table.

o 228 - Could not handle chipset bug workaround successfully

Failed to operate chipset bug workaround because PCI BIOS returned

a fail status in one of the calls that operates this mechanism.

o 229 - This machine Host to PCI bridge has a known bug. Driver

limits DC21X4 bursts as a workaround.

This message indicates that your computer does not correctly

support the PCI Local Bus Specification (v2.0). The SMC driver

will work in this environment, but you are urged to contact the

computer manufacturer to determine if your BIOS correctly supports

the features of the PCI Local Bus Specification (v2.0).

o 240 - Unable to locate or read the Ethernet ROM address on board!

Could be caused by corrupt data stored in the IEEE serial ROM.

o 244 - Serial ROM-Read Failed

Driver failed to read the IEEE serial flash ROM due to hardware

handshake failure. Make sure the IEEE flash ROM on your adapter

is operational using EZStart's Basic Adapter Test.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is SMC Networks, Inc.,  350 Kennedy,

Hauppauge,  NY    11788.

SMC is a registered trademark; and EtherPower,

EtherPower^2, EtherPower 10/100, EtherPower 10/100 B, EZStart, and

SuperDisk are trademarks of SMC Networks, Inc.. Other

product and company names are registered trademarks or trademarks of

their respective holders.