STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows for Workgroups v3.11 NDIS v3.0 Driver

For EtherPower, EtherPower^2, and the EtherPower 10/100 Family

of PCI Network Cards

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1996 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  An overview

o  Supported network cards

o  Preinstallation hardware and software requirements

o  Supplied files

o  Network card installation notes

o  Driver installation

- Installing Windows for Workgroups and the SMC Driver

- Installing the SMC Driver Only

o  Modifying the data rate

OVERVIEW

This file explains how to install the Windows NT driver for x86-based

platforms.

SUPPORTED NETWORK CARDS

o  EtherPower Ethernet network cards

(SMC8432 board types)

o  EtherPower^2 Dual-Channel Ethernet network cards

(SMC8434 board types)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332DST board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332BDT board type)

o  EtherPower 10/100 Dual-Channel

Fast Ethernet 100BASE-TX network cards

(SMC9334BDT board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-T4 network cards

(SMC9332BVT board type)

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

- An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus.

- An EtherPower, EtherPower^2, or EtherPower 10/100 network

card for that bus.

Software Requirements:

- Windows for Workgroups v3.11

SUPPLIED FILES

General files

o  RELEASE.DOC      Release notes for this version of the driver

o  WFW_311.DOC     This file

Files to install the driver on Windows for Workgroups

o  SMCPWRW.38_      NDIS3 driver

o  SMCPWR.DO_       NDIS3 driver

o  OEMSETUP.INF     Network information file

NETWORK CARD INSTALLATION NOTES

Refer to the user guide for complete network card installation instructions.

When installing an SMC PCI network card in a PCI machine, make sure the

slot you select supports bus master operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using

the appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the appropriate

motherboard jumper.

3. Enter your computer's setup menu and:

- Enable bus master on the PCI slot.

- Enable I/O on the PCI slot.

- Define interrupt number on the PCI slot.

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

Important Parameters in Advanced Setup:

Configuration information is manually modified using the Advanced Setup

Option located in Network->Network Setup->Setup->Advanced. The

DATA RATE is used to specify the speed the network card will

operate at (10 Mbps or 100 Mbps).

SMC9334BDT EtherPower 10/100 network card ONLY:

During the network card selection process, the SMC9334BDT

EtherPower 10/100 dual channel network card may be represented as two

SMC9332BDT EtherPower 10/100 adapters (one for each channel). If no

option for 'SMC9334BDT EtherPower 10/100 Dual Channel' is presented,

select the option for 'SMC9332BDT EtherPower 10/100' for both channels

SMC9332DST EtherPower 10/100 network card ONLY:

Ensure that the line speed (10 Mbps vs. 100 Mbps) configured for the

network card matches the speed of the hub to which it is attached.

SMC9332BDT/SMC9334BDT EtherPower 10/100 network cards ONLY:

The Auto-Negotiation feature of the SMC9332BDT/SMC9334BDT EtherPower

10/100 network card will set the operating speed of the network card

(10 Mbps [MB10] or 100 Mbps [MB100]). If the attached hub is full duplex

capable and uses Auto-Negotiation, the SMC9332BDT/SMC9334BDT EtherPower

10/100 network card will configure itself for full duplex operation. If

the attached hub is full duplex capable, but does not use Auto-

Negotiation, you must manually specify full duplex mode. Refer to the

MODIFYING THE DATA RATE section later in this document.

DRIVER INSTALLATION

o Installing Windows for Workgroups and the SMC Driver

The following procedure describes how to install Windows for

Workgroups v3.11 along with the driver files. This procedure

is designed to expedite the driver installation. For information

on customizing your installation and Windows desktop, refer to

your Windows for Workgroups manual.

If you already have Windows for Workgroups installed and only want

to install the SMC driver, proceed to 'Installing the SMC Driver Only'.

Note: To install an ODI driver, you will need the NETWARE.DRV file.

You can download the current version of this file from Novell's

NetWire Forum on CompuServe.

1. Insert the Windows for Workgroups Disk 1 Setup diskette in your

computer's disk drive.

2. Change to the drive where you inserted the disk in step 1.

Then, at the DOS prompt, type: SETUP

3. Press the Enter key to start the setup procedure. Typically,

this procedure takes from 10 to 15 minutes.

4. Follow the screen prompts until the Network Setup window

appears.

a. Select "Advanced"

b. Select "Networks" and "Install Microsoft Windows Network".

c. Select "No additional network" or "Other", as necessary.

Click on "OK". Refer to your Windows for Workgroups manual

for information.

d. Select "Sharing" and specify whether you want other users to

be able to share your files and printer(s). Then select

"OK".

e. Select "Drivers".

f. Select "Add Adapter". The Add Network Adapter dialog box

appears.

g. Select "Unlisted or Updated Network Adapter" followed by

"OK".

5. When the Install Driver window appears, insert the SuperDisk

into your computer's disk drive.

6. In the Install Driver window, specify the path to the driver

on the SuperDisk (e.g., A:\WFW).

7. Select "OK". The Unlisted or Updated Network Adapter window

appears.

8. Select the appropriate SMC network card (8432 - EtherPower

with Rom Socket, 8432 without Rom Socket, 8434 EtherPower2,

SMC 9332BVT EtherPower 10/100T4, 9332BDT/9334BDT-EtherPower

10/100, SMCDST EtherPower 10/100) you are installing followed

by "OK".

9. When the Network Drivers window appears, you can use

"Add Adapter" and/or "Add Protocol" to add more

network cards (up to four) and protocols. Refer to your

Windows for Workgroups manual for more information.

o  To change the Advanced Network-Adapter Settings and/or

select a driver type, select "Setup" and make the approp-

riate selections. (The default selection is 'Real Mode and

Enhanced Mode').

o  To change the Burst Length, Data Rate, Pci Mechanism, Receive

Buffers, Store and Forward, Transmit Threshold, or Transmit

Threshold 100mbps, click 'Advanced' and make appropriate

selections.

o  Select "Close" followed by "OK".

o  When prompted with the Microsoft Windows Networks Names box,

select "OK".

o  Follow the screen prompts to complete the driver

installation.

Note: The "Install Driver" window will appear. You will be prompted

to insert the SMC SuperDisk or a disk with the updated or vendor-

provided SMCPWR.DOS file. The driver source path is A:\WFW. You

will be prompted to do the same thing to update the SMCPWR.386 file.

The driver source path is A:\WIN95NT.

11. At the end of the installation, you will be prompted whether

you want to accept all the options you selected and restart

your computer or return to DOS. Restart your computer so

your options can take effect.

Some specifics of the installation process may vary according to

each user's needs. Refer to the Microsoft Windows for Workgroups

user manual for more information.

o Installing the SMC Driver Only

The following procedure describes how to install the SMC driver

and assumes that Windows for Workgroups v3.11 is already installed.

1.  Double-click on the Network Setup icon in the Network window.

2.  When the Network Setup dialog box appears, refer to "Installing

Windows for Workgroups and the SMC Driver", step 4b.

MODIFYING THE DATA RATE

SMC9332DST EtherPower 10/100 ONLY:

Ensure that the data rate (10 Mbps or 100 Mbps) configured for the

network card matches the speed of the hub to which it is attached.

Set the network card to operate at the correct data rate (10 Mbps

or 100 Mbps) using the Data Rate parameter in Network Setup,

Setup, Advanced Options for the SMC Driver.

If the DATA RATE is set at AUTOSENSE, the driver will detect and

set the correct data rate during power-up.

SMC9332BDT/SMC9334BDT EtherPower 10/100 ONLY:

This network card supports NWAY (Auto-Negotiation). Setting the

DATA RATE to AUTOSENSE will allow the driver to automatically

detect the line speed (10 Mbps [MB10] or 100 Mbps [MB100]) and

operate at the highest possible data rate.

The recommended setting for the DATA RATE is AUTOSENSE. If you

manually set the DATA RATE to anything other than AUTOSENSE,

(for example, either half-duplex or full-duplex operation), the

settings you select must be supported by the hub you are attaching to.

The methods used to manually specify operating speed will vary

depending on your networking software and the driver being used.

All SMC drivers for the SMC9332BDT EtherPower 10/100 network card

support five different data rate settings.

AUTOSENSE -  when using this default setting, the driver

will automatically detect the line speed of the attached hub

and set the network card to operate at the highest possible data rate.

10 Mbps

Full duplex - network card operates at 10 Mbps only

10 Mbps

Half duplex - network card operates at 10 Mbps only

100 Mbps

full duplex - network card operates at 100 Mbps only

100 Mbps

half duplex - network card operates at 100 Mbps only

After completing the installation and configuration, you must

reboot your computer.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are trademarks

of Standard Microsystems Corporation. Other product and company names

are registered trademarks or trademarks of their respective holders.