STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows 95 NDIS v3.1, NDIS v2.01, and DOS ODI Drivers

For EtherPower, EtherPower^2, and the EtherPower 10/100 Family

of PCI Network Cards

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1996 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Supported network cards

o  An overview

o  Preinstallation hardware and software requirements

o  Supplied files

o  Network card installation notes

o  Dual channel support

o  Driver installation

o  Manual driver removal

o  Modifying the data rate

SUPPORTED NETWORK CARDS

o  EtherPower Ethernet network cards

(SMC8432 board types)

o  EtherPower^2 Dual-Channel Ethernet network cards

(SMC8434 board types)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332DST board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332BDT board type)

o  EtherPower 10/100 Dual-Channel

Fast Ethernet 100BASE-TX network cards

(SMC9334BDT board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-T4 network cards

(SMC9332BVT board type)

OVERVIEW

This file explains how to install the Windows 95 driver for x86-based

platforms.

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

- An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus.

- An EtherPower, EtherPower^2, or EtherPower 10/100 PCI

network card for that bus.

Software Requirements:

- Windows 95.

SUPPLIED FILES

General files

o  WIN_95.DOC       This file

o  RELEASE.DOC      Release notes for this version of the driver

Files to install the driver on Windows 95

o  SMCPWRN.SYS      The NDIS3 32-bit driver

o  SMCPWR.DOS       The NDIS2 16-bit driver

o  SMCPWR.COM       The ODI 16-bit driver

o  NETPCI.INF       The driver's setup file

One other file, LSL.COM, is required for successful ODI driver

installation operation. It can be obtained from Novell's BBS.

NETWORK CARD INSTALLATION NOTES

o  When installing an SMC PCI network card in a PCI computer, make sure

the slot you select supports bus master operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using the

appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the appropriate

motherboard jumper.

3. Enter your computer's BIOS setup menu and:

a. Enable bus master on the PCI slot.

b. Enable I/O on the PCI slot.

c. Define interrupt number on the PCI slot.

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

SMC9332DST EtherPower 10/100 network card ONLY:

Ensure that the line speed (10 Mbps vs. 100 Mbps) configured for the

network card matches the speed of the hub to which it is attached.

The SMC9332DST EtherPower 10/100 network card is default configured

to operate at 100 Mbps. This card supports automatic data rate speed

detection at power-up only.

SMC9332BDT EtherPower 10/100 network cards ONLY:

The Auto-Negotiation feature of the SMC9332BDT EtherPower 10/100 network

card will set the operating speed of the network card (10 Mbps [MB10]

or 100 Mbps [MB100]). If the attached hub is full duplex capable and uses

Auto-Negotiation, the SMC9332BDT EtherPower 10/100 network card will

configure itself for full duplex operation. If the attached hub is full

duplex capable, but does not use Auto-Negotiation, you must manually

specify full duplex mode. Refer to the MODIFYING THE DATA RATE section

later in this document.

SMC9334BDT EtherPower 10/100 network card ONLY:

During the network adapter selection process, the SMC9334BDT

EtherPower 10/100 dual channel network card may be represented as two

SMC9332BDT EtherPower 10/100 adapters (one for each channel). If no

option for 'SMC9334BDT EtherPower 10/100 Dual Channel' is presented,

select the option for 'SMC9332BDT EtherPower 10/100' for both channels

DUAL CHANNEL SUPPORT

EtherPower^2 and SMC9334BDT EtherPower 10/100

The EtherPower^2 Dual Channel PCI Ethernet and the SMC9334

EtherPower 10/100 Fast Ethernet network cards conform to the

PCI-to-PCI Bridge Architecture Specification, Revision 1.0, April 5,

1994. Networking environments that support dual channel operation

include Novell server (32-bit ODI driver), Windows NT, and SCO

UNIX. Other networking environments may not support the unique

dual channel capability of the EtherPower^2 or the SMC9334

EtherPower 10/100 network card.

DRIVER INSTALLATION

Do not perform a manual driver installation using the 'Have Disk' method.

The manual installation procedure bypasses the autodetection capabilities

of Windows 95. Instead, follow the procedure described below.

NOTE: To ensure the correct loading of the driver within Windows 95, you

must substitute the SMC NETPCI.INF file on the SMC SuperDisk for the

NETPCI.INF file included with Windows 95. You must also remove any

drivers previously loaded for SMC network cards. The procedure

described below performs these tasks automatically using the

W95INST installation utility (refer to step 3).

WARNING!

By removing the driver for a network card, you may lose protocol and client

network settings. Make sure you have a record of settings such as IP address

and workgroup name. Check with your network administrator for complete

information.

1. Complete the Windows 95 installation without an SMC network card

installed in your PC.

2. Start Windows 95. Insert the SMC SuperDisk into a floppy drive.

3. Select 'Start' and then 'Run'. Type A:\W95INST[Enter]. (Executing

the W95INST batch file will update the NETPCI.INF driver installation

file on your hard drive and remove any previously loaded drivers

for SMC network cards.)

4. Remove the SMC SuperDisk, shut down your PC, install the SMC network card,

and attach cable.

5. Restart Windows 95. Make sure you have your Windows 95 CD available

(or you are aware of the path to those files on your hard drive).

6. Windows 95 detects the addition of new hardware and begins installing

files from the Windows 95 CD (or the Windows 95 path you have

designated). The message 'Please insert the disk labeled SMC

SuperDisk' appears. Insert the SMC SuperDisk and click 'OK'.

7. The message 'The file NETPCI.INF on SMC SuperDisk could not be found'

appears. Specify the path to the A:\ root directory, and click 'OK'.

8. You may be prompted for Windows 95 installation files. If so, click

'OK' and then type the path to your Windows 95 CD, or change the path

to those files on your hard drive, and click 'OK'.

9. Follow the remaining Windows 95 instructions to complete the

installation. After all the files have been copied, you are prompted

to reboot for the changes to take effect. Select 'Yes' when prompted

to restart your computer and remove the SMC SuperDisk.

Optional procedures to modify card settings and add network protocols:

o Double-click on the 'My Computer', 'Control Panel', and 'Network'

icons. The 'Network' box appears. If the correct network protocols

are not installed, in the 'Primary Network Logon' list box, select

the primary network to which you will be attaching. Click 'Add'.

The 'Select Network Component Type' box appears. Follow the

0n-screen directions to select network protocols.

o For peer-to-peer networks, you may wish to enable 'File and Print

Sharing' in the 'Network' box.

o To modify other network card properties, select the appropriate tab

or click on the 'Properties' button from the 'Network' box and

modify properties as desired.

MANUAL DRIVER REMOVAL

You can run the W95INST batch file to  remove any previously loaded drivers

for SMC network cards. Running this batch file will also replace the

Windows 95 NETPCI.INF driver installation file on your hard drive with the

SMC NETPCI.INF file on the SMC SuperDisk.

If you do not choose to use the W95INST batch file provided on the SMC

SuperDisk (v2.3 or later), you can use the procedure described below to

remove previously loaded drivers.

WARNING!

By removing the driver for a network card, you may lose protocol and client

network settings. Make sure you have a record of settings such as IP address

and workgroup name. Check with your network administrator for complete

information.

1. Start Windows 95.

2. Click 'Start'. Move the mouse pointer to 'Settings>Control Panel' and

then click the mouse.

3. Double click the 'Network' icon.

4. Highlight the network driver to remove (PCI Ethernet, Fast Ethernet

DECchip Based Adapter, or SMC 9332 EtherPower 10/100 PCI Fast

Ethernet Adapter) and click 'Remove'.

5. Exit the 'Network' applet by clicking the 'OK' button and restart

Windows 95 when prompted.

6. Follow the procedure described in 'Driver installation' to load a

driver from the SMC SuperDisk.

MODIFYING THE DATA RATE (OPTIONAL)

1. Open the Control Panel and double-click on the "Network" icon.

2. Highlight the SMC driver in the "Network Components" list (click once).

3. Click on the "Properties" button.

4. Click on the "Advanced" tab and then highlight the "Data Rate" option.

5. Scroll to the "Value" you wish to use. (Not all values listed are

supported by all SMC PCI network cards.)

VALUE			SUPPORTED NETWORK CARDS

* AutoSense		All SMC EtherPower family PCI network cards

AutoDetect		SMC8432, SMC8434, SMC9332DST

100 Mbps T4		SMC9332BVT

100 Mbps		SMC9332DST, SMC9332BDT, SMC9334BDT

10 Mbps			All SMC EtherPower family PCI network cards

10 Mbps Full_Duplex	SMC8432, SMC8434, SMC9332BDT, SMC9334BDT

100 Mbps Full_Duplex	SMC9332DST, SMC9332BDT, SMC9334BDT,

SMC9332BVT

* AutoSense is the recommended setting for all SMC PCI network cards.

SMC9332BDT/SMC9334BDT/SMC9332BVT EtherPower 10/100 ONLY:

This network card supports NWAY (Auto-Negotiation). Setting the

Data Rate to AutoSense will allow the driver to automatically

detect the data rate (10 Mbps or 100 Mbps) and operate at the

highest possible data rate.

The recommended setting for the data rate is AutoSense. If you

manually set the data rate to anything other than AutoSense,

(for example, either half-duplex or full-duplex operation), the

settings you select must be supported by the hub you are attaching to.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are trademarks

of Standard Microsystems Corporation. Other product and company names are

registered trademarks or trademarks of their respective holders.