Release Notes for STB Pegasus (928)

Windows Drivers Update

December 1, 1993

This driver update for Windows 3.1 corrects several bugs that specific

customers have experienced.  These bugs are listed below.  Updating to these

drivers is unnecesary unless you have experienced one of these problems.

Note that the naming convention for these drivers has changed as follows.

Summary of New Device Driver Files

PEG6_8   DRV    158880 11-29-93   2:18p          640 x  480 x 256 Colors.

PEG8_8   DRV    158880 11-29-93   1:56p          800 x  600 x 256 Colors.

PEG10_8  DRV    158912 11-29-93   2:30p         1024 x  768 x 256 Colors.

PEG12_8  DRV    160960 11-29-93   2:52p         1280 x 1024 x 256 Colors.

PEG6_16  DRV    165120 11-29-93   4:41p          640 x  480 x 64K Colors.

PEG8_16  DRV    165120 11-29-93   4:46p          800 x  600 x 64K Colors.

PEG10_16 DRV    165088 11-29-93   2:33p         1024 x  768 x 64K Colors.

PEG12_16 DRV    167360 11-29-93   5:11p         1280 x 1024 x 64K Colors.

PEG6_24  DRV    168416 11-30-93   6:06p          640 x  480 x 16M Colors.

PEG8_24  DRV    168096 11-30-93   6:10p          800 x  600 x 16M Colors.

PEG10_24 DRV    168128 11-30-93   6:15p         1024 x  768 x 16M Colors.

Description of bugs corrected by this Driver Update

1.  In some applications, which display a wide box of text, the text would not

appear, or would appear and then immediately disappear.  This problem was very

intermittant and would only occur at resolutions below 1024 x 768, and only if

the text box was nearly as wide as the displayed resolution.  It also was

sensitive to the selected font resources and color depth.

2.  Winbench version 4.0 would cause a General Protection Fault in the DRV

module when the advanced stretch blt test was run.  The stretch blt had to

involve a patterned (3 operand) raster operation to fail.  This is the only

application I know of that would do this.

3.  Adobe Photoshop would cause a General Protection Fault while performing

certain stretch blt operations.  This problem would not occur with the 256 color

drivers, but with any of the others.

4.  The true color drivers would fail on some inferior motherboards, such as

those based on the Contaq chip set.  Simply moving large windows on the desktop

would corrupt the colors in the desktop image.

Clay Montgomery, Windows Driver Development