QUICK PALETTE CHANGER

~~~~~~~~~~~~~~~~~~~~~~~

I bet some of you out there who  do a lot of word processing would love

to find an easy way to change the rather dross GEM colours to something

more relaxing. Well, I very proudly present (very proudly, because it's

my first assembly program, which I did  all by my little self!) a small

program which can change the colours for you. Here it is:

clr.l -(sp)

move.w #$20,-(sp)

trap #1

add.l #6,sp

move.l d0,up_save

move.w #0,$ff8240

move.w #$777,$ff8242

move.w #3,$ff8244

move.w #$770,$ff8246

move.l #txt,-(sp)

move.w #9,-(sp)

trap #1

addq.l #6,sp

move.l #100000,d0

pause:

dbra d0,pause

move.l up_save,-(sp)

move.w #$20,-(sp)

trap #1

add.l #6,sp

move.w #20,-(sp)

move.w #$4c,-(sp)

trap #1

txt:   dc.b "BLACK EAGLE desktop colour changer",0

up_save:ds.l 1

I couldn't be bothered to 'rem' it, so here's the description:

First 5 lines: Enter supervisor mode (you have to be in supervisor mode

to be able to change the hardware palette)

Lines 7 to 10: Change the colours (here, they are black, white, blue,

and yellow)

Lines 11 to 14:Print out message (at end of code)

Lines 15 to 17:Pause a bit for the message

Lines 19 to 22:Return out of supervisor mode

Lines 24 to 26:Exit cleanly

Lines 27 to 28:Set up space for message and non-supervisor mode.

Any assembler at all  should  be  able  to  assemble  this code. Simply

change the colour data if required,  and assemble it. (Knowing you, you

will probably change the message, and take all of the credit...)

The file COLOURS.S and COLOURS.PRG are included somewhere on this disk.

The above program is for medium  resolution.  If you want to change the

16-colour low-resolution palette, shove these lines in after line 10:

move.w #$rgb,$ff8248

move.w #$rgb,$ff824A

move.w #$rgb,$ff824C

move.w #$rgb,$ff824E

move.w #$rgb,$ff8250

move.w #$rgb,$ff8252

move.w #$rgb,$ff8254

move.w #$rgb,$ff8256

move.w #$rgb,$ff8258

move.w #$rgb,$ff825A

move.w #$rgb,$ff825C

move.w #$rgb,$ff825E

Swap the 'rgb' for your usual hexadecimal colour values.

Article: BLACK EAGLE 1/7/93