PROGRAMMABLE SOUND GENERATOR REGISTER 6

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

As you may already know,  there  are  14  different PSG registers which

control the music or sound effects  that the YM2149 chip generates. One

use for three of these registers  is  for the ever-famous VU meters (8,

9, and 10). After a little bit  of research and experimentation, I have

come across something which may  interest  people who want different VU

meters.

The register that I am talking about  is number 6; the noise generator.

This is used by the  musicians  to  produce  the  drum effects in their

music.

By reading PSG(6) while music  is  playing,  you  are able to determine

what percussion instrument is playing at  any given time. First of all,

however, you need to find out which value each different drum has.

This program shows what I mean:

10 dreg(0)=1 : call 3 : loke $4d2,start(3)+8

15 rem                    Assumes you have MAD MAX music in bank 3

20 repeat

30 print PSG(6)

40 h=hardkey

50 until h=57

60 loke $4d2,0 : silence

This should produce a continuous list  of numbers, mostly 0's, but some

12's or 7's or 14's. These other numbers are printed whenever the noise

generator is called.

So, by watching which number  is  produced  whenever a drum sounds, you

can note it's value,  and  use  it  later  on. Unfortunately, different

pieces of music use different values  for  each  drum, so the values of

PSG(6) are unique to each demo.

Load up PSG6.BAS and you will see what can be done with this function.

Article: BLACK EAGLE 26/6/93