How to install FAST.COM or SPEED.COM on your system.

If you are using any of the following disk controllers,

then the .COM files will work as they are distributed.

Tarbell single density

Digital Systems or Digital Microsystems (single

density)

Intel MDS 800 or iSBC-201

If you are using any of the following disk controllers,

then you will have to insall the appropriate patch file

named below:

Thinkertoys Discus double density	DSCUSPAT.HEX

Digital Systems double density		DMSPATCH.HEX

If you are using something else with a physical sector size

other than 128, you will have to be a hacker and examine

the various patch files, FAST.ASM, and the skew tables in

your SYSGEN and BDOS.  Good luck.  If you manage to get a

good speed improvement on some controller not listed above,

please send a listing of the patch to the author:

Robert A. Van Valzah

1140 Hickory Trail

Downers Grove, IL    60515

Thank you.

If you have a single density controller other than those

listed above, then use the follwoing procedure:

You will need a stopwatch or a watch with a second hand.

Pick some operation that takes a fixed amount of time, like

PIPing a file around or assembling something.  Something that

takes about 30 to 60 seconds is good.  Execute it without

FAST, and record the execution time from when you hit return

to when you see the 'A>' again.  Now try the same thing

with FAST (see FMAN.PRN for instructions), and record the

execution time.  If it got faster (look for 20 - 30%) then

you are all set and can use FAST.COM and SPEED.COM as

supplied.

If it got slower or stayed the same, then you'll need

RSPEED.COM.  Load RSPEED and enter the number of sectors

per track (most likely 26).  Start with a skew factor of

1 and time how long it takes to read a track (the time from

when you hit return to when you get the prompt again).

Now increase the skew to 2 or 3 and time a track read

again.  Sooner or later, you should note a marked decrease in

the track read time.  The skew factor you are then using

is the fastest your controller can read an entire track.

Integrate the file SKEWnPAT.HEX into FAST.COM and SPEED.COM

where n is the skew obtained above.

How to integrate a patch file:

It's simple, all you need is  DDT.COM.

DDT FAST.COM       or for speed:  DDT SPEED.COM

IfooPAT.HEX

R

G0

SAVE 8 FASTX.COM   or for speed:  SAVE 17 SPEEDX.COM

And test the new file, it should be as fast as your

system can go.  Enjoy your trip!.