CompuServe

CSEXEC(TM)  3.0

Intelligent Terminal Emulator for

CP/M Systems

Copyright (c) 1983

CompuServe Incorporated

Page 1

1.0  INTRODUCTION

CSEXEC(TM)   for   CP/M   is   an   easy-to-use,    powerful

communications  program  written  for  CompuServe  by  Susan

Cavanaugh.   This  program  allows  your  CP/M   system   to

communicate  with the CompuServe Information Service as well

as any other host computer system with  telephone  answering

capabilities  utilizing  the  same communications standards.

This manual explains how to use  the  CSEXEC  software  with

examples   illustrating   its   use   on   CompuServe.   For

information  on  accessing  and  using  a  particular   host

computer, refer to the User's Guide for that system.

1.1  Hardware Required

CSEXEC requires a CP/M computer system with at least 32K  of

RAM,  one disk drive, a serial communications interface, and

an RS-232 modem.

2.0  CUSTOMIZING THE EXECUTIVE TO YOUR SYSTEM

Due to the variety  of  I/O  configurations  possible  among

computer  systems  running  under  CP/M,  you  will  need to

customize the Executive's I/O routines for  your  particular

system  configuration.   This  requires a basic knowledge of

programming in 8080 assembler language.

2.1  The Input/Output Overlay

Your CSEXEC diskette  contains  a  file  called  OVERLAY.ASM

which  contains  assembler  source  code  of  the serial I/O

routines which service the modem.  It  is  to  be  used  for

customizing   the   executive   to   different   serial  I/O

configurations.  OVERLAY.ASM contains three subroutines:

SIOINI  - initializes the UART.

SIOGET  - checks modem receiver and returns with:

Z = 1 if no data ready.

Z = 0 if data was read, data is in

the accumulator.

SIOPUT  - stores the contents of the accumulator in

the UART's transmitter.  It waits until

the transmitter accepts data before

returning.

Page 2

Modify OVERLAY.ASM according to your own communications UART

specifications.  For instance, it may be necessary to modify

the initialization routine to specify the baud rate and  you

may  need  to  change  the sense of the receive and transmit

flags in the get and put routines.

Timing considerations require that the console  and  listing

devices  be  capable of handling data at a speed which is at

least TWICE the baud setting of the modem serial  port.   If

this  is not the case for your computer, it is probable that

incoming data will be lost.

The necessary specifications are:

8 data bits

no parity

1 stop bit

Included in  OVERLAY.ASM  are  the  program  stack  and  the

compressed ascii dictionary.  These don't need to be changed

for your system.  They are included in the  overlay  because

they must follow the I/O routines in memory.

2.2  Creating CSEXEC For Your System

After  you  have  made  all   the   necessary   changes   to

OVERLAY.ASM, assemble it.

Then, type the following at the CP/M prompt:

DDT EXECOR.COM <cr>

IOVERLAY.HEX <cr>

R <cr>

This will overlay your I/O routines with  the  generic  CP/M

Executive,  EXECOR.COM,  in  your  computer's RAM.  Now type

CTRL-C to exit DDT and return to CP/M.

To save  the  completed  CSEXEC  for  your  system  on  your

diskette, type:

SAVE 24 CSEXEC.COM <cr>

Note that if you have supplied more than two pages  of  code

in OVERLAY.ASM, you will need to save more than 24 pages.

Page 3

3.0  USING A PRINTER WITH CSEXEC

If you have a printer connected to your  computer,  you  can

use it to obtain hardcopy with CSEXEC.

3.1  Copying Data To The Printer

If you type  a  CTRL-R  on  the  keyboard,  a  copy  of  all

subsequent  characters received will be sent to the printer.

A CTRL-T stops the copying.  When a CTRL-R (ASCII  character

DC2)  is  received from the host, the copying to the printer

will  start.   When  a  CTRL-T  (ASCII  character  DC4)   is

received,  the  copying  is  stopped.  If the printer is not

ready, an error message will be displayed.

4.0  RUNNING CSEXEC

To load and  execute  CSEXEC,  type  "CSEXEC"  at  the  CP/M

prompt.

5.0  EXITING CSEXEC

CTRL-X will display the menu, at which time, typing  an  "X"

(for  eXit) will exit CSEXEC and return control to CP/M.  To

continue the terminal session, reload and execute CSEXEC.

Page 4

6.0  AUTO LOGON

Auto logon allows you to keep  logon  information  for  many

different systems.  It also has the capacity to act in place

of the user for a number of applications.

6.1  Auto Logon Control

The auto logon menu allows you to control all available auto

logon  operations.  To display the menu, type CTRL-X.  While

looking at the menu, press the letter  that  corresponds  to

the operation you want.  The figure below shows the menu.

Auto Logon Functions

C  Create Auto Logon File

E  Execute Auto Logon File

A  Abort Auto Logon

T  Transmit CTRL-X

X  Exit program

Press your choice or <RETURN> for terminal mode.

To return to terminal mode, press  <RETURN>.   Type  "X"  to

exit the program and return to CP/M.

6.2  Creating A New Auto Logon File

In order to create a new auto logon file on  your  diskette,

follow these steps:

1. Type CTRL-X to display the auto logon  menu.   Press  "C"

(for  Create)  and you will then be prompted for the auto

logon file name you wish to use.  If you  press  <RETURN>

with no file name, the program will assign a default file

name of CSEXEC.ATO.  To abort auto logon  file  creation,

type a CTRL-C at any time during the creation process and

the file will be erased from your diskette and  the  menu

redisplayed.

2. You will then be asked for a prompt to wait for from  the

host.   If  you  do  not  want to wait for a prompt, just

press the <RETURN> or <ESC>  key.   Otherwise  enter  the

host's  prompt.   The  prompt  sequence  is translated to

upper case before it is stored  on  disk.   All  received

characters are translated to upper case for matching with

the prompt.  The maximum length of a prompt from the host

is  20  characters.   A  prompt  length  of  four to five

characters or less is recommended due to the  possibility

of  noise occurring over the phone lines and garbling the

incoming prompt.  You may press the <RETURN> or <ESC> key

Page 5

to  terminate  the  prompt.   Neither  terminator will be

included in the prompt to be searched for.

3. You will then be asked for a response to transmit.  Enter

your  response  terminated  by  <RETURN>  or  <ESC>.  The

carriage return will be transmitted  with  the  response.

The  <ESC>  will transmit no terminator when the response

is sent.  If you enter just an <ESC> for  a  response,  a

null  response  is  indicated.   The maximum length for a

response is 64 characters.  A response of  more  than  64

characters can be transmitted by entering the response in

two or more parts, each less than  64  characters,  while

entering a null prompt between each part of the response.

4. Repeat the prompt/response input sequence as  many  times

as  is  necessary  to complete your logon sequence.  When

you are done, type  an  <ESC>  at  the  next  prompt  and

response  request.  The auto logon menu will reappear and

allow you to specify which action to take next.

6.3  Executing An Auto Logon

To execute an auto logon file, press CTRL-X to  display  the

menu, then "E" (for Execute).  You will then be prompted for

the auto logon file name to read  from  disk.   The  default

file  name  CSEXEC.ATO will be used if you press <RETURN> at

the file name prompt.  If the file exists on your  diskette,

CSEXEC will enter auto logon mode.  To abort execution of an

auto logon sequence, press CTRL-X to  return  to  the  menu,

then  type  "A"  (for  Abort).   When  using  auto logon for

logging on to a system, you should  wait  until  the  remote

system  answers  the phone to begin the auto logon sequence.

CSEXEC will automatically exit  auto  logon  mode  when  all

prompts have been found and all responses sent.

The keyboard remains active during an auto logon  execution.

This  allows you to type during auto logon without affecting

the execution of the auto logon file.

6.4  Additional Auto Logon Features

Auto logon can do more than simply log on to a host  system.

The  prompt-seeking  function  allows  CSEXEC to act in your

place on the keyboard  to  provide  a  response  to  a  host

program.   A  prompt  should  always  be  specified  after a

response that will cause the host to display text.   Prompts

can  be  any  text  that the host displays.  Typical prompts

are:  "!", ":", and "OK".  The response from auto logon  can

be  any  text  or  control  characters.   To enter a control

character into the response, type a carat (^) then type  the

letter of the control key.

Page 6

To  obtain  a   delay   of   one-half   second,   enter   ^&

(carat-ampersand)  into  the  response.   For a delay of two

seconds, enter ^% (carat-percent) into the response.

A single carat mark is entered into a response  by  entering

two consecutive carats.

6.5  Auto Logon File Chaining

The execution of a new auto logon file may be  started  from

an  already-executing  auto logon file.  This feature allows

you to create an auto logon sequence in multiple files.   To

change a part of the sequence, re-create the file containing

that part.  The other files do not need  to  be  re-created.

When  the  CTRL-X  is  executed from an auto logon file, the

current file is  closed  and  the  file  specified  is  then

executed.   The  contents  of  the auto logon file after the

"CTRL-XEfilename<RETURN>" is ignored.

6.6  Auto Logon Examples

A  sample  auto  logon  creation  sequence  to  log  on   to

CompuServe is shown below.

Creating New Auto Logon

Enter file name: cpusrv.ato       Select the file

Prompt: <ESC>                     Don't wait for prompt,

transmit immediately

Response: ^C<ESC>                 (<Carat><C>)

Transmit a CTRL-C

with no carriage return

Prompt: ID:                       Wait for 'USER ID:'

Response: 70000,1                 Then send '70000,1'

Prompt: word:                     Wait for 'PASSWORD:'

Response: SECRET                  Then send 'SECRET'

Prompt: !                         Wait until DISPLA

prompts for input

Response: GO HOM-1                Transmit 'GO HOM-1'

Prompt: <ESC>                     Terminate the input

Response: <ESC>                   sequence by entering

null prompt and null

response

Page 7

A  sample  auto  logon  creation  sequence  to  log  on   to

CompuServe through TYMNET is shown below.

Creating New Auto Logon

Enter file name: tymnet.ato       Select the file

Prompt: FIER                      Wait for 'PLEASE

TYPE YOUR TERMINAL

IDENTIFIER' from

TYMNET

Response: A<ESC>                  Transmit an 'A'

without a carriage

return

Prompt: IN:                       Wait for TYMNET login

message

Response: CIS02                   Request connection

to CompuServe

Prompt: ID:                       Wait for 'USER ID:'

Response: 70000,1                 Then send '70000,1'

Prompt: word:                     Wait for 'PASSWORD:'

Response: SECRET                  Then send 'SECRET'

Prompt: !                         Wait until DISPLA

prompts for input

Response: GO HOM-1                Transmit 'GO HOM-1'

Prompt: <ESC>                     Terminate the input

Response: <ESC>                   sequence by entering

null prompt and null

response

Page 8

The following example shows how the autolog feature could be

used  to  automatically  dial and login with the D. C. Hayes

Smartmodem.  This example programs the  Smartmodem  to  dial

'555-1234'  using  tone  dialing.   Refer  to the Smartmodem

manual for instructions on creating a programming string.

Creating New Auto Logon

Enter file name: smart.ato        Select the file

Prompt: <ESC>                     Don't wait for prompt,

transmit immediately

Response: AT DT555-1234           Program the Smartmodem

to dial 555-1234

using tone dialing

Prompt: CONNECT                   Wait for connection

Response: ^%^C<ESC>               Delay for two seconds

and transmit a

CTRL-C with no

carriage return

Prompt: ID:                       Wait for 'USER ID:'

Response: 70000,1                 Then send '70000,1'

Prompt: word:                     Wait for 'PASSWORD:'

Response: SECRET                  Then send 'SECRET'

Prompt: !                         Wait until DISPLA

prompts for input

Response: GO HOM-1                Transmit 'GO HOM-1'

Prompt: <ESC>                     Terminate the input

Response: <ESC>                   sequence by entering

null prompt and null

response

Page 9

The following auto logon could be used  to  get  to  the  CB

simulator  program  from  most any other menu on CompuServe.

HOM-21 is used for illustration only  and  may  not  be  the

actual page number of CB on the system.

Creating New Auto Logon

Enter file name: cb.ato           Select the file

Prompt: <ESC>                     Don't wait for prompt

Response: T                       Send T for TOP

Prompt: !                         Wait until top menu

is displayed

Response: GO HOM-21               Transmit 'GO HOM-21'

Prompt: ?                         Wait until CB prompts

for handle

Response: (Your handle)           Transmit handle

Prompt: <ESC>                     Terminate the input

Response: <ESC>                   sequence by entering

null prompt and null

response

The following shows how the auto logon might execute.  It is

initiated  by  typing  a CTRL-X and responding with 'E' (for

Execute) at the menu, then typing 'cb.ato' at the file  name

prompt.

T

CompuServe            Page CIS-1

CompuServe Information Service

1 Home Services

2 Business & Financial

3 Personal Computing

4 Services for Professionals

5 User Information

6 Index

Enter your selection number,

or H for more information.

!GO HOM-21

CompuServe           Page HOM-21

Request Recorded,

One Moment, Please

Thank You for Waiting

CB Simulator Ver 3(45)

What's your handle? (Your handle)

(Channel) users tuned in

Which channel: 1

(1,Susana) Hi, CB'er

Page 10

7.0  FILE TRANSFER USING CSEXEC

Most of CompuServe's CSEXEC programs contain the ability  to

transfer a file from the microcomputer's disk to CompuServe.

While this  is  a  feature  found  in  many  other  terminal

programs,  most  of them merely transmit the contents of the

file or write exactly what  is  received.   CSEXEC  uses  an

error-free communications protocol to perform file transfer.

This means that if noise occurs on the communications  line,

the  error  will  be  detected  and  the  data block will be

retransmitted.

The procedure for performing a file transfer  on  CompuServe

is  relatively  simple  and is described below.  A few terms

must first be defined in order to  explain  file  transfers.

The  process  of  transferring  a  file from CompuServe to a

microcomputer  is  called  DOWNLOADing.   The   process   of

transferring  a  file  from a microcomputer to CompuServe is

called UPLOADing.   The  CompuServe  mainframe  computer  is

referred   to   as   the   HOST  computer  system  and  your

microcomputer is referred to as the REMOTE computer.

There are three types of file transfers:  text, binary,  and

machine specific.

A text transfer is used to transfer ASCII text files such as

untokenized  BASIC  programs and text files produced by word

processing programs.

A binary transfer is used to transfer eight-bit  files  such

as  tokenized  BASIC programs and machine language programs.

This can also be used to transfer most microcomputer's BASIC

data  files.  CSEXEC does not alter any data during a binary

file transfer.

A   machine-specific   transfer   is   used   to    transfer

machine-dependent files.  CSEXEC inserts all the information

it needs to recreate  the  file  exactly  as  it  originally

existed.    If   an   attempt   is   made   to   download  a

machine-specific file to  a  microcomputer  which  is  of  a

different  type  than  the microcomputer from which the file

was  uploaded,  CSEXEC  will  issue   a   warning   message.

Machine-specific  transfers  can  be  used to transfer files

that have information kept outside of the file  (usually  in

the  disk  directory) that is essential to the file (such as

file type, record length).

For both an upload  and  a  download,  the  CompuServe  file

extension  determines  the  type  of  transfer  that will be

performed.  The valid extensions and the transfer types are:

BIN     Binary transfer

CMP     Reserved for CompuServe

IMG     Machine-specific (image) transfer

TXT     Text transfer

Page 11

If any other extension or no extension at  all  is  used,  a

text  transfer  will  be  performed.   To  initiate  a  file

transfer, at the 'OK' prompt, enter:

R FILTRN

CompuServe will respond with:

CompuServe File Transfer Program

Your machine will then be interrogated to  see  if  you  are

running  a  version of CSEXEC which supports file transfers.

If you are not running a CSEXEC program or if  your  version

of  CSEXEC does not support file transfers, an error message

will be printed and you will be returned to the 'OK' prompt.

If  you  are running a version of CSEXEC which supports file

transfer, you  will  be  asked  for  the  direction  of  the

transfer:

Select direction-

D if to your computer

U if to CompuServe

:

Enter a 'U' to transfer a file from  your  microcomputer  to

CompuServe  (upload)  or  a  'D'  to  transfer  a  file from

CompuServe to  your  microcomputer  (download)  at  the  ':'

prompt.

You will then be prompted with:

Enter the CompuServe file name:

Enter an appropriate file name and extension.  Remember that

the   file   extension   determines  the  type  of  transfer

performed.  For a download this is the name of the  existing

file to be transferred to your microcomputer.  For an upload

this is the name of the file which will be created  in  your

Programming Area.  You will then be prompted with:

Enter a file name for your computer:

You may specify any file following the naming rules of CP/M.

This   might  include  a  drive  designation,  a  file  name

extension, or punctuation.  The remote file  name  does  not

affect the type of transfer performed.  For an upload, enter

the name of the file to be transferred to CompuServe.  For a

download,  enter  the  name of the new file to be created on

your diskette.  If the file already exists on your diskette,

you  will  be asked if you would like it to be replaced.  If

so, CSEXEC will erase the existing file and download the new

contents.  If not, the file transfer will be aborted and you

will be returned to the 'OK' prompt.  The original file will

not  be erased from the diskette.  As a transfer proceeds, a

series of digits will be displayed on your screen,  one  for

Page 12

each  block  of data.  For each 32 bytes of data transferred

within each block, a plus sign (+) will be displayed between

these digits.

When the transfer is completed, the message:

*** File Transfer Completed!  ***

will appear and you will be returned to the 'OK' prompt.

To signal CSEXEC to abort a file transfer, hold CTRL-X until

the next digit appears on the screen.

8.0  TECHNICAL INFORMATION

The  following  information  is  provided  for   determining

compatibility with hosts other than the CompuServe network.

8.1  Flow Control

CSEXEC  recognizes   the   standard   ASCII   flow   control

characters,  <X-OFF>  (CTRL-S  or DC3) and <X-ON> (CTRL-Q or

DC1).  If CSEXEC receives an <X-OFF> from the host, it  will

halt   transmission.   If  an  <X-ON>  is  not  received  in

approximately five seconds, transmission will be resumed.

CSEXEC expects the host  to  observe  the  same  rules.   An

<X-OFF> is sent to stop transmission from the host while the

menu is being used.  An <X-ON> is sent when transmission can

be  resumed.   If the host does not respond to flow control,

characters may be dropped.

8.2  ESCAPE Sequences

CSEXEC responds to certain ESCAPE  sequences  when  received

from the host computer.  These sequences cannot be performed

locally;  they must be received from the host.

8.2.1  Keyboard Lock And Unlock

When an <ESC><b> sequence is received, CSEXEC will lock  the

keyboard  and  not transmit any more characters typed.  This

condition  is  reset  when  CSEXEC  receives   an   <ESC><c>

sequence.   These  sequences  can  be used by a host program

which does  not  want  to  be  interrupted  during  critical

processing.

Page 13

8.2.2  Video Enable And Disable

When an <ESC><e> sequence is received, the video is disabled

and  subsequent  characters are not displayed.  The video is

enabled again when an <ESC><f> sequence is received.

8.3  The Protocol

CSEXEC contains a data communications protocol which  allows

error-free   data   transfer  between  CompuServe  and  your

computer.  The details of this protocol are  copyrighted  by

CompuServe,  Inc.  and can only be disclosed under a special

license (not included with this software).

8.4  CompuServe Control Characters

This  section  is  provided  to  demonstrate   how   control

characters  are  used  on a host computer.  In the following

table, "^x" means to hold the the CTRL  key  and  press  the

letter "x".

Transmitted Control Characters

^A   Suspends output being displayed by  CompuServe  at

the end of the current line.  Repeated issuance of

^A  causes  successive  lines  of  output  to   be

displayed,  one line at a time.  Running output is

turned back on with a ^Q.  ^A is not typed out.

^S   Suspends output immediately.

^Q   Restores the running  display  of  output  at  the

point where it had been interrupted by a ^A or ^S.

^C   Interrupts display or  a  program's  execution  so

that  you  can  enter  another  menu  selection or

command.

^B   Used as a type ahead ^C, forcing an interrupt when

CompuServe requests input.

Page 14

^P   Interrupt display or a program's execution so that

you can enter another command to the program.

^H   Backspaces, deleting the character that was there.

Same as BACKSPACE key.

^U   Deletes the line currently being typed.

^V   View current line;  redisplays the  line  you  are

currently typing.

^O   Tells host to  turn  off  output  currently  being

displayed.

8.5  The ASCII Character Set

0   1   2   3   4   5   6   7

0  NUL DLE SPC  0   @   P   `   p

1  SOH DC1  !   1   A   Q   a   q

2  STX DC2  "   2   B   R   b   r

3  ETX DC3  #   3   C   S   c   s

4  EOT DC4  $   4   D   T   d   t

5  ENQ NAK  %   5   E   U   e   u

6  ACK SYN  &   6   F   V   f   v

7  BEL ETB  '   7   G   W   g   w

8  BS  CAN  (   8   H   X   h   x

9  HT  EM   )   9   I   Y   i   y

A  LF  SUB  *   :   J   Z   j   z

B  VT  ESC  +   ;   K   [   k   {

C  FF  FS   ,   <   L   \   l   |

D  CR  GS   -   =   M   ]   m   }

E  SO  RS   .   >   N   ^   n   ~

F  SI  US   /   ?   O   _   o

9.0  CSEXEC ERROR MESSAGES

Typical Errors:

% VTXFAE - That file already exists on your disk.

Do you wish to replace it (Y or N)?

The file you specified for your system to download

to  is  already  on your diskette.  If you want to

overwrite it, reply "Y";  if not,  reply  "N"  and

the file transfer will be aborted.

Page 15

% VTXPNR - Printer not ready!

The printer is not ready.   Make  sure  the  power

switch is on and the printer is online.

? VTXDKF - Diskette is full!

The diskette has run out of storage  space  during

file   transfer.    CSEXEC  erases  the  partially

downloaded file from the diskette and  aborts  the

file transfer.

? VTXNEF - That file is not on your diskette!

The file you specified  for  upload  to  the  host

doesn't exist on your diskette.  The file transfer

is aborted.

% VTXFNC - This file may not be  compatible  with  your

computer.

Do you still want to receive it?  (Y or N)

This message signifies that you are attempting  to

download  an  image file which was uploaded from a

different  microcomputer  than   yours.    A   "Y"

response  will cause the host to download the file

anyway,  and  an  "N"  response  will  abort   the

transfer.

Page 16

10.0  GLOSSARY

This glossary defines some of the more frequently used terms

associated with the CompuServe Information Service.

database

is a collection of information which is stored  on  the

computer.

defaults

are settings telling your  terminal  specific  ways  to

respond.   For  example, how many lines on a screen you

want to have displayed while scrolling, the first  page

to  see  after logon, the number of characters to delay

after issuing a command, etc.   These  default  setting

can  be  made  permanent  or  can  be in effect for the

current session only.

disk

storage refers to the storing or saving of data in  the

Programming Area on a device called disk.  The data can

then be retrieved for use.   Each  Standard  and  Prime

Service  customer  is  allocated  128,000 characters of

disk space at no extra charge.  Additional storage  can

be requested through Feedback for an extra charge.

DISPLAY

is the program which accesses the  Videotex  Area  from

the Programming Area.

download

is the process of transferring a file of data from your

on-line  disk  storage  to  your  own personal computer

system.

EMAIL

is the CompuServe Information Service  Electronic  Mail

program.

ENTER

is the specific key on the keyboard  which  enters  the

current  line  into the computer;  acts like a carriage

return on a typewriter.

Executive

See CSEXEC.

Page 17

FEEDBK

is the  program  which  enables  you  to  provide  your

comments, ask questions or get help with a problem from

Customer Service.  You may also order documentation and

CompuServe  Information  Service  products while in the

Feedback program.

file

is a collection of data, uniquely identified by a  name

up  to 6 characters long and optionally an extension of

up to 3 characters (for example, TEST.DAT).

Filge

is CompuServe's File Generator and Editor which is used

for creating and modifying files, for use in EMAIL, the

Bulletin Board, and the Programming area files.

hardcopy

is a printout or a paper copy of computer data.

host

is the CompuServe computer to which you are connected.

initial page

is the first page of  information  which  is  displayed

after logon.

job

is a user's individual session while  logged  onto  the

CompuServe computer.

logoff

is the sequence of events which  disconnects  you  from

the CompuServe computer.

logon

is the sequence of events which connects you  with  the

CompuServe computer.

menu

refers to the item choices from which you may select  a

page to be displayed in the Videotex Area.

Page 18

network

is the communications link equipment that  enables  you

to connect to CompuServe's computers in Columbus, Ohio.

CompuServe operates  its  own  network  in  most  major

cities in the 48 contiguous United States.  Access from

other cities in  the  U.S.   and  Canada  is  available

through  a  supplemental  network  (such  as  TYMNET or

DataPac) at an additional communications surcharge.

node

is a specialized communications computer  which  allows

many  terminals to communicate through the same line to

CompuServe's large computer complex in Columbus, Ohio.

OK

is the prompt which is used in the Programming Area  to

indicate readiness to accept a command.

page

is any one of the 16-line displays which appear on your

terminal.

password

is a unique set of characters that secures  entry  into

the  service  for  a  unique User ID.  Your password is

your key to safeguarding your data and  usage  charges.

You are responsible for all usage of your User ID.  The

CompuServe Information Service strongly recommends that

you  keep your password in a secure place separate from

your User ID and never give it to anyone,  verbally  or

through the service.  The most secure password consists

of two non-related words connected with a symbol.

program

is a set of machine  instructions  which  the  computer

uses  to  perform  a  defined  function.  References to

particular programs used on the CompuServe  Information

Service  are  made  by  program  name,  such  as EMAIL,

FEEDBK, DISPLAY.  When  a  program  is  requested,  the

computer loads and executes (runs) the specified set of

instructions (program).

prompt

is the message which is displayed to alert you that the

computer  is waiting for input.  For example, Key M for

menu!  in the Videotex Area, or OK in  the  Programming

Area.

Screen

refers to one page of information.

SIGS

is the program  which  accesses  the  Special  Interest

Groups  area.   The  SIGS offering contains many groups

with mutual special interests which have been  given  a

unique  area  to allow members to communicate with each

Page 19

other.

SOFTEX

is the name for the  Software  Exchange  program  which

allows  a  user who is equipped with the proper version

of the Executive to purchase programs on-line that  can

be  run  on  his/her personal computer.  These programs

can be downloaded to the personal computer system  upon

purchase.

terminal

is a keyboard and printing or display mechanism used to

enter data into a computer and to display output from a

computer.  A microcomputer which  is  running  terminal

emulation software is considered to be a terminal.

upload

is  the  process  of  transferring  a  file  from  your

personal   computer  system  to  your  CompuServe  disk

storage area.

User ID

is a unique number assigned to your account that,  when

used  with  the  correct  password,  allows  access  to

CompuServe   Information   Service.     For    example,

77777,777.

Videotex

is an easy to use interactive menu formatted system for

accessing remote databases or programs.  On CompuServe,

the Videotex format is in pages of text.

CSEXEC(TM)

is a terminal emulator program  which  is  specifically

designed   for  use  with  the  CompuServe  Information

Service and which is in the CompuServe Videotex format.

CSEXEC(TM)  software  is  sold through computer dealers

and through the CompuServe Information Service  on-line

product ordering area found in Feedback.

Table of Contents

Introduction  . . . . . . . . . . . . . . . . . . . . . . . 1

Hardware Required  . . . . . . . . . . . . . . . . . . . 1

Customizing the Executive to Your System  . . . . . . . . . 1

The Input/Output Overlay . . . . . . . . . . . . . . . . 1

Creating CSEXEC for Your System  . . . . . . . . . . . . 2

Future Versions of the Executive . . . . . . . . . . . . 3

Using A Printer With CSEXEC . . . . . . . . . . . . . . . . 3

Copying Data to the Printer  . . . . . . . . . . . . . . 3

Running CSEXEC  . . . . . . . . . . . . . . . . . . . . . . 3

Exiting CSEXEC  . . . . . . . . . . . . . . . . . . . . . . 3

Auto Logon  . . . . . . . . . . . . . . . . . . . . . . . . 4

Auto Logon Control . . . . . . . . . . . . . . . . . . . 4

Creating Auto Logon File . . . . . . . . . . . . . . . . 4

Executing An Auto Logon  . . . . . . . . . . . . . . . . 5

Additional Auto Logon Features . . . . . . . . . . . . . 5

Auto Logon File Chaining . . . . . . . . . . . . . . . . 6

Auto Logon Examples  . . . . . . . . . . . . . . . . . . 6

File Transfer Using CSEXEC . . . . . . . . . . . . . . . . 10

Technical Information  . . . . . . . . . . . . . . . . . . 12

Flow Control  . . . . . . . . . . . . . . . . . . . . . 12

ESCAPE Sequences  . . . . . . . . . . . . . . . . . . . 12

Keyboard Lock and Unlock . . . . . . . . . . . . . . 12

Video Enable and Disable . . . . . . . . . . . . . . 13

The Protocol  . . . . . . . . . . . . . . . . . . . . . 13

CompuServe Control Characters . . . . . . . . . . . . . 13

The ASCII Character Set . . . . . . . . . . . . . . . . 14

CSEXEC Error Messages  . . . . . . . . . . . . . . . . . . 14

Glossary . . . . . . . . . . . . . . . . . . . . . . . . . 16

##CSEXEC Er