EA

July 3, 1983

SUBJECT: Installing ZCPR2 on the KAYPRO II

1.0     Purpose of this Memorandum

This  memorandum is intended to explain to a CP/M novice how

to install a legal[1] copy of ZCPR2[2] on his KAYPRO II.  If

these instructions are insultingly  basic  for  you,  please

forgive.

I have to the degree  possible  automated  the  installation

process.   But  the  first  and  the  last  steps  cannot be

automated, so they are described in detail herein.

2.0     Files You Will Need.

On your system disk that came with your KAYPRO  you  already

have  some  of the files that you need.  The remaining files

that you need are in this package.

_________________________

[2]  By  *legal*  I  mean  doing it in a way that does not offend

Digital Research.  One can do anything one wants  to  do  to

one's  own  copy  of  CP/M,  *except* to sell it (or give it

away) to others.  It is always useful to remain  legal  when

it its possible to do so and still get the job done -- hence

this instruction for a legal approach to installing ZCPR2 on

the KAYPRO II instead of distributing a COM file.

[2]  There  are  an  incredible number of different ways in which

ZCPR2 can  be  installed.   The  version  provided  in  this

package  includes  all  options  **except** I/O redirection,

named directories, and password protection.

- 2 -                  ZCPR2 for KAYPRO II

3.0     Getting a Disk Ready for the Task.

3.1  To begin the process:

.  FORMAT a fresh disk in Drive B:

.  SYSGEN your current system to that disk.[3]

3.2  PIP the following files to that disk:

.  SUBMIT.COM  (from your system disk)

.  XSUB.COM                "

.  DDT.COM                 "

.  SYSGEN.COM              "

.  MOVCPM.COM              "

.  The two HEX files, and the SUB file, from this package.

4.0     Step Number One.

4.1  First  you must create a 63K version of your CP/M system.[4]

That is necessary because to take advantage of ZCPR2's  fine

features  you  need  a  little  more  space  than NON-LINEAR

SYSTEMS gave you in the KAYPRO II BIOS.  You won't miss  (or

even notice) that extra 1k, so don't worry about it.

4.2  Type  (without  the quotation marks) "MOVCPM 63 *", and do a

carriage return.  You will then see the  following  on  your

screen:

CONSTRUCTING 63k CP/M vers 2.2

READY FOR "SYSGEN" OR

"SAVE 34 CPM63.COM"

A>

_________________________

[3]  If you have set up your Number Pad  so  as  to  replace  the

1,2,3,4,etc.  with other characters, makes sure that you put

that   system   on   the  disk.   Changing  the  Number  Pad

subsequently is not so easy (though it can  be  done).   The

CONFIG.COM  program  will not longer do that, inasmuch as it

expects the table of data to be  in  a  place  where  it  no

longer is.

- 3 -                  ZCPR2 for KAYPRO II

4.3  When  the  prompt  (i.e.,  the  >A) comes back, type SAVE 34

CPM63.COM and do a <cr>.  That will create a file  with  the

name  CPM63.COM  on  your  disk.  We'll need the file in the

automated sequence that follows.

5.0     The Automated Portion of the Process.

5.1  Type the following line at the prompt, and then do a <cr>:

SUBMIT ZCPR2

If you want to know what is going to happen, study the  file

ZCPR2.SUB  that  you got from the library file.  When you do

SUBMIT ZCPR2, you in effect tell the computer to perform the

operations listed in ZCPR2.SUB in sequence.

6.0     Finishing the Job.

6.1  When the computer has typed the following message:

;I am now ready for the Last Step

type the following line at the prompt, and then do a <cr>:

SYSGEN ZCPR2-KP.COM

When you are asked for DESTINATION, respond with an  A,  and

when you are asked again, respond with a <cr>.

7.0     The Grand Finale.

Now  comes  the  big moment.  Do a cold boot, and watch what

happens.  If everything worked OK, you'll  see  a  different

log-on  message  and  will  see a different prompt.  If not,

something got screwed up and you'll have to track  it  down.

Just  re-SYSGEN  the  disk  in  Drive  A:  with your current

system, and start figuring our the problem.

If it's working OK, SYSGEN to your  other  disks.   As  Rick

Conn (ZCPR2's principal author) is fond of saying at the end

of his messages, 'Enjoy'.

EOS

####

#####################################