PADS-LOGIC Version 3.0 ADVISORIES

PART TYPES

When changing a part type to a different part type, The reference

designation will not be affected. For example, if a resistor part type

were changed to an I.C. part type (an R1/4W for a 7404), the reference

designation would remain as it was assigned to the R1/4W part type. In

this example, the user must manually update the reference designation

after the part type has been changed.

NETLIST

When a user specifies a Reports, Netlist out, using PADS-PCB, the user

should note that the PCB decal defined in this ASCII file identifies a

specific decal which appears in his/her PCB database.  This new version is

enhanced to allow all the alternates to be available in Pads-Pcb providing

the alternate decal being called out is one of the alternates presently

defined in the part type used to bring the part in.  If the decal is

completely independent of the part type being used, then only that specific

decal will be available in the board design.

REPLACE PART

Attempting to replace a part in a schematic which has less pins than the

original part results in the prompt:  "NEW PART HAS LESS PINS THAN ORIGINAL

PART", and the part will not be replaced.  This is an error checking

function that is performing correctly.  You cannot change a part with fewer

pins because connections would be lost. Always replace with a part

containing the same number of pins or more pins.  If not possible, the

original part must be deleted and the new part (with fewer pins) put in and

reconnected.

DUPLICATE PIN NAMES

Unable to have duplicate pin names in LOGIC decal!  Names such as NC or N/C

must be different, ie: NC1, NC2, etc.. This is functioning correctly and

will not be changed.  If desired, add the names as free text in the Mod Gate

routine.

REFERENCE DESIGNATORS

Pure alpha reference designator names are not supported.

BUS NAMES

Bus names with alphanumeric characters are not supported, ie: DA1[00:13],

only names such as  DA[00:13]

DISK SPACE

Please be advised that on systems with low amounts of hard disk space, a

message appears indicating your disk space is low, and the backup files

cannot be created without clearing up more disk space.  If the message goes

unheeded, the system in some cases will attempt to try backing up the

schematics and get caught in a never ending loop. A reboot of the system is

the only way out of the software.

CAE DECAL GENERATION

Many users are modifying existing parttypes to make other parts.  While

this is a valid procedure, please be aware that if you make changes to a

gate decal at this level (physical changes rather than text modifications),

any other parttype that utilizes this gate decal in its configuration, will

also assume these physical changes.  The correct way to implement these

changes is:

1)      To use an existing gate decal as a base for generating another, call

up gate "A" under the "GATE DECAL" command in the library menus,

produce all the modifications necessary at this point, and save it

away as gate"B".  This way gate "A" is preserved in its original form

and gate"B" has now been created.

2)      To generate a new gate decal at the parttype stage, you simply define

the name in the electrical section.  When you complete (F9) the

parttype, the system alerts you that your gate decal does not

exist.  At this time, select "MOD GATE" and generate your gate decal.

Once you have finished defining the gate, select complete (F9), and

save the part away in the normal fashion.

OFFPAGE

If a schematic is brought to another design station and a GROUP/CUT is

performed which contained offpage symbols, when that cut is re-added to a

new sheet, the offpage symbol may be added using the wrong symbol.  This is

caused by the differences between the offpage parts which can vary from

system to system.  We recommend modifying the offpage part on system two to

match that of system one, then paste the group back in.

HARDWARE NOTES

This version is 32-bit only and is not compatible with 80286 or XT computers.

FOR PADS-PCB USERS

If you plan to use the PADS-LOGIC 32-bit option with PADS-PCB, take care

not to exceed the limits of PADS-PCB. The 16-bit option of PADS-LOGIC will

not allow you to create a schematic that exceeds the maximum design size of

PADS-PCB.

NET NAMES

PADS-LOGIC 32-bit supports 47 character net names. PADS-PCB only supports

12 character net names. Problems may occur if you are using PADS-PCB and

exceed the 12 character limitation. PADS-2000 v3.x does support 47 character

net names.

BUS NAMES

PADS-LOGIC 32-bit supports BUS NAMES up to 44 characters.

SIGNAL PINS

Do not exceed more than 20 characters in the EDIT SIGPIN routine, problems

may result.

PRINTING REPORTS

To directly print out report files to your printer, type PRN <CR>.

If, after reviewing this document and consulting your PADS-LOGIC manual,

you would like additional information, then contact our Technical Support

Staff at (508) 486-3328 or 1-800-423-PADS.

7/92

*****   END OF LOGIC.DOC FILE   *****