November 24, 1994

INTRODUCTION

ME is a program designed to help manufacturing engineers,

estimators, CNC programmers, inspectors, machinists, and other

technical personnel in the metalworking industry.  I have worked in

these fields for the past eighteen years, and a significant portion of

my working time has been spent searching through reference books

for data and making manual calculations based on those data.  After

reinventing the wheel countless times in this fashion, I decided to

make my life easier by creating a program which would give me

instant access to much of the information I needed for my work.  I

have used ME on a daily basis for the past six months - it performs

as intended and helps me in many ways.  I have decided to share

this program with the manufacturing community and perhaps

expand it significantly if the feedback is encouraging.

ME is a DOS program which will run on virtually any MS-DOS

computer in existence.  It will run, with acceptable speed, on a

384K 8088 with monochrome monitor and one floppy drive.  I like

to run it as a windowed DOS application in Windows, where I can

access it from inside other programs.  I find that 285K of memory

are enough in the .PIF file.  ME is a plain program, menu-driven

and friendly, but the emphasis is on function and not glamour.  It

requires a very short learning curve, and will be useful to

metalworking personnel of all experience levels.

OVERVIEW OF FEATURES

THREADS

complete dimensional data for nearly two hundred of the most

commonly used threads, including UNC, UNF, UNEF, UNJ, NPT,

ACME, and ISO

standard and close-fit thread classes

wire sizes and dimensions for measurement

suggested tap drill and tap class

root radius limits (UNJ)

minor diameters required for various percentages of thread

engagement

inch or metric display

MATERIAL WEIGHTS

twenty one different materials

shapes include hexagonal, octagonal, rectangular, square, round,

and tubular

inch or metric dimensional input

output in pounds and kilograms

MATERIAL HARDNESS

Rc, Brinell, and PSI comparisons

DRILL DEPTHS

center drills #00 - #8 to achieve a chamfer of desired diameter

countersinks of 82, 90, and 120 degree included angles to achieve a

chamfer of desired diameter

spot drills of 90 and 130 degrees,  to achieve a chamfer of desired

diameter

twist drills of 118 and 135 degree included angles to achieve a

full-diameter hole of desired depth (including compensation for

width of chisel point)

FEEDS AND SPEEDS

milling and turning operations

twenty different material categories

nine types of cutting tools

output includes machining time and horsepower requirements

output in interactive format - any output value can be modified with

immediate update of all  dependent outputs

input and output can be independently toggled between inch and

metric

CIRCULAR FEEDRATES

feedrate compensation for circular milling

OPERATION TIPS

the menus are case sensitive

if you press a key and nothing happens, you are probably pressing a

key that's not on the current menu or are inputting an illegal value

at a screen used for inputting information, the Esc key will take you

back to the previous screen at any time

at any output screen the 'z' key will take you directly to the main

menu

the current input and output modes are displayed in the upper right

corner

input cannot be a calculation - input "5", not "3 + 2"

inch and metric input cannot occur at the same screen.

Main Menu

At the main menu, the six major options mentioned previously are

available.  Additionally, there is an option to shell out to DOS (y)

and an option to toggle between inch and metric input mode.  The

current mode is indicated in the upper right corner of the screen.

The input mode is modal, and will stay in effect until changed by

the user.

(c)  circular feedrates

When an end mill is cutting a straight line, the contact point of the

cutter is moving the same distance as the centerline of the cutter.

When the same tool is externally profiling a radius, the centerline is

moving further than the contact point - when internally profiling, the

centerline is moving the smaller distance.  As the centerline always

moves at the programmed feedrate, that feedrate must be adjusted

when machining a radius if the contact point of the cutter is to move

at the desired rate.

The menu provides the four possible combinations of inputs needed

to calculate this adjustment.  Options 'a' and 'b' provide for external

interpolation, options 'c' and 'd' for internal.  Options 'a' and 'c' are

used when the blueprint identifies the arc with a radius size - this is

normal when the arc is part of a continuous profile.  Options 'b' and

'd' are used when the blueprint specifies a diameter size - more

common when the circle covers 360 degrees.

There is no option to toggle inch/metric input mode here because it

is not needed.  As long as all input values are in the same mode, the

output will be correct.

Feedrate Adjustment Menus

The first value to be input is the radius or diameter to be machined.

This value will come from the blueprint.

The tool diameter is input next.  The program checks for negative

values and tool radius larger than part radius in the case of internal

interpolation.  If your input is rejected, check for these possibilities,

and be sure that you are not trying to mix inch and metric input.

The program next prompts for linear feedrate - this is the rate you

would feed the tool in a straight line (the uncompensated feedrate).

Input in distance per revolution or distance per minute.

The adjusted feedrate will be output along with a summary of the

inputs.

If you want to do another calculation, enter 'y'.  Any other key will

return you to the main menu.

(d)  drill depths

A calculation that CNC programmers and machinists make

repeatedly is how deep to send a particular type of drill or

countersink to achieve the desired result.

Center Drills

Options 'a' thru 'j' involve standard center drills (also known as

combination drill and countersinks).  The problem is how deep to

drill to produce a chamfer of a given diameter at the face of the

workpiece.  The user must select a center drill of a size capable of

achieving this diameter - after the tool is selected, the program

displays the high and low limits for the tool, and rejects inputs

outside those limits.  If you select a tool and find that it won't

meet your needs, use the Esc key to go back one screen and get a

different one.  The only input required is the desired chamfer

diameter.  Output will be a summary of the inputs and the calculated

depth in both inch and metric units.

Countersinks

Options 'k' thru 'm' involve standard countersinks.  The problem is

how deep to send a tool of a given included angle, with a flat of

known diameter on the end, to produce a chamfer of a given

diameter at the face of the workpiece.  The user selects a

countersink with the included angle specified on the

blueprint.  The input screen first asks for the required chamfer

diameter.

The diameter at the small end of the tool is next requested.  As a

reminder, the maximum value allowed is displayed, which must be

smaller than the chamfer diameter.  Output will be a summary of the

inputs and the calculated depth in both inch and metric units.

Spot Drills

Options 'n' thru 'o' involve standard spot drills.  The problem is

familiar - how deep to drill to chamfer the correct diameter.  Input is

the chamfer diameter required and the drill diameter, which in this

case must be larger than the chamfer diameter.  The program, taking

into account that spot drills have a chisel point on the end, of a

width proportionate to their diameter, outputs the required depth.

Twist Drills

Options 'p' and 'q' involve twist drills.  Blueprints normally specify a

drilled hole of a specific depth - but this means the full diameter of

the drill must go to that depth, rather than the point of the drill.

Input is the full-diameter depth required and the drill diameter.

CNC programmers and machinists commonly use multipliers of .3

and .207 times diameter to compensate for the length of 118 and

135 degree drill points, respectively, but these figures assume a

sharp point on the end of the drill.  The program, taking into

account that twist drills have a chisel point on the end, of a width

proportionate to their diameter, outputs a more accurate calculation.

(f)  feeds and speeds

An important part of the work of many manufacturing planners,

estimators, and CNC programmers in the metalworking industry is

the advance calculation of machining times for specific

combinations of workpiece materials, cutting tools, and machine

tools. Such people need a consistent and logical  method for

calculating efficient feedrates and spindle speeds - not too

slow, which is wasteful of machine time and manpower, and not too

fast, which is wasteful of tools and can increase costs in the long

run.  While optimal cutting parameters cannot always be calculated

ahead because of the wide range of subjective variables (variations

in material, machine rigidity, workpiece rigidity, clamping integrity,

etc), it is possible to establish reasonable starting points which can

then be optimized as necessary after visual observation of the cutting

operation at the machine.  That is the intent of this section of the

ME program.

The initial screen displays twenty material categories and, in

parentheses, a specific material designation which is representative

of that category.  Some programs exist which try to give specific

data for hundreds of different but similar materials - I have not

found this degree of detail to be especially helpful in my own work.

Remember, the purpose of this program is to provide starting points -

a knowledgeable and observant person at the machine will

always be the best judge of what is optimal.

After selection of a material category, the user is presented with a

selection of cutting tool types.

hss twist drill

An input screen appears which displays the currently chosen

material and tool type and asks for a tool diameter.  The maximum

and minimum diameters allowed by ME are also displayed, in

inches or millimeters depending on the active input mode.

The next prompt is for the hole depth - this is needed to determine

machining time and also to see if another prompt must be issued.  If

the ratio of hole depth to drill diameter is greater than or equal to

3:1, it is common practice to reduce feedrates and spindle speeds.

The larger the ratio, the greater the reduction.  This reduction is

probably more valuable with manual machines where regular drill

retraction or pecking is not easily done.  With CNC machines,

unless the ratio is quite large, it is probably less important.  In any

event, the user has a choice as to whether or not to apply the

compensation.

The output screen is now displayed.  This screen displays complete

information for the machining process selected, and a menu at the

bottom which allows the user to make changes to the data which

result in immediate updates to all data which are logically affected.

As this output screen is similar no matter which material or tool has

been selected, I will give a detailed explanation once and only

discuss the differences for specific tools as they occur.

Note that the upper right corner now displays status for input mode

and output mode.  The user can toggle the entire display between

inch and metric with the 'x' menu option.  Changing the input mode

(option 'X') will be discussed later.

The MATERIAL, HOLE DEPTH, TOOL, and tool DIAMETER

are based on user inputs.

DEPTH/DIAMETER is the ratio of hole depth to drill diameter.

HARDNESS displays the approximate Brinell and Rockwell "C"

material hardness values upon which the machining data are based.

These numbers will always reflect the low end of possible values for

the chosen material.  If the material you are machining has higher

values, spindle speeds, and sometimes feedrates, must be reduced.

A very general note is included at the top of the display screen as a

reminder to this effect.  The actual reduction necessary for

productive machining may vary from the values in the reminder.

MINUTES displays the machining time for one hole and the time

for all holes. The initial display will assume that one hole is being

machined, so both times will be the same.  The value for the total

number of holes can be modified from the menu, as will be discussed later.

SFM displays the surface feet per minute calculated for the

material/tool combination.

SMM displays the surface meters per minute calculated for the

material/tool combination.

RPM displays the spindle speed calculated for the material/tool

combination.  This will not change when the output units are toggled.

IPR displayes the feedrate in inches per revolution calculated for the

material/tool combination.

MPR displayes the feedrate in millimeters per revolution calculated

for the material/tool combination.

IPM displayes the feedrate in inches per minute calculated for the

material/tool combination.

MPM displayes the feedrate in millimeters per minute calculated for

the material/tool combination.

HP displays the power required at the motor (horsepower)

KW displays the power required at the motor (kilowatts)

MMR(ci) displays the material removal rate (cubic inches per

minute)

MMR(cc) displays the material removal rate (cubic centimeters per

minute)

Menu Options (case sensitive)

(t) Tool diameter - the 't' key will bring up an input box which can

be used to modify the current tool diameter.  The minimum and

maximum allowable values will be displayed.  The Esc key will back

out of the box with no changes being made.  After the new diameter

is input, all values on the display which depend on the tool diameter

will be automatically updated.

The 'Shift' key combined with the 't' key will increment the tool

diameter by approximately one percent.

The 'Ctrl' key combined with the 't' key will decrement the tool

diameter by approximately one percent.

The ability to use these 'Shift' and 'Ctrl' key combinations allows the

user to play some fast "what-if" or goal-finding games.  For

example, if you only have a five horsepower machine and the tool

you initially chose requires seven horsepower, 'Ctrl  t' will quickly let

you find the largest tool possible for that machine.  As the tool

diameter decreases, the power requirement and some other values

change right along with it.  These key combinations are available for

all menu options up to and including "Spindle override".

(l)  hole depth - the input box will allow modification of the hole

depth

(h)  number of holes - the input box will allow modification of the

number of holes

(d)  option unavailable  -  these options do not apply to the current

tool type

(w)  option unavailable                        "

(i)  option unavailable                        "

(f) Feed override - the input box will allow modification of the feed

override, expressed as a percentage of the feedrate initially

calculated by the program

(s)  Spindle override - the input box will allow modification of the

spindle override, expressed as a percentage of the speed originally

calculated by the program

(r)  Rpm limit - the input box will allow the user to set the maximum

spindle speed of which a particular machine tool is capable.  The

limit is initially set at 200,000 RPM, which is intended to mean "no

limit".  If the program calculates a spindle speed for a tool which

exceeds this rpm limit, the RPM display will show the value for the

rpm limit, but an asterisk will appear beside it to indicate that it has

been restricted, and underneath it, in parentheses, will be a number

showing what percentage this restricted value is of the calculated

value.  The rpm limit is modal.

(e) spindle efficiency - the input box will allow modification of the

spindle efficiency.  This figure, expressed as a percentage, allows

the planner to fine tune the power requirement value for a

machining operation.  This is because some machines are more

efficient at delivering power to the spindle than others - thus some

machines must have a higher power rating than others to do the

same job.

(x)  set metric output - toggles the display between inch and metric

units

(X)  set metric input - toggles the input mode between inch and

metric.  The user can display in one mode and input in the other.

(y)  previous menu - return to the previous menu - all displayed

values are initialized with the exception of the rpm limit, which is

modal

(z) main menu - return to the main menu - all displayed values are

initialized with the exception of the rpm limit, which is modal

indexable drill

An input screen appears which displays the currently chosen

material and tool type and asks for a tool diameter.  The maximum

and minimum diameters allowed by ME are also displayed, in

inches or millimeters depending on the active input mode.

The user is next prompted for a hole depth - this is needed to

determine machining time.

The output screen is now displayed. It is identical in content and

operation to the display for twist drills.

hss spade drill

Input and output screens are identical to those for indexable drills.

gun drill

Input and output screens are identical to those for indexable drills.

hss end mill

An input screen appears which displays the currently chosen

material and tool type and asks for a tool diameter.  The maximum

and minimum diameters allowed by ME are also displayed, in

inches or millimeters depending on the active input mode.

The user is next prompted for the number of flutes, or cutting

edges, on the tool.  An arbitrary upper limit of twelve is built into

the program.

The next value requested is the width of cut.  This cannot exceed

the tool diameter.

The fourth prompt is for the depth of cut (distance along the spindle

axis). The generally accepted upper limit, beyond which results

become less predictable (and less productive), is one and one half

times the tool diameter. This limit has been built into the program.

The output screen is now displayed.  Differences from the features

previously discussed are:

DEPTH OF CUT - note that this value cannot exceed one and one

half times the tool diameter - if the tool diameter is reduced, the

program checks the depth and reduces it also, if necessary.

WIDTH OF CUT - note that this value cannot exceed the tool

diameter - if the tool diameter is reduced, the program checks the

width and reduces it also, if necessary

IPT displays the feedrate in inches per tooth (flute) calculated for

the material/tool combination.

MPT displays the feedrate in millimeters per tooth (flute) calculated

for the material/tool combination.

(l)  Length of pass - the input box will allow modification of the

pass length

(p)  number of passes - the input box will allow modification of the

number of passes

(d)  Depth of cut - the input box will allow modification of the

depth of cut - this cannot exceed one and one half times the current

tool diameter

(w)  Width of cut  - the input box will allow modification of the

width of cut - this cannot exceed the current tool diameter

(n) Number of flutes - the input box will allow modification of the

number of cutting edges

WARNING

The user bears responsibility for correct tool selection.  For

example, most experienced persons would prefer a two flute end

mill over a four flute for heavy roughing cuts in aluminum, owing to

its greater chip clearance and freer cutting action.  The opposite

might be true when machining steel.  The program will, in both

cases,  show twice the feedrate for a four flute tool.  This might

sound good to the unwary, but results at the machine would not be

what was expected.  The ME program is an excellent planning aid,

but is not a substitute for experience.

carbide end mill

Inputs and outputs are identical to those for hss end mills.

face mill

An input screen appears which displays the currently chosen

material and tool type and asks for a tool diameter.  The maximum

and minimum diameters allowed by ME are also displayed, in

inches or millimeters depending on the active input mode.

The second prompt is for the number of inserts, or cutting edges, on

the tool. An arbitrary upper limit of one hundred is built into the

program.

The next value requested is the width of cut.  This cannot exceed

the tool diameter.

The last value requested is the depth of cut (distance along the

spindle axis).

The program does not try to take into account the many sizes and

geometric orientations of the inserts in face mills.  The maximum

depth of cut recommended for a specific face mill is a direct

reflection of these factors.  A limit on depth of cut has not been

built into the program.  The user is responsible for inputting realistic

depth of cut values.

The output screen is now displayed.  It is identical to that for end

mills, with the exception that option (i) Number of inserts replaces

option (n) Number of flutes.

hss reamer

Input and output screens are identical to those for indexable drills,

with the exception that power requirements and material removal

rate are not displayed.  The amount of material removed by the

reamer is not always known.  In any event, reamers are typically

used to remove very small amounts of material, and these figures

would have no practical value.

turning

An input screen appears which displays the currently chosen

material and tool type and asks for a turn diameter.  Note that this

must be the diameter actually being machined, rather than the

diameter of the material.

The next value requested is the depth of cut per side.  The value for

depth of cut cannot exceed one inch.   This limit will not prevent

some pretty unlikely combinations from being accepted.  Therefore,

the user is responsible for inputting realistic depth of cut values.

The output screen is now displayed.  It is identical to that for end

mills, with the exception that option (t) Turn diameter replaces

option (t) Tool diameter, and options (w) and (n) become

unavailable.

The output for turning is very generalized.  It assumes the use of

titanium- coated inserts under conditions of moderate roughing.

Feedrate can be adjusted down for finishing passes or up for heavy

roughing.  Speed can be adjusted up for finishing or down for heavy

roughing.  The user must know what he is trying to accomplish and

how to do it.  The machining times and power requirements are the

most valuable outputs here - the speeds and feeds are merely

references based on the combined recommendations of industry

professionals and my own experience.

(h)  hardness equivalents

There are three measures of steel hardness most commonly used in

the American metalworking industry.  They are Rockwell "C",

Brinell, and PSI.  The planner, CNC programmer, inspector, or

machinist often needs to convert a value from one scale into an

approximate equivalent on another.

Hardness Equivalence Menu (Steel)

The user selects the scale for which he has a value.

The input screen prompts for the known value.  Minimum and

maximum acceptable values are indicated.

The output screen shows three columns of eleven values each, with

the left column containing the values of the chosen scale.  The sixth

(middle) value in the left column will be the closest available to that

input by the user.  By reading from that value across the screen, the

equivalents may be obtained.

The '+' key will simultaneously scroll the tables by one value larger.

The '-' key will scroll one value smaller.  In this way, the entire table

of values may be browsed.

(t)   thread data

CNC programmers, machinists, and quality control personnel have

frequent need for dimensional data on threads.  This information is

in print, but complete analysis of a specific thread usually requires

that half a dozen manuals be searched and error-prone calculations

be made.  This is messy and time-consuming, and works poorly

when several persons need data from the same source

simultaneously.  The ME program displays a complete summary of

dimensional data for nearly two hundred of the most commonly

encountered threads.

Thread Menu

A menu of seven thread categories is displayed.

The next screen displays a menu of the specific threads within the

selected category.

The output screen displays data for the selected thread.  I will not

explain each output, the presumption being that a person who has

need of the data will understand what it means. The output can be

toggled between inch and metric with the 'x' key.  Entering a 'y' at

the display screen  returns to the Thread Menu.  Any other response

returns to the Main Menu.

(w)   material weights option

Estimators, process planners, and buyers must frequently calculate

the weight of material of a given composition and shape.  CNC

programmers, fixture designers, or machinists might need to

calculate the weight of a workpiece to make sure a machine or

fixture is capable of supporting it, or for safety considerations.  The

ME program displays a complete summary, including material type,

shape, dimensions, and weight in pounds and kilograms.

Material Weight Menu

A menu of nineteen materials is displayed.

The next screen displays a menu of supported shapes.

hexagonal

octagonal

An input screen appears which displays the currently chosen

material and shape and asks for the dimension across the flats.

The next prompt is for the material length.

Output is displayed.

rectangular

An input screen appears which displays the currently chosen

material and shape and asks for the material length .

The  next prompt is for the material width. The final prompt is for

material thickness.

Output is displayed.

square

An input screen appears which displays the currently chosen

material and shape and asks for the length of one side of the square.

The next prompt is for material thickness.

Output is displayed.

round

An input screen appears which displays the currently chosen

material and shape and asks for the material diameter.

The next prompt is for the material length.

Output is displayed.

tubing - o.d. and i.d.

An input screen appears which displays the currently chosen

material and shape and asks for the material outside diameter .

The next prompt is for the material inside diameter.

The final prompt is for the material length.

Output is displayed.

tubing - o.d. and wall

An input screen appears which displays the currently chosen

material and shape and asks for the material outside diameter.

The next prompt is for the material wall thickness, which must be

less than half the diameter.

The final  prompt is for the material length.

Output is displayed.

(y)  DOS Shell

This feature allows a temporary exit to DOS.  While at the DOS

prompt, type "exit" followed by the Enter key to return to ME.

SALES PITCH

I spent many hours on this project - gathering and interpreting the

data was a big job.  Learning C and writing the program took up

many long evenings.  I have some thoughts on ways the program

could be expanded.  In addition to more data and more types of

calculations, I can see value in such things as report generation and

menu/mouse support.  In order to improve the program, I will need

some feedback, both financial and intellectual.  If you like ME well

enough to use it on a regular basis, I think a check for twenty dollars

would be a fair deal for both of us.  If you register, you'll get the

next version free of charge.  Along with the check, send your

comments and ideas for improvement.  What do you especially like

or dislike about the program?  Does it help you do your job?  What

would you like to see new or different about it?

If you need a program like ME but don't think it's worth twenty

dollars, tell me why.

Michael Rainey

103 Fawn Lane

Kings Mountain, NC  28086

704-435-2459

MRAINEY - Genie