CMF Voice Editor

Version 1.0

by

Colin Paterson

April 1993

To run this program you require : EGA or VGA graphics, a mouse and a

sound blaster card.

Introduction

------------

This is my first attempt at PD programming and was prompted by the lack

of free software for using the FM synthesis on my Sound Blaster card.

This is very much an on going project on which I have grand plans,

conquer the PC market and become a household name, that sort of thing.

To this end I would be grateful if people would send me any comments and

suggestions for version 2.0.

In particular I have been slowed by the lack of publication of file

standards !! how can a file be standard if no one knows the format ? so

if anyone knows the format of BNK or ROL files please let me know.

Thanks.

CMF files

---------

CMF stands for creative music file and is essentially a straight MIDI

(musical instrument digital interface) file with a header containing

sound blaster (ADLIB) instrument definitions. Although CMF music editors

are fairly rare it is more than possible to create MID and ROL files

and convert them into the CMF format.

FM synthesis

------------

Each voice on the Sound Blaster uses two operators which are configured

to create either FM or additive synthesis. During FM synthesis operator

one is known as the phase modulator and determines the frequency of the

sound, whilst operator two acts as an envelope thus shaping the sound.

The resultant wave form may be feed back and in this wave very complex

sounds created.

The envelope is split into four phases  Attack, Decay, Sustain and

Release (ADSR). The best way to evaluate the effects is to try editing a

sound envelope.

|----------------------------|

|                            |

|       /\                   |

|      /  \Decay             |

|     /    \____________     |

|    /        Sustain   \    |

|   /                    \   |

|  /Attack                \  |

| /                 Release\ |

|/                          \|

------------------------------

During additive synthesis both operators produce a sine wave which are

then added together. This form of synthesis is easier to understand,

but does not produce complex sounds.

The Editing panel

-----------------

Once the program is loaded you will be greeted with a block diagram

schematic of the operators and their interconnections (sorry I'm a

control engineer). This is much easier to use than it looks see

The entire program is mouse driven and the various boxes are activated

by clicking in them with the mouse button. This either causes the value

(or diagram) to change automatically or enables you to control the

position of the controller with the mouse (Use it and you'll see what I

mean).

KSR 	Key Scale Rate, when this is on ( the box shows a tick) the

envelope is shortened as the notes pitch rises.

Hold 	When on the sustain level is held until the note is

released, otherwise the sound begins to decay once the sustain

level is reached.

Vib 	Toggle switch for vibrato.

AM 	Toggle Switch for amplitude modulation.

Harm 	Takes a value of between 0 and 15 which determines the

frequency at which the note will be produced.

To edit the ADSR envelope click the mouse button in one of the four

small boxes on the envelope vertices and then move the mouse. Click the

mouse button once more when the envelope is the desired shape.

The L.P (Low Pass) filter has four positions, each providing more

attenuation at high frequencies.

The three sliders work in much the same way as the ADSR envelope

shaping, click the mouse in the slider box, move to the desired position

and click to release.

The final sound editing box is the switch after the first operator, and

places the system in either additive or FM synthesis mode.

If you don't understand any of these options then just load up a CMF

file and play with the voices.

The editing screen also offers a number of menu options

Exit 	Leave the program

Play 	Output the current sound to the sound blaster until a key is

pressed on the keyboard.

Next 	Edit the next instrument in the CMF file

Prev 	Edit the previous instrument in the CMF file

Save 	Save the current voice back to the CMF file

Using the program

-----------------

To start the program you must give the name of the CMF file you wish to

work with,

vedit file.cmf

you will then be presented with a title screen, press any key

to proceed to the voice editing panel. I suggest you make a backup of

the CMF file before you edit it as you may not like the results of your

edits. After exiting the program use the PlayCMF file distributed by

Creative software to check your results.

Have fun.

Colin