CDTRAP.COM -- Carrier Loss Trap for PClones  18 Sep 87

Tom Jennings

Fido Software

164 Shipley

San Francisco CA 94107

CDTRAP is a program used to monitor the modem while

"outside" the Fido system, and reboot the system if carrier

is lost.

The assumption here is that you are using Fido's O)utside or

E)xitDOS command for sysop-only access to DOS commands.

Without CDTRAP or a similar program, if you were

disconnected for some reason, your computer would

mindlessly wait for the next command, but since Fido is no

longer in control it would sit there until you manually

rebooted the machine.

(Previous Fido versions would give the NEXT PERSON WHO

CALLED IN access to your DOS.)

CDTRAP is easy to use: in your batch file (see sample

"RUNFIDO.BAT" on Fido Software's BBS) you install CDTRAP

before you run COMMAND.COM; before you exit to Fido, you un

install CDTRAP.

CDTRAP			;installs it

CDTRAP R		;removes it

When CDTRAP is run, it first checks to see if was installed

previously; if so, it turns that previous copy "ON" or "OFF"

as indicated. Only if there is no CDTRAP resident does it

install itself in memory.

CDTRAP as supplied inserts itself into the PClones timer

tick, and monitors the modem status port of COM1 every tick,

or 55mS. If carrier is lost it causes the system to reboot.

To use CDTRAP with COM2, or a non-PClone machine, you will

need to edit and reassemble the program.