Here it is boys and girls, a kludgy driver to use your buffered

uart.  A couple of caveats, the command line IS CASE SENSETIVE.

You MUST declare the device driver path in lower case, and the

parameters in upper case.

I reccomend you just cut and paste from the following examples to

put it in your config.sys.  It's a plug in replacement for com16450.sys,

so if you already use that, then just make a minor edit to the filename

in your device statement.  here's my config.sys snippet for a 16450 on

com1, and a 16550 on com2:-

DEVICE=c:\mydll\com16450.sys COM1 3F8 4

DEVICE=c:\mydll\com16550.sys COM2 2F8 3

Both device drivers are included in this care package.  As you will see,

there are some command line options.  First is the device name.  It MUST

start with COM, and be followed by a single digit, ie COMx.  Case is

sensative.  the entire command line is case sensative.  The second item

on the command line is the base port for the device (in hex), and the

final is parameter is the irq to use.  You cannot share irq lines at

this time.  The above sample shows the standard configuration for com1

and com2.  You can install as many instances as you have serial ports

and irq lines to spare.  I currently have 4 instances installed in my

config.sys to handle 4 com ports.

One final note about com16450.sys.  It's not buffer aware, so if you

install it on a serial port with a 16550 in there, it's liable to trap

unless you re-init the system from a power down.  Since there is a

separate driver for the 16450 and 16550, I'm not going to fix that.  It

does turn into a minor performance issue.  I dont particularily like to

add 2 more instructions to an already bloated interrupt handler.

If you have comments, send netmail to 1:153/905.  I will listen to

constructive comments only, and also to bug reports.  I have a policy

around here of

Yelling >> NULL

So if you have problems, please describe them in detail.  If you start

yelling about a broken drive or some such, and want to blame my driver

I just wont listen.  If you approach me reasonably, I'll do everything

I can to help rectify your problems.  I too want this driver to be reliable

cuz it's running on my system as well.

With that out of the way, have fun with the new toy.  I sure am.

Gerry.  Sysop @ 1:153/905