PEDAL.COM

Author:		Keith Walker 	76505,2411

Date:		92.11.29

Description:

A memory-resident (TSR) routine to use a foot pedal (connected to the

game port) as a remote spacebar.  This allows the pedal to be used to

start and stop playback in CAKEWALK.

Call:

C> PEDAL	Installs the TSR routine

C> PEDAL/r	Removes the TSR routine from memory

NOTE: no space before "/", "r" is lowercase

Messages:

PEDAL installed		   Normal result of installation

PEDAL already installed	   Attempt to reinstall when already installed

PEDAL removed		   Normal result of removal

PEDAL is not installed	   Attempt to remove when not installed

Special Hardware Required:

Requires a game port

Requires a normally-closed foot switch, with ground wire connected to

pin 4 of the game port, and the live wire connected to pin 2.