* Initialization Of Microcom QX4232B 2400/9600 bps ADLC Gateway Use *

1. Set front panel dip switches to 2 ON (DOWN), all others OFF (UP).

2. Set all rear panel dip switches to OFF (UP).

3. Reset modem by cycling power.

4. Execute configuration batch file by typing:

gwintmcm COMx

where x is the number of the COM port used by the modem.

5. Verify that Modem Activation String in gateway setup is ATZ and the

Deactivation String is set to ATS0=0.