* Initialization Of Telebit T3000 For 9600 bps ADLC Workstation Use *

1. Verify that front panel A/B switch is in the A position.

2. With modem unplugged from interface, power OFF modem by placing the rocker

switch on back panel) in the 0 position.

3. Holding the front panel T/D button IN, power modem ON. Continue holding

button until MR, OH, CD, HS, EC all flash continually; then release T/D

button (the lights remain ON). Press and hold the T/D button once more

until LEDs flash again, then release. The MR and CTS lights should remain

ON, all other LEDs should be OFF, indicating that option set 0 is loaded.

4. Execute configuration batch file by typing:

WSI96TBT COMx

where x is the number of the COM port used by the modem.

5. Verify that modem Initialization String in workstation setup is AT&F and

Configuration String is ATZ.