[technical specifications]

eicon

technology

corporation

# EiconCard DPNA/PC

- Hardware Features

- ISA 8-bit

- Motorola 68000 @ 16MHz

- RAM: 2 MB

- Interrupt levels selectable via software (2, 3, 4, 5, 6, 7, 10, 11, 12, 14, 15)

- I/O address selectable via on-board switch (14 options)

- 4K shared memory access with a software selectable starting address (32 options between A000-DFFF)

- two serial high speed interface ports

- Serial Ports

- Software and hardware selectable interface types:

- V.24 and X.21bis (EIA RS-232-C)

- V.35

- X.21 with V.11 (X.27) signaling

- 26-pin high-density D-type female connector, supporting all interfaces with appropriate cable

- Line speed up to 128 Kbps per port

- Internal or external clocking

- Protocol support: X.25, Frame Relay, PPP, SDLC, HDLC

- Throughput:

- X.25: 155 pps

- Frame Relay: 220 pps

- Maximum Power Requirements

- 2A at +5 Volts

- 70 mA at -5 Volts

- 70 mA at +12 Volts

- 70 mA at -12 Volts

- Environment Requirements

- Operating temperature: 0°C to 50°C

- Operating humidity: 0 to 90% (non-condensing)

- Barometric operating pressure: 86 to 106 kPascals

- Maximum tolerance in power supply variation: +5% to -5%

- Certification

- FCC Class B, Part 15

- CE marking

- Warranty

- 5-year

- Dimensions

- 13.18" x 4.2" (33.48cm x 10.67cm)

- Supported OS platforms

- DOS

- NetWare

- OS/2

- UNIX

- Windows 3.x

- Windows NT

- Packaging Contents

- EiconCard DPNA/PC

- EiconCard DPNA/PC Installation Guide

- Hardware Diagnostic diskette

- V.24 HSI converter

| - Ordering Information |  |
| --- | --- |
| - Product Name | - Product Code |
| - EiconCard DPNA/PC | - 310-077 |
| - Related Accessories |  |
| - V.24 HSI modem cable | - 310-026 |
| - V.24 /V.35 HSI null modem cable | - 310-031 |
| - V.24 HSI converter | - 310-046 |
| - V.35 HSI modem cable | - 310-024 |
| - V.35 HSI to AS-400 null modem cable | - 310-047 |
| - X.21 HSI modem cable | - 310-025 |
| - X.21 HSI null modem cable | - 310-032 |
| - HSI null modem conversion cable | - 310-033 |

Eicon Technology Corporation

Tel.:	(514) 745-5500

Fax:	(514) 745-5588

In the United States, Canada, 
Latin America:

Tel.:	1-800-80-EICON

(214) 239-3270

Fax:	(214) 239-3304

In Europe, Middle East, Africa:

Tel.:	(44) 181 967-8000

Fax:	(44) 181 967-8050

In Australia, New Zealand, Asia:

Tel.:	(61) 2 9919-7200

Fax:	(61) 2 9929-6300

Internet:

North America: sales@eicon.com

Europe: sales.europe@eicon.com

Asia Pacific: sales.asiapac@eicon.com

Worldwide Web: http://www.eicon.com/

© 1996 Eicon Technology Corporation.  All rights reserved.