[technical specifications]

eicon

technology

corporation

# EiconCard S51

- Hardware Features

- ISA 8-bit

- Motorola 68302 @ 20 MHz

- 2 MB RAM

- Interrupt levels selectable via software (2, 3, 4, 5, 6, 7, 10, 11, 12, 14, 15)

- I/O address selectable via on-board switch (14 options)

- 4KB shared memory access; software selectable starting address (48 options; ranges C000-EF00 and FC000-FEF00)

- Serial High Speed Interface port

- ISDN Basic Rate Interface port

- Serial Port

- Software and hardware selectable interface types:

- V.24 and X.21bis (EIA RS-232-C)

- V.35

- X.21 with V.11 (X.27) signaling

- 26-pin high-density D-type female connector, supporting all interfaces with appropriate cable

- Line speed up to 512 Kbps

- Internal or external clocking

- Protocol support: X.25, Frame Relay, PPP, SDLC, HDLC

- Throughput:

- X.25: 400 pps

- Frame Relay: 500 pps

- ISDN Port

- DIN-4 female connector

- S/T interface using external cable assembly (RJ45)

- U interface using Eicon ISDN NT1

- Line speed up to 128 Kbps

- Channel support:

- B-channel circuit switched

- B-channel packet switched (X.31 case A, B)

- D-channel circuit switched (X.31 case B)

- B-channel protocol support: X.25, Frame Relay, PPP, Multilink PPP, SDLC, HDLC

- D-channel protocol support: X.25

- Maximum Power Requirements

- 3 A at +5 Volts

- 80 mA at +12 Volts

- 50 mA at -12 Volts

- Environment Requirements

- Operating temperature: 0°C to 50°C

- Operating humidity: 0 to 90% (non-condensing)

- Barometric operating pressure: 86 to 106 kPascals

- Maximum tolerance in power supply variation: +5% to -5%

- Certification

- FCC Class B, Part 15

- CE marking

- Industry Canada (formerly DOC)

- National ISDN-1 (NI-1)

- AT&T 5ESS Custom

- Euro-ISDN (including German, French and Swiss extensions)

- JATE (Japan)

- TPH (Australia)

- Warranty

- 5-year

- Dimensions

- 8.980" x 4.2" (22.80cm x 10.67cm)

- Supported OS platforms

- DOS

- NetWare

- OS/2

- UNIX

- Windows 3.x

- Windows NT

- Packaging Contents

- EiconCard S51

- EiconCard S51 Installation Guide

- Hardware Diagnostic diskette

- V.24 HSI converter

- with ISDN S/T interface cable

- ISDN S/T Interface cable

- RJ45 cable

- Terminating Resistor (except Europe)

- with ISDN NT1

- Eicon ISDN NT1 unit and associated cable

- power supply for NT1

- Eicon ISDN NT1 installation guide

- RJ45 cable

| - Ordering Information |  |
| --- | --- |
| - Product Name | - Product Code |
| - EiconCard S51 | - 310-117 |
| - EiconCard S51 with ISDN S/T cable | - 310-151 |
| - EiconCard S51 with ISDN NT1 unit | - 310-160 |
| - Related Accessories |  |
| - V.24 HSI modem cable | - 310-026 |
| - V.24 /V.35 HSI null modem cable | - 310-031 |
| - V.24 HSI converter | - 310-046 |
| - V.35 HSI modem cable | - 310-024 |
| - V.35 HSI to AS-400 null modem cable | - 310-047 |
| - X.21 HSI modem cable | - 310-025 |
| - X.21 HSI null modem cable | - 310-032 |
| - HSI null modem conversion cable | - 310-033 |
| - ISDN S/T1 cable - North America | - 310-090 |
| - ISDN S/T1 cable - Europe, Japan | - 310-092 |
| - ISDN NT1 unit | - 310-095 |
| - ISDN N/T1 cable - North America only | - 310-097 |

Eicon Technology Corporation

Tel.:	(514) 745-5500

Fax:	(514) 745-5588

In the United States, Canada, 
Latin America:

Tel.:	1-800-80-EICON

(214) 239-3270

Fax:	(214) 239-3304

In Europe, Middle East, Africa:

Tel.:	(44) 181 967-8000

Fax:	(44) 181 967-8050

In Australia, New Zealand, Asia:

Tel.:	(61) 2 9919-7200

Fax:	(61) 2 9929-6300

Internet:

North America: sales@eicon.com

Europe: sales.europe@eicon.com

Asia Pacific: sales.asiapac@eicon.com

Worldwide Web: http://www.eicon.com/

© 1996 Eicon Technology Corporation.  All rights reserved.