# [technical information]

# Eicon WAN Interface Cards

| Card | Bus Type | Serial Interfaces | ISDN port | CPU | Max. Serial Line Speed | Throughput (pps) | Memory RAM/Flash | Product Code | Accessories Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EC/PC | ISA 8-bit | V.24 | - | 68008@10MHz | 64 Kbps | 50/80 | 1/0 MB | 310-053 | A |
| HSI/PC | ISA 8-bit | HSI | - | 68000@16MHz | 128Kbps | 155/220 | 1/0 MB | 310-052 | B |
| HSI/MC | MC | HSI | - | 68000@16MHz | 128Kbps | 155/220 | 1/0 MB | 310-050 | B |
| DPNA/PC | ISA 8-bit | 2 x HSI | - | 68000@16MHz | 128Kbps | 155/220 | 2/0 MB | 310-077 | B |
| PacketBlaster (NetWare) | ISA 16-bit | 2 x VHSI | - | HD64570 | 2.048 Mbps | - | 64/0 KB | 305-674 | C |
| EC C20 | ISA 8-bit PnP | V.24 | - | 68302@16MHz | 64 Kbps | 300/350 | 512/512 KB | 310-198 | A |
| EC C21 | ISA 8-bit PnP | V.24 | BRI | 68302@16MHz | 64 Kbps | 300/350 | 512/512 KB | 310-150 | A, E, G |
| EC C21 (ISDN S/T cable) | ISA 8-bit PnP | V.24 | BRI | 68302@16MHz | 64 Kbps | 300/350 | 512/512 KB | 310-118 | A, E, G |
| EC C21 (ISDN NT1) | ISA 8-bit PnP | V.24 | BRI | 68302@16MHz | 64 Kbps | 300/350 | 512/512 KB | 310-158 | A, E, G |
| EC C31 | PCMCIA II | V.24 | BRI | 68302@16MHz | 64 Kbps | 300/350 | 512/512 KB | 310-152 | D, F, G |
| EC C31 (ISDN S/T cable) | PCMCIA II | V.24 | BRI | 68302@16MHz | 64 Kbps | 300/350 | 512/512 KB | 310-116 | D, F, G |
| EC C31 (ISDN NT1) | PCMCIA II | V.24 | BRI | 68302@16MHz | 64 Kbps | 300/350 | 512/512 KB | 310-159 | D, F, G |
| EC S50 | ISA 8-bit | HSI | - | 68302@20MHz | 512 Kbps | 400/500 | 2/0 MB | 310-138 | B |
| EC S51 | ISA 8-bit | HSI | BRI | 68302@20MHz | 512 Kbps | 400/500 | 2/0 MB | 310-117 | B, E, G |
| EC S51 (ISDN S/T cable) | ISA 8-bit | HSI | BRI | 68302@20MHz | 512 Kbps | 400/500 | 2/0 MB | 310-151 | B, E, G |
| EC S51 (ISDN NT1) | ISA 8-bit | HSI | BRI | 68302@20MHz | 512 Kbps | 400/500 | 2/0 MB | 310-160 | B, E, G |

Notes:

- HSI interface supports V.24, V.35, X.21

- VHSI  interface supports V.24, V.35, X.21, RS 449 (V.36), EIA-530

- Detailed hardware information is available in the EiconCard Reference Guide

- Throughput information is provided for X.25/Frame Relay respectively

| Accessories Category | Product Code |
| --- | --- |
| A |  |
| V.24 RS-232 modem cable | 300-007 |
| V.24 RS-232 null modem cable | 300-022 |
| V.24 RS-232 gender-changed null modem cable | 300-048 |
| B |  |
| V.24 HSI modem cable | 300-026 |
| V.35 HSI modem cable | 300-024 |
| X.21 HSI modem cable | 300-025 |
| V.24/V.35 HSI-HSI null modem cable | 300-031 |
| X.21 HSI-HSI null modem cable | 300-032 |
| HSI null-modem conversion cable | 300-033 |
| V.24 HSI converter | 300-046 |
| V.35 HSI-AS/400 null modem cable | 300-047 |
| C |  |
| V.24 VHSI modem cable | 300-077 |
| V.24 VHSI DTE modem cable | 300-078 |
| V.35 VHSI modem cable | 300-076 |
| V.35 VHSI modem cable (France) | 300-083 |
| X.21 VHSI modem cable | 300-081 |
| V.36/RS-449 VHSI cable | 300-079 |
| EIA-530 VHSI cable | 300-080 |
| VHSI-to-VHSI null modem cable | 300-075 |
| D |  |
| V.24 PCMCIA modem cable | 300-094 |
| E |  |
| ISDN S/T1 interface cable - NA | 300-090 |
| ISDN S/T1 interface cable - Europe, Japan | 300-092 |
| ISDN N/T1 interface cable - NA | 300-097 |
| F |  |
| ISDN S/T2 interface cable - NA | 300-091 |
| ISDN S/T2 interface cable - Europe, Japan | 300-093 |
| ISDN N/T2 interface cable - NA | 300-098 |
| G |  |
| ISDN NT1 unit | 300-095 |

# [OS platform support]

|  |  | Operating System |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EiconCard | DOS/ Windows 3.x |  | NetWare | OS/2 | UNIX | Windows NT | Windows 95 |
| EC/PC | û |  | û | û | û | û |  |
| HSI/PC | û |  | û | û | û | û |  |
| HSI/MC | û |  |  | û |  | û |  |
| DPNA/PC | û |  | û | û | û | û |  |
| PacketBlaster |  |  | û |  |  |  |  |
| EC C20 |  |  |  |  | û |  |  |
| EC C21 | û |  |  |  | û | û |  |
| EC C31 | û |  |  |  |  |  |  |
| EC S50 |  |  |  |  | û |  |  |
| EC S51 | û |  | û | û | û | û |  |

Note: Detailed software/driver information is available in Eicon's OS-specific Reference Guides