* Initialization Of Telebit T3000 For 9600/2400 bps ADLC Gateway Use *

1. Verify that front panel A/B switch is in the A position.

2. With modem unplugged from interface, power OFF modem by placing the rocker

switch (on back panel) in the 0 position.

3. Holding the front panel T/D button IN, power modem ON. Continue holding

button until MR, OH, CD, HS, EC all flash continually; then release T/D

button (the lights remain ON). Press and hold the T/D button once more

until LEDs flash again, then release. The MR and CTS lights should remain

ON, all other LEDs should be OFF, indicating that option set 0 is loaded.

4. Execute configuration batch file by typing:

GWINTTBT COMx

where x is the number of the COM port used by the modem.

5. Verify that modem Activation String in gateway setup is ATZ and

Deactivation String is ATS0=0.